| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0471-v1 |
| --- | --- |

| Title: | On CCLM |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Frank Bossen | Tel: Email: | fbossen@sharpsec.com |
| Source: | Sharp Electronics of Canada Ltd. |  |  |

_____________________________

# Abstract

Contributions JVET-R0314 and JVET-R0375 propose changes to the CCLM process to address ill-defined cases that can arise when the top-left CTU is not available while the left and top CTUs are available. An alternative set of changes is proposed, which is asserted to be simpler and to have no impact on behaviour under common test conditions (CTC). Additionally, numerous issues are identified in the CCLM text and changes are proposed to resolve these.

# Introduction

In VVC draft 8, the CCLM process assumes that if the CTU to the left of the current CTU is available and the CTU above the current CTU is available, then the above-left CTU is also available. However, this is not necessarily true when using non-rectangular slices.

# Proposed solution

For a block located at luma location (x, y) the CCLM process may refer to the luma samples located at (x − 1, y − 1), (x − 1, y − 2), and (x − 2, y − 1), which may lie in the above-left CTU. It is proposed to properly account for the availability of those samples as further described below.

Handling of sample located at (x − 2, y − 1)

Reference to the sample located at (x − 2, y − 1) may happen when the chroma format is 4:2:0, sps_chroma_vertical_collocated_flag is equal to 1, and numSampL is greater than 0 (i.e., one or more samples from the left are used for deriving parameters of the CCLM linear model).

When the sample located at (x − 2, y − 1) is referenced but unavailable, it is proposed to use the neighbouring sample located at (x − 2, y) instead (it is available since numSampL is greather than 0).

Handling of samples located at (x − 1, y − 1) and (x − 1, y − 2)

Reference to the samples located at (x − 1, y − 1) and (x − 1, y − 2) may happen when the chroma format is 4:2:0 or 4:2:2, and numSampT is greater than 0 (i.e., one or more samples from above are used for deriving parameters of the CCLM linear model).

When the sample located at (x − 1, y − 1) is referenced but unavailable, it is proposed to use the neighbouring sample located at (x, y − 1) instead (it is available since numSampT is greather than 0).

When the sample located at (x − 1, y − 2) is referenced but unavailable, it is proposed to use the neighbouring sample located at (x, y − 2) instead (it is available since numSampT is greather than 0 and bCTUboundary is false).

## Text changes

The derivation of the variable availTL is changed from:

- The variable availTL is derived as follows:

availTL  =  availL  &&  availT		(352)

to:

- The derivation process for neighbouring block availability as specified in clause 6.4.4 is invoked with the current luma location ( xCurr, yCurr ) set equal to ( xTbY, yTbY ), the neighbouring luma location ( xTbY − 1, yTbY − 1 ), checkPredModeY set equal to FALSE, and cIdx as inputs, and the output is assigned to availTL.

The following rules are added to the derivation of the neighbouring luma samples pY[ x ][ y ]:

    - When numSampL is greather than 0 and availTL is equal to FALSE, the neighbouring top-left sample pY[ −2 ][ −1 ] is set equal to the luma sample pY[ −2 ][ 0 ].

    - When numSampT is greather than 0 and availTL is equal to FALSE, the neighbouring top-left samples pY[ −1 ][ y ] with y = −1..−2 are set equal to the luma samples pY[ 0 ][ y ].

Note that the attached draft text includes the above changes and further reorganizes the derivation of the neighbouring luma samples pY[ x ][ y ] as follows:

- Derivation of left samples that may be referenced in equations (370) and (371):

    - When availL is equal to FALSE, the neighbouring left luma samples pY[ x ][ y ] with x = −1, y = 0..SubHeightC * nTbH − 1, are set equal to the luma samples pY[ 0 ][ y ].

    - When availL is equal to TRUE, the neighbouring left luma samples pY[ x ][ y ] with x = −1, y = 0..SubHeightC * nTbH − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

- Derivation of left and above-left samples that may be referenced in equations (372), (373) and (374):

    - When numSampL is greater than 0, the neighbouring left luma samples pY[ x ][ y ] with x = −1..−3, y = 0..SubHeightC * numSampL − 1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY + x , yTbY +y ).

    - When numSampL is greather than 0 and availTL is equal to TRUE, the neighbouring top-left sample pY[ −2 ][ −1 ] is set equal to the reconstructed luma sample prior to the deblocking filter process at the location ( xTbY – 2, yTbY – 1 ).

    - When numSampL is greather than 0 and availTL is equal to FALSE, the neighbouring top-left sample pY[ −2 ][ −1 ] is set equal to the luma sample pY[ −2 ][ 0 ].

- Derivation of above samples that may be referenced in equation (370):

    - When availT is equal to FALSE, the neighbouring top luma samples pY[ x ][ y ] with x = 0..SubWidthC * nTbW − 1, y = −1, are set equal to the luma samples pY[ x ][ 0 ].

    - When availT is equal to TRUE, the neighbouring top luma samples pY[ x ][ y ] with x = 0..SubWidthC * nTbW − 1, y = −1, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

- Derivation of above and above-left samples that may be referenced in equations (375), (376), (377), (378), and (379):

    - When numSampT is greater than 0, the neighbouring top luma samples pY[ x ][ y ] with x = 0..SubWidthC * numSampT − 1, y = −1..−3, are set equal to the reconstructed luma samples prior to the deblocking filter process at the locations ( xTbY+ x, yTbY + y ).

    - When numSampT is greather than 0 and availTL is equal to TRUE, the neighbouring top-left samples pY[ −1 ][ y ] with y = −1..−2 are set equal to the reconstructed luma samples prior to the deblocking filter process at the location ( xTbY – 1, yTbY + y ).

    - When numSampT is greather than 0 and availTL is equal to FALSE, the neighbouring top-left samples pY[ −1 ][ y ] with y = −1..−2 are set equal to the luma samples pY[ 0 ][ y ].

## Other fixes

A number of additional issues are addressed in the attached modified draft text. The changes are believed to be consistent with previously agreed design intent.

- Samples pY[ −1 ][ y ] may be needed in equations (370) and (371). However, no value is assigned to them if numSampL is 0 and availL is TRUE. This is corrected as in the section above;

- Samples pY[ x ][ −1] may be needed in equation (370). However, no value is assigned to them if numSampT is 0 and availT is TRUE. This is corrected as in the section above;

- sps_chroma_vertical_collocated_flag may be equal to 0 only when the chroma format is 4:2:0. The filter F4 is used only when sps_chroma_vertical_collocated_flag is 0. Defining F4 for chroma formats other than 4:2:0 is unnecessary. Such unnecessary definition is removed;

- Horizontal filtering when the chroma format is 4:2:2 is correctly defined;

- Various fixes found in other input contributions such as JVET-R0314 and JVET-R0452 are included.

Regarding the 4:2:2 case, it should be noted that the VTM software appears to incorrectly set sps_chroma_vertical_collocated_flag to 0 but nevertheless correctly applies a (1 2 1) horizontal filter on luma samples. When correcting the flag value to 1, filter definitions should be updated in VTM (in accordance to the text changes listed above).

# Patent rights declaration(s)

Sharp Labs of America may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).