| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 18th Meeting: by teleconference, 15–24 April 2020 | Document: JVET-R0488-v1 |
| --- | --- |

| Title: | Sample clipping in LMCS |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhi-Yi Lin, Tzu-Der Chuang, Ching-Yeh Chen, Chih-Wei Hsu, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {zhiyi.lin, peter.chuang, chingyeh.chen, cw.hsu, yuwen.huang, shawmin.lei} @mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

- Abstract

This document reports a mismatch between VTM8.0 and VVC Draft 8 for LMCS. In VVC Draft 8, non-CIIP inter prediction samples are not clipped within [0, 2bitDepth – 1] after LMCS forward mapping. However, in VTM8.0, same as intra prediction samples, IBC prediction samples, and CIIP inter prediction samples after LMCS forward mapping, non-CIIP inter prediction samples are also clipped within [0, 2bitDepth – 1] after LMCS forward mapping. It is suggested to either align text to software or algin software to text. Aligning text to software is preferred by the authors of this document because the design of the current LMCS was based on the results of software simulation. In addition, it is asserted that VTM8.0 has a more consistent design of sample clipping behavior between intra prediction, IBC prediction, CIIP inter prediction, and non-CIIP inter prediction.

Introduction

In VVC Draft 8 [1], if the luma mapping and chroma scaling (LMCS) is applied, the forward mapping is performed on inter prediction samples. There is a mismatch between VTM8.0 [2] and VVC Draft 8 on the inter prediction sample clipping process after the LMCS forward mapping. In VVC Draft 8, CIIP inter prediction samples are clipped within the range of [0, 2bitDepth – 1], while non-CIIP inter prediction samples are not clipped. When the current block is coded with CIIP mode, the combined prediction sample values are derived as below (in clause 8.5.6.7 in JVET-Q2001-vE.docx):

When cIdx is equal to 0 and slice_lmcs_enabled_flag is equal to 1, predSamplesInter[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are modified as follows:

idxY = predSamplesInter[ x ][ y ]  >>  Log2( OrgCW )
predSamplesInter [ x ][ y ] = Clip1( LmcsPivot[ idxY ] + 	(1028)
	( ScaleCoeff[ idxY ] * ( predSamplesInter[ x ][ y ] − InputPivot[ idxY ] ) +
	( 1  <<  10 ) )  >>  11 )

The prediction samples predSamplesComb[ x ][ y ] with x = 0..cbWidth − 1 and y = 0..cbHeight − 1 are derived as follows:

predSamplesComb[ x ][ y ] = ( w * predSamplesIntra[ x ][ y ] + 	(1029)
 	( 4 − w ) * predSamplesInter[ x ][ y ] + 2)  >>  2

For non-CIIP inter blocks, the clipping process after the LMCS forward mapping is skipped, and the residuals are directly added to the un-clipped forward mapped prediction samples to generate reconstruction samples. Sample clipping is applied to the reconstruction samples to make sure the reconstruction samples are within the range of the sample bit depth. The process can be described as below (in clause 8.7.5.2 in JVET-Q2001-vE.docx):

…

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ] >> Log2( OrgCW )
predMapSamples[ i ][ j ] =  LmcsPivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + 
	  ( 1 << 10 ) ) >> 11 					(1232)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

The reconstructed luma picture sample recSamples is derived as follows:

recSamples[ xCurr + i ][ yCurr + j ] = Clip1( predMapSamples[ i ][ j ]+ resSamples[ i ][ j ] ] )	(1233)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

However, in VTM8.0, non-CIIP inter prediction samples are always clipped after the LMCS forward mapping. The software behavior is described as below:

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ] >> Log2( OrgCW )
predMapSamples[ i ][ j ] =  Clip1(LmcsPivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + 
	  ( 1 << 10 ) ) >> 11) 					(1232)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh − 1

The highlight part is the software/text mismatch.

Proposed methods

Align software to text

Removing the sample clipping for non-CIIP inter prediction samples in VTM8.0.

Align text to software

Adding the clipping for non-CIIP inter prediction samples. The text changes are shown as follows (in clause 8.7.5.2 in JVET-Q2001-vE.docx).

- Otherwise (CuPredMode[ 0 ][ xCurr ][ yCurr ] is equal to MODE_INTER and ciip_flag[ xCurr ][ yCurr ] is equal to 0), the following applies:

idxY =  predSamples[ i ][ j ] >> Log2( OrgCW )
predMapSamples[ i ][ j ] =  Clip1(LmcsPivot[ idxY ] 
	 + ( ScaleCoeff[ idxY ] * ( predSamples[ i ][ j ] − InputPivot[ idxY ] ) + 
	  ( 1 << 10 ) ) >> 11) 					(1232)
	with i = 0..nCurrSw − 1, j = 0..nCurrSh – 1

Since intra prediction samples and IBC prediction samples are also clipped within [0, 2bitDepth – 1] before adding the residual to generate the reconstruction sample, it is suggested to align text to software so that inter prediction samples are also clipped within [0, 2bitDepth – 1] before generating reconstruction sample.

Conclusions

There is a software/text mismatch when LMCS is applied. It is suggested to align text to software to perform sample clipping for non-CIIP inter prediction samples after LMCS forward mapping.

References

- B. Bross, J. Chen, and S. Liu, “Versatile Video Coding (Draft 8),” Document of Joint Video Experts Team, JVET-Q2001.

- VTM8.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM

Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).