| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 |  |  | Document: JVET-S0010 |
| --- | --- | --- | --- |
| Title: | JVET AHG report: Encoding algorithm optimizations (AHG10) |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Report |  |  |  |
| Author(s) or Contact(s): | Alberto Duenas Alexis Michael Tourapis Sergey Ikonin Andrey Norkin Rickard Sjöberg J. Le Tanou J.-M. Thiesse | Email: | albertoduenas@fb.com atourapis@apple.com Sergey.Ikonin@huawei.com anorkin@netflix.com rickard.sjoberg@ericsson.com julien.letanou@mediakind.com jean-marc.thiesse@vitec.com |  |
| Source: | AHG |  |  |  |

_____________________________

# Abstract

The document summarizes the activities of the AHG on Encoding algorithm optimizations between the 18th meeting conducted by teleconference 15-24 April 2020 and the 19th meeting conducted by teleconference (22 June -01 July 2020)

# Introduction

At the 18th JVET meeting, the AHG on Encoding algorithm optimizations was established with the following mandates:

- Study the impact of using techniques such as GOP structures and perceptually optimized adaptive quantization for encoder optimization.

- Study quality metrics for measuring subjective quality using e.g. the CfP response MOS scores.

- Study the impact of adaptive quantization on individual tools in the test model.

- Investigate other methods of improving objective and/or subjective quality, including adaptive coding structures and multi-pass encoding.

- Study methods of rate control and their impact on performance, subjective and objective quality.

The regular JVET e-mail reflector was used for discussions (jvet@lists.rwth-aachen.de). No e-mail related to AHG10 activity was sent to the JVET reflector during the AHG period.

# Overview of input documents related to the AHG

The following input documents were identified to be related to the AHG:

## JVET- S0180: Addition of a GOP hierarchy of 32 for random access configuration for VTM

This contribution is proposing to include a GOP hierarchy of 32 for random access configuration to VTM and it is also suggested to use it for CTC. This will increase the number of pictures in the DPB by one compared to a GOP hierarchy of 16 such that the DPB requirements are 8, e.g. at the maxDpbPicBuf, instead of 7. It will also increase the end-to-end latency by additional 2x16/60 seconds for 60 Hz video which is expected to be acceptable for a wide range of random access applications. Enabling a GOP hierarchy of 32 also for sequences which not have an intra period which is a multiple of 32 requires adjustment of the intra period for those sequences such that the intra period becomes a multiple of 32, e.g. frame rate 50/60Hz will have an intra period of 64 and frame rate 30Hz or below will have an intra period of 32.

The BD rate effect of this change is as follows:

CTC SDR RA -3.0%, -6.3%, -6.4% (luma,Cb/Cr)

CTC HDR RA -7.8%, -5.2, -4.4%, -4.2% (deltaE, psnrL, wpsnrY, psnrY)

It is also asserted that the visual quality is improved.

In JVET-B0039 the benefits of a GOP hierarchy of 32 compared to GOP hierarchy of 16 was shown. An informal viewing was also held at the meeting and reported in JVET-B0075. The results indicated visual benefits of using a GOP hierarchy of 32 compared to a GOP hierarchy of 8. The doubling of the memory for the encoder compared to a GOP hierarchy of 16 was at the time (year 2016) regarded as an issue and a GOP hierarchy of 16 was instead selected for CTC. However, we argue that these memory constraints are more relaxed now, more than four years later. At a recent meeting the maxDpbPicBuf was also increased for VVC to 8 pictures which gives additional capability that can be used by a larger GOP hierarchy for random access.

The impact of extending the length of the GOP hierarchy from 16 to 32 are:

- The number of temporal layers increases from 5 to 6,

- The DPB size increases from 7 to 8

- An increase of the maximum reorder latency from 16 to 32 pictures which corresponds to an end-to-end (encoder + decoder) latency of 2x32/60 instead of 2x16/60 for a frame rate of 60 Hz.

Since most current and future content is expected to be at least 60 Hz, the added latency may be found acceptable for several broadcast, streaming and offline video delivery applications.

### VTM-9.0

Table 1: BDR for a GOP hierarchy of 32 for SDR CTC VTM-9.0 random access configuration

|  | Over VTM-9.0 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,94% | -4,83% | -5,05% | 101% | 90% |
| Class A2 | -3,10% | -6,87% | -6,07% | 100% | 92% |
| Class B | -3,32% | -7,23% | -7,58% | 100% | 94% |
| Class C | -3,39% | -5,95% | -6,13% | 100% | 93% |
| Class E |  |  |  |  |  |
| Overall | -3,02% | -6,33% | -6,38% | 100% | 92% |
| Class D | -3,28% | -5,93% | -6,23% | 101% | 96% |
| Class F | -6,46% | -7,36% | -7,23% | 96% | 90% |

### HM-16.20

Table 4: BDR for a GOP hierarchy of 32 for SDR HM-16.20 CTC random access configuration

|  | Over VTM-9.0 (Intra period 64 for 50Hz and 32 for 20Hz) |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1,12% | -4,57% | -4,65% | 95% | 99% |
| Class A2 | -1,67% | -4,63% | -4,83% | 94% | 96% |
| Class B | -2,59% | -7,87% | -7,38% | 93% | 95% |
| Class C | -3,33% | -5,65% | -5,53% | 94% | 96% |
| Class E |  |  |  |  |  |
| Overall | -2,31% | -5,97% | -5,83% | 94% | 96% |
| Class D | -3,21% | -5,80% | -5,64% | 96% | 92% |
| Class F | -6,07% | -6,86% | -6,69% | 92% | 97% |

# Recommendation

The AHG recommends that the related input contributions are reviewed and to further continue the study of encoding algorithm optimizations in JVET.