| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0045 |
| --- | --- |

| Title: | AHG9: On the sign of DeltaPocValSt |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu Rickard Sjöberg Zhi Zhang Martin Pettersson Mitra Damghanian Jack Enhorn Jacob Ström Ruoyang Yu | Email: | du.liu@ericsson.com rickard.sjoberg@ericsson.com zhi.a.zhang@ericsson.com martin.m.pettersson@ericsson.com mitra.damghanian@ericsson.com jack.enhorn@ericsson.com jacob.strom@ericsson.com ruoyang.yu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

The authors claim that in the VVC draft specification v10, a sign flag value equal to 0 means that the corresponding value is positive and equal to 1 means that it is negative. However, for DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] and its sign flag strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], the use is asserted to be the opposite. Secondly, the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] associates the flag value with an “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )”. However, the “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )” is not necessarily a value, and associating a sign flag for that seems inaccurate to the proponents. Thirdly, RefPicPocList[ i ][ j ] is constructed by subtracting DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] from pocBase. It is claimed that it is cleaner to add a delta POC value to pocBase than to subtract it from pocBase.

This contribution proposes the following three modifications related to DeltaPocValSt:

- Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], such that it a) swaps the meaning of positive and negative to align with the semantics of other sign flags and b) associates strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] with DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] instead of with an “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )”.

- Swap the sign assignment of DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] in equation (155).

- Change the subtraction to addition when assigning RefPicPocList[ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] in equation (205).

# Introduction

The authors claim that in the VVC draft specification v10, a sign flag value equal to 0 means that the corresponding value is positive and equal to 1 means that it is negative, e.g., mvd_sign_flag[ compIdx ]. However, this is asserted to be the opposite for DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] and the associated sign flag strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ]. In the text, strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 1 means a positive or a zero value of DeltaPocValSt [ listIdx ][ rplsIdx ][ i ], and strp_entry_sign_flag [ listIdx ][ rplsIdx ][ i ] equal to 0 indicates a negative DeltaPocValSt[ listIdx ][ rplsIdx ][ i ].

In addition, the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] associates the flag value with an “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )”. However, the “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )” is not necessarily a value, and associating a sign flag for that seems inaccurate to the proponents.

Moreover, when constructing the reference picture list RefPicPocList [ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ], DeltaPocValSt [ i ][ RplsIdx[ i ] ][ j ] is subtracted from pocBase. It would be cleaner to add a delta POC value to pocBase than to subtract it from pocBase.

As an example, assume that the POC value of the current picture is equal to 100 and that RefPicPocList[ 0 ][ 0 ] is equal to 101. In the current draft, DeltaPocValSt[ 0 ][ RplsIdx[ 0 ] ][ 0 ] is equal to −1 and subtracted from 100. We believe that it is cleaner if DeltaPocValSt[ 0 ][ RplsIdx[ 0 ] ][ 0 ] is equal to +1 and added to 100 instead.

# Proposal

This contribution proposes the following three modifications related to DeltaPocValSt:

- Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], such that it a) swaps the meaning of positive and negative to align with the semantics of other sign flags and b) associates strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] with DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] instead of with an “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )”.

- Swap the sign assignment of DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] in equation (155).

- Change the subtraction to addition when assigning RefPicPocList [ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] in equation (205).

The three proposed modifications on top of the VVC draft (JVET-R2001-v10) are shown below with added text marked in yellow and removed text in red strike-through.

7.4.10 Reference picture list structure semantics

strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 10 specifies that i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx ) has a valueDeltaPocValSt[ listIdx ][ rplsIdx ][ i ] is greater than or equal to 0. strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] equal to 01 specifies that the i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx ) has a valueDeltaPocValSt[ listIdx ][ rplsIdx ][ i ] is less than 0. When not present, the value of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] is inferred to be equal to 10.

The list DeltaPocValSt[ listIdx ][ rplsIdx ] is derived as follows:

for( i = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++ )
	if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ]  &&  st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] )	(155)
		DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] = ( strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] ) ?
				0 − AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] : 0 − AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ]

8.3.2 Decoding process for reference picture lists construction

…

for( i = 0; i < 2; i++ ) {
	for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
		if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
			if( st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
				RefPicPocList[ i ][ j ] = pocBase +− DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ]
				if( there is a reference picture picA in the DPB with the same nuh_layer_id as the current picture
							and PicOrderCntVal equal to RefPicPocList[ i ][ j ] )
					RefPicList[ i ][ j ] = picA
				else
					RefPicList[ i ][ j ] = "no reference picture"	(205)
				pocBase = RefPicPocList[ i ][ j ]
			} else {
			…

The following text is provided as supplementary information for the semantics of a sign flag:

mvd_sign_flag[ compIdx ] specifies the sign of a motion vector component difference as follows:

- If mvd_sign_flag[ compIdx ] is equal to 0, the corresponding motion vector component difference has a positive value.

- Otherwise (mvd_sign_flag[ compIdx ] is equal to 1), the corresponding motion vector component difference has a negative value.

When mvd_sign_flag[ compIdx ] is not present, it is inferred to be equal to 0.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).