| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0050-v1 |
| --- | --- |

| Title: | AHG9: On general constraints information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Ye-Kui Wang2, Li Zhang2, Kai Zhang2, Kui Fan2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | zhipin.deng@bytedance.com yekui.wang@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com kui.fan@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on the general constraints information (GCI):

- Add a new flag, named gci_present_flag, in the profile_tier_level( ) syntax structure to condition the presence of general_constraint_info( ) syntax structure.

  - When gci_present_flag equal to 0 for a profile_tier_level( ) syntax structure with profileTierPresentFlag equal to 1, the value of max_bitdepth_constraint_idc is inferred to be equal to 8, the value of max_chroma_format_constraint_idc is inferred to be equal to 3, and the value of the each of the other syntax elements in the general_constraint_info( ) syntax structure starting from general_non_packed_constraint_flag to no_aps_constraint_flag, inclusive, is inferred to be equal to 0.

  - To keep the starting position of general_level_idc to be a byte-aligned position, move the byte alignment check outside the general_constraint_info( ), change the GCI syntax elements gci_num_reserved_bytes to gci_num_reserved_bits, coded with u(11) instead of u(8), and change the GCI syntax element gci_reserved_byte[ i ] to gci_reserved_bit[ i ], coded with u(1) instead of u(8).

- Add the following new GCI flags in the general_constraint_info( ) syntax structure:

  - Add no_weighted_pred_constraint_flag to constrain the value of sps_weighted_pred_flag.

  - Add no_weighted_bipred_constraint_flag to constrain the value of sps_weighted_bipred_flag.

  - Add no_virtual_boundaries_constraint_flag to constrain the value of sps_virtual_boundaries_enabled_flag.

  - Add no_explicit_scaling_list_constraint_flag to constrain the value of sps_explicit_scaling_list_enabled_flag.

- Change the semantics of the following GCI flags to be as follows:

  - The value of max_bitdepth_constraint_idc is required to be in the range of 0 to 8, inclusive.

  - one_tile_per_pic_constraint_flag equal to 1 specifies that the value of NumTilesInPic that applies to each picture shall be equal to 1.

  - one_subpic_per_pic_constraint_flag equal to 1 specifies that the value of sps_num_subpics_minus1 shall be equal to 0.

- Use some GCI flags to constrain the values of other GCI flags as follows:

  - When no_transform_skip_constraint_flag is equal to 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

  - When general_one_picture_only_constraint_flag is equal to 1, the values of single_layer_constraint_flag, intra_only_constraint_flag, no_mixed_nalu_types_in_pic_constraint_flag, no_trail_constraint_flag, no_stsa_constraint_flag, no_rasl_constraint_flag, and no_radl_constraint_flag shall all be equal to 1.

  - When intra_only_constraint_flag is equal to 1, the values of all_layers_independent_constraint_flag, no_ref_pic_resampling_constraint_flag, no_res_change_in_clvs_constraint_flag, no_weighted_pred_constraint_flag, no_weighted_bipred_constraint_flag, and no_sbt_constraint_flag shall all be equal to 1.

  - When single_layer_constraint_flag is euqal to 1, the value of all_layers_independent_constraint_flag shall be equal to 1.

  - When no_lmcs_constraint_flag, no_explicit_scaling_list_constraint_flag,  no_ccalf_constraint_flag is equal to 0, the value of no_aps_constraint_flag shall be equal to 0.

- Use some GCI flags to constrain the values of some SPS/PPS/PH/SH syntax elements, but specified in the GCI semantics, as follows:

  - When no_aps_constraint_flag is equal to 1, the values of ph_num_alf_aps_ids_luma, slice_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_alf_cb_flag, sh_alf_cr_flag, and sps_ccalf_enabled_flag shall all be equal to 1.

  - When all_layers_independent_constraint_flag is equal to 1, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0.

  - When one_slice_per_pic_constraint_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1 and the value of pps_num_slices_in_pic_minus1 shall be equal to 0.

  - When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint flag are both equal to 1, the value of pps_no_pic_partition_flag shall be equal to 1.

- Add a constrain such that at least one of the no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag shall be equal to 0.

# Introduction

In the current draft text, the following issues on GCI syntax were observed:

- Currently, whenever there is a PTL syntax structure with profileTierPresentFlag equal to 1, all the general constraint flags/fields are signalled in that PTL syntax structure. Considering that PTL syntax structures may be included in DCI (one or more times, each time with profileTierPresentFlag equal to 1), in VPS (one or more times, the first time with profileTierPresentFlag equal to 1, other times with profileTierPresentFlag equal to either 0 or 1), and in SPS (zero or one time, with profileTierPresentFlag equal to 1), potentially there can be lots of redundant signalling of the general constraint flags/fields. Furthermore, it is possible that none of the general constraints may apply in addition to those imposed by the profile constraints while in this case at least one set of general constraint flags/fields is still signalled for the bitstream of each OLS.

- There are some coding tools/features, e.g., weighted prediction, explicit scaling list, virtual boundaries that don't have general constraint flags.

- The semantics of the followings GCI syntax elements can be described more accurately to help encoders (encoder implementers) to put the right values:

  - The coding of max_bitdepth_constraint_idc is u(4), while the value range of sps_bit_depth_minus8 is 0..8. Therefore it makes sense to specify that the value of max_bitdepth_constraint_idc to be not greater than 8.

  - Unlike most other GCI flags, the semantics of one_tile_per_pic_constraint_flag and one_subpic_per_pic_constraint_flag don't impose a constraint on a syntax element or variable, while it is possible to do that.

- The following GCI flags are highly related to each other, and their values should depend on each other according to their semantics, but currently not constrained, which may cause problems when conflict values are set:

  - The value of no_bdpcm_constraint_flag is not constrained on no_transform_skip_constraint_flag.

  - The values of single_layer_constraint_flag, intra_only_constraint_flag, no_mixed_nalu_types_in_pic_constraint_flag, no_trail_constraint_flag, no_stsa_constraint_flag, no_rasl_constraint_flag, and no_radl_constraint_flag are not constrained on general_one_picture_only_constraint_flag.

  - The values of all_layers_independent_constraint_flag, no_ref_pic_resampling_constraint_flag, no_res_change_in_clvs_constraint_flag, no_weighted_pred_constraint_flag, no_weighted_bipred_constraint_flag, and no_sbt_constraint_flag are not constrained on intra_only_constraint_flag.

  - The value of all_layers_independent_constraint_flag is not constrained on single_layer_constraint_flag.

  - The value of no_aps_constraint_flag is not constrained on no_lmcs_constraint_flag, no_explicit_scaling_list_constraint_flag, and no_ccalf_constraint_flag.

- The following SPS/PPS/PH/SH syntax elements are highly related to and their values depend on the corresponding GCI flags but currently not constrained yet, which may cause problems when conflict values are set:

- When no_aps_constraint_flag is equal to 1, there should be no ALF APS, i.e., the values of ph_num_alf_aps_ids_luma, slice_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_alf_cb_flag, and sh_alf_cr_flag shall be equal to 1.

- When all_layers_independent_constraint_flag is equal to 1, there should be no inter-layer reference pictures present, i.e., the value of sps_inter_layer_ref_pics_present_flag shall be equal to 0.

- When one_slice_per_pic_constraint_flag is equal to 1, there should be only one slice in a picture and this slice shall be a rectangular slice instead of a raster-scan slice, i.e., pps_rect_slice_flag shall be equal to 1 and pps_num_slices_in_pic_minus1 shall be equal to 0.

- When one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint flag are equal to 1, there should be no picture partition, i.e., the value of pps_no_pic_partition_flag shall be equal to 1.

- Currently there is no constraint on the values of no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag. When all these three flags are set to 1, there is no random access point, and the bitstream won't be a conforming bitstream.

# Proposal

The changes as summarized in the abstract are proposed.

The proposed spec text changes for these proposals are provided below. The proposed text changes are relative to the latest VVC draft text in JVET-R2001-vA/v10. The most relevant parts that have been added or modified are highlighted in turquoise, and the most relevant removed parts are highlighted in yellow strikethrough in red fonts.

The syntax structure profile_tier_level ( ) is changed as follows:

| profile_tier_level( profileTierPresentFlag, maxNumSubLayersMinus1 ) { | Descriptor |
| --- | --- |
| if( profileTierPresentFlag ) { |  |
| general_profile_idc | u(7) |
| general_tier_flag | u(1) |
| gci_present_flag | u(1) |
| if( gci_present_flag ) |  |
| general_constraint_info( ) |  |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| } |  |
| general_level_idc | u(8) |
| if( profileTierPresentFlag ) { |  |
| ptl_num_sub_profiles | u(8) |
| for( i = 0; i < ptl_num_sub_profiles; i++ ) |  |
| general_sub_profile_idc[ i ] | u(32) |
| } |  |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| ptl_sublayer_level_present_flag[ i ] | u(1) |
| while( !byte_aligned( ) ) |  |
| ptl_alignment_zero_bit | f(1) |
| for( i = 0; i < maxNumSubLayersMinus1; i++ ) |  |
| if( ptl_sublayer_level_present_flag[ i ] ) |  |
| sublayer_level_idc[ i ] | u(8) |
| } |  |

And the semantics are changed as follows:

…

gci_present_flag equal to 1 specifies that the general_constraint_info( ) syntax structure is present in the profile_tier_level( ) syntax structure when profileTierPresentFlag is equal to 1. gci_present_flag equal to 0 specifies that the general_constraint_info( ) syntax structure is not present in the profile_tier_level( ) syntax structure.

When gci_present_flag equal to 0 for a profile_tier_level( ) syntax structure with profileTierPresentFlag equal to 1, the value of max_bitdepth_constraint_idc is inferred to be equal to 8, the vlaue of max_chroma_format_constraint_idc is inferred to be equal to 3, and the value of the each of the other syntax elements in the general_constraint_info( ) syntax structure starting from general_non_packed_constraint_flag to no_aps_constraint_flag, inclusive, is inferred to be equal to 0.

ptl_alignment_zero_bit shall be equal to 0.

…

The syntax structure general_constraint_info ( ) is changed as follows:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| general_one_picture_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| single_layer_constraint_flag | u(1) |
| all_layers_independent_constraint_flag | u(1) |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_weighted_pred_constraint_flag | u(1) |
| no_weighted_bipred_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_explicit_scaling_list_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_chroma_qp_offset_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_virtual_boundaries_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bitsytes | u(811) |
| for( i = 0; i < gci_num_reserved_bitsytes; i++ ) |  |
| gci_reserved_bityte[ i ] | u(81) |
| } |  |

And the semantics are changed as follows:

…

general_one_picture_only_constraint_flag equal to 1 specifies that there is only one coded picture in the bitstream. general_one_picture_only_constraint_flag equal to 0 does not impose such a constraint.

intra_only_constraint_flag equal to 1 specifies that sh_slice_type shall be equal to I. intra_only_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of intra_only_constraint_flag shall be equal to 1. When general_one_picture_only_constraint_flag is equal to 1, the value of intra_only_constraint_flag shall be equal to 1.

max_bitdepth_constraint_idc specifies that sps_bit_depth_minus8 shall be in the range of 0 to max_bitdepth_constraint_idc, inclusive. The value of max_bitdepth_constraint_idc shall be in the range of 0 to 8, inclusive.

max_chroma_format_constraint_idc specifies that sps_chroma_format_idc shall be in the range of 0 to max_chroma_format_constraint_idc, inclusive.

single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0. single_layer_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1.

all_layers_independent_constraint_flag equal to 1 specifies that vps_all_independent_layers_flag shall be equal to 1 and sps_inter_layer_ref_pics_present_flag shall be equal to 0. all_layers_independent_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag or single_layer_constraint_flag is equal to 1, the value of all_layers_independent_constraint_flag shall be equal to 1.

no_ref_pic_resampling_constraint_flag equal to 1 specifies that sps_ref_pic_resampling_enabled_flag shall be equal to 0. no_ref_pic_resampling_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_ref_pic_resampling_constraint_flag shall be equal to 1.

no_res_change_in_clvs_constraint_flag equal to 1 specifies that sps_res_change_in_clvs_allowed_flag shall be equal to 0. no_res_change_in_clvs_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_res_change_in_clvs_constraint_flag shall be equal to 1.

one_tile_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one tile, the value of NumTilesInPic that applies to each picture shall be equal to 1. one_tile_per_pic_constraint_flag equal to 0 does not impose such a constraint.

pic_header_in_slice_header_constraint_flag equal to 1 specifies that each picture shall contain only one slice and the value of sh_picture_header_in_slice_header_flag in each slice shall be equal to 1. pic_header_in_slice_header_constraint_flag equal to 0 does not impose such a constraint.

one_slice_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one slice pps_rect_slice_flag shall be equal to 1 and pps_num_slices_in_pic_minus1 shall be equal to 0, and, when one_tile_per_pic_constraint_flag is also equal to 1, pps_no_pic_partition_flag shall be equal to 1. one_slice_per_pic_constraint_flag equal to 0 does not impose such a constraint. When pic_header_in_slice_header_constraint_flag is equal to 1, the value of one_slice_per_pic_constraint_flag shall be equal to 1.

one_subpic_per_pic_constraint_flag equal to 1 specifies that each picture shall contain only one subpicture sps_num_subpics_minus1 shall be equal to 0. one_subpic_per_pic_constraint_flag equal to 0 does not impose such a constraint. When one_slice_per_pic_constraint_flag is equal to 1, the value of one_subpic_per_pic_constraint_flag shall be equal to 1.

no_qtbtt_dual_tree_intra_constraint_flag equal to 1 specifies that sps_qtbtt_dual_tree_intra_flag shall be equal to 0. no_qtbtt_dual_tree_intra_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_qtbtt_dual_tree_intra_constraint_flag shall be equal to 1.

no_partition_constraints_override_constraint_flag equal to 1 specifies that sps_partition_constraints_override_enabled_flag shall be equal to 0. no_partition_constraints_override_constraint_flag equal to 0 does not impose such a constraint.

no_sao_constraint_flag equal to 1 specifies that sps_sao_enabled_flag shall be equal to 0. no_sao_constraint_flag equal to 0 does not impose such a constraint.

no_alf_constraint_flag equal to 1 specifies that sps_alf_enabled_flag shall be equal to 0. no_alf_constraint_flag equal to 0 does not impose such a constraint.

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0 or no_alf_constraint_flag is equal 1, the value of no_ccalf_constraint_flag shall be equal to 1.

…

no_mip_constraint_flag equal to 1 specifies that sps_mip_enabled_flag shall be equal to 0.  no_mip_constraint_flag equal to 0 does not impose such a constraint.

no_weighted_pred_constraint_flag equal to 1 specifies that sps_weighted_pred_flag shall be equal to 0.  no_weighted_pred_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_weighted_pred_constraint_flag shall be equal to 1.

no_weighted_bipred_constraint_flag equal to 1 specifies that sps_weighted_bipred_flag shall be equal to 0.  no_weighted_bipred_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_weighted_bipred_constraint_flag shall be equal to 1.

…

no_mts_constraint_flag equal to 1 specifies that sps_mts_enabled_flag shall be equal to 0. no_mts_constraint_flag equal to 0 does not impose such a constraint.

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_sbt_constraint_flag shall be equal to 1.

…

no_transform_skip_constraint_flag equal to 1 specifies that sps_transform_skip_enabled_flag shall be equal to 0. no_transform_skip_constraint_flag equal to 0 does not impose such a constraint.

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

no_palette_constraint_flag equal to 1 specifies that sps_palette_enabled_flag shall be equal to 0. no_palette_constraint_flag equal to 0 does not impose such a constraint.

no_act_constraint_flag equal to 1 specifies that sps_act_enabled_flag shall be equal to 0. no_act_constraint_flag equal to 0 does not impose such a constraint.

no_lmcs_constraint_flag equal to 1 specifies that sps_lmcs_enabled_flag shall be equal to 0. no_lmcs_constraint_flag equal to 0 does not impose such a constraint.

no_explicit_scaling_list_constraint_flag equal to 1 specifies that sps_explicit_scaling_list_enabled_flag shall be equal to 0. no_explicit_scaling_list_constraint_flag equal to 0 does not impose such a constraint.

no_cu_qp_delta_constraint_flag equal to 1 specifies that pps_cu_qp_delta_enabled_flag shall be equal to 0. no_cu_qp_delta_constraint_flag equal to 0 does not impose such a constraint.

no_chroma_qp_offset_constraint_flag equal to 1 specifies that pps_cu_chroma_qp_offset_list_enabled_flag shall be equal to 0. no_chroma_qp_offset_constraint_flag equal to 0 does not impose such a constraint.

no_dep_quant_constraint_flag equal to 1 specifies that sps_dep_quant_enabled_flag shall be equal to 0. no_dep_quant_constraint_flag equal to 0 does not impose such a constraint.

no_sign_data_hiding_constraint_flag equal to 1 specifies that sps_sign_data_hiding_enabled_flag shall be equal to 0. no_sign_data_hiding_constraint_flag equal to 0 does not impose such a constraint.

no_virtual_boundaries_constraint_flag equal to 1 specifies that sps_virtual_boundaries_enabled_flag shall be equal to 0. no_virtual_boundaries_constraint_flag equal to 0 does not impose such a constraint.

…

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that pps_mixed_nalu_types_in_pic_flag shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

no_trail_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to TRAIL_NUT present in OlsInScope. no_trail_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of no_trail_constraint_flag shall be equal to 1.

no_stsa_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to STSA_NUT present in OlsInScope. no_stsa_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of no_stsa_constraint_flag shall be equal to 1.

no_rasl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RASL_NUT present in OlsInScope. no_rasl_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of no_rasl_constraint_flag shall be equal to 1.

no_radl_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to RADL_NUT present in OlsInScope. no_radl_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of no_radl_constraint_flag shall be equal to 1.

no_idr_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to IDR_W_RADL or IDR_N_LP present in OlsInScope. no_idr_constraint_flag equal to 0 does not impose such a constraint.

no_cra_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to CRA_NUT present in OlsInScope. no_cra_constraint_flag equal to 0 does not impose such a constraint.

no_gdr_constraint_flag equal to 1 specifies that sps_gdr_enabled_flag shall be equal to 0. no_gdr_constraint_flag equal to 0 does not impose such a constraint.

At least one of no_gdr_constraint_flag, no_idr_constraint_flag, and no_cra_constraint_flag shall be equal to 0.

no_aps_constraint_flag equal to 1 specifies that there shall be no NAL unit with nuh_unit_type equal to PREFIX_APS_NUT or SUFFIX_APS_NUT present in OlsInScope, and sps_lmcs_enabled_flag, sps_explicit_scaling_list_enabled_flag,  sps_ccalf_enabled_flag, ph_num_alf_aps_ids_luma, slice_num_alf_aps_ids_luma, ph_alf_cb_flag, ph_alf_cr_flag, sh_alf_cb_flag, and sh_alf_cr_flag shall both all be equal to 0. no_aps_constraint_flag equal to 0 does not impose such a constraint. When no_lmcs_constraint_flag, no_explicit_scaling_list_constraint_flag, or no_ccalf_constraint_flag is equal to 0, the value of no_aps_constraint_flag shall be equal to 0.

gci_alignment_zero_bits shall be equal to 0.

gci_num_reserved_bitsytes specifies the number of the reserved constraint bitsytes. The value of gci_num_reserved_bitsytes shall be 0. Other values of gci_num_reserved_bitsytes are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

gci_reserved_bityte[ i ] may have any value. Its presence and value do not affect decoder conformance to profiles specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore the values of all the gci_reserved_bityte[ i ] syntax elements.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).