| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0052-v1 |
| --- | --- |

| Title: | AHG9: Removing separate colour plane coding from VVCv1 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 (Bytedance) 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China Jill Boyce (Intel) | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com jill.boyce@intel.com |
| Source: | Bytedance Inc., Intel |  |  |

_____________________________

# Abstract

This contribution proposes to remove the support of the separate colour plane coding from VVCv1, by removing the syntax elements sps_separate_colour_plane_flag and sh_colour_plane_id and the related texts. If such functionality is desired in the future, the SPS and SH extension mechanisms could be used. Another option is to rename these syntax elements to be reserved bits.

It is asserted that there is no current or a planned profile that supports the separate colour plane coding mode, and currently at least the following aspects in the latest VVC draft text are not fully specified (or broken) when the separate colour plane coding mode is on:

- The specification in clause 6.3.3 of each of the following being a partitioning:

–	The division of each picture into subpictures

–	The division of each picture into tile columns

–	The division of each picture into tile rows

–	The division of each tile into CTUs

–	The division of each picture into slices

–	The division of each slice into CTUs

- The specification of the decoding order of NAL units in the bitstream in clause 7.4.2.4

- All places in the text that involve subpicture index, tile column index, tile row index, tile index, slice index, CTU index or address (need to be clearly specified to be of which colour plane)

The proposed text changes are included in an attachment to this contribution.

# Patent rights declaration(s)

Bytedance Inc. and Intel do not have any current or pending patent rights relating to the technology described in this contribution (to the extent of the personal awareness of the authors).