| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0057 |
| --- | --- |

| Title: | AHG8/AHG9: On signaling reference picture resampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The following modifications to the syntax elements related to reference picture resampling are proposed:

- When sps_ref_pic_resampling_enabled_flag equal to 1, a slice may refer to a reference picture with a different spatial resolution or a different scaling window in a reference picture list.

- When sps_res_change_in_clvs_allowed_flag is equal to 0, the values of pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples are inferred to be equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively, without explicit signaling.

# Proposal - 1

In the VVC draft (JVET-R2001-vA), two flags sps_ref_pic_resampling_enabled_flag and sps_res_change_in_clvs_allowed_flag are present in SPS, to indicate whether or not the reference picture resamplig is enabled and the picture resolution can be changed within a CLVS. When sps_ref_pic_resampling_enabled_flag is equal to 0, sps_res_change_in_clvs_allowed_flag is inferred to be equal to 0, so that the picture resolution cannot be changed when the reference picture resampling is disabled.

The asserted issue is that the current semantics of sps_ref_pic_resampling_enabled_flag do not clearly specifies the cases with sps_ref_pic_resampling_enabled_flag equal to 1. When sps_ref_pic_resampling_enabled_flag is equal to 1, a reference picture with a different spatial resolution as well as a reference picture with a different scaling window and the same spatial resolution may be referenced by the picture. Even though all reference pictures have the same scaling window within a CLVS, the value of  sps_ref_pic_resampling_enabled_flag shall be equal to 1 when a reference picture has a different spatial resolution. Reversely, although all reference pictures have the same spatial resolution, the value of  sps_ref_pic_resampling_enabled_flag shall be equal to 1 when a reference picture has a different scaling window. To clearly specifies the cases, it is proposed to modify the semantics of sps_ref_pic_resampling_enabled_flag as follows:

Proposed Text (Yellow part is added):

sps_ref_pic_resampling_enabled_flag equal to 1 specifies that reference picture resampling is enabled and one or more slices of pictures in the CLVS may refer to a reference picture with a different spatial resolution or a different scaling window in an active entry of a reference picture list. sps_ref_pic_resampling_enabled_flag equal to 0 specifies that reference picture resampling is disabled and no slice of pictures in the CLVS refers to a reference picture with a different spatial resolution or a different scaling window in an active entry of a reference picture list.

NOTE 2 – When sps_ref_pic_resampling_enabled_flag is equal to 1, for a current picture the reference picture with a different spatial resolution or a different scaling window may either belong to the same layer or a different layer than the layer containing the current picture.

# Proposal - 2

When the value of sps_res_change_in_clvs_allowed_flag is equal to 0, signaling pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples in PPS can be skipped, because the values of pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples can be inferred to be equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples. Note that there are already the constraints that, when sps_res_change_in_clvs_allowed_flag equal to 0, the value of pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples shall be equal to sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples, respectively.

Therefore, the following modifications (in the below paragraph) to picture width and height signaling in PPS are proposed with a new flag pps_res_change_in_clvs_allowed_flag to avoid parsing depencency on SPS. The modification can save roughly 43 bits in PPS, when the picture resolution is 4K. Also, the proposed modification can remove unnecessary constraint checks. It is noticeable that the syntax elements of the conformance window have the same inferrence rule from SPS to PPS.

Proposed Text:

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| pps_res_change_in_clvs_allowed_flag | u(1) |
| if( pps_res_change_in_clvs_allowed_flag ) { |  |
| pps_pic_width_in_luma_samples | ue(v) |
| pps_pic_height_in_luma_samples | ue(v) |
| } |  |
| … |  |
| } |  |

pps_res_change_in_clvs_allowed_flag shall be equal to sps_res_change_in_clvs_allowed_flag.

pps_pic_width_in_luma_samples specifies the width of each decoded picture referring to the PPS in units of luma samples. pps_pic_width_in_luma_samples shall not be equal to 0, shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to sps_pic_width_max_in_luma_samples.

When sps_res_change_in_clvs_allowed_flag equal to 0, the value of pps_pic_width_in_luma_samples shall be equal to sps_pic_width_max_in_luma_samples.

When not present, the value of pps_pic_width_in_luma_samples is inferred to equal to sps_pic_width_max_in_luma_samples.

When sps_ref_wraparound_enabled_flag is equal to 1, the value of ( CtbSizeY / MinCbSizeY + 1 ) shall be less than or equal to the value of ( pps_pic_width_in_luma_samples / MinCbSizeY − 1 ).

pps_pic_height_in_luma_samples specifies the height of each decoded picture referring to the PPS in units of luma samples. pps_pic_height_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ), and shall be less than or equal to sps_pic_height_max_in_luma_samples.

When sps_res_change_in_clvs_allowed_flag equal to 0, the value of pps_pic_height_in_luma_samples shall be equal to sps_pic_height_max_in_luma_samples.

When not present, the value of pps_pic_height_in_luma_samples is inferred to equal to sps_pic_height_max_in_luma_samples.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).