| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0058 |
| --- | --- |

| Title: | AHG9: Additional General Constraint Flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Fabrice Le Leannec Tangi Poirier Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes adding general constraint flags for the following coding tools: MTT, Weighted Prediction, long term reference picture prediction, scaling lists, and maximum transform size.

# Introduction

The general constraint flags are indicators of the used coding tools/features of a certain profiles/sub-profiles. Although in VVC draft 9 many constraint flags are defined, still few are missing. Specifically, it is identified that the following tools, whose SPS flag is given below, do not have their constraint flags.

| Coding tool/feature | SPS flag |
| --- | --- |
| MTT | sps_max_mtt_hierarchy_depth_intra_slice_luma sps_max_mtt_hierarchy_depth_inter_slice sps_max_mtt_hierarchy_depth_intra_slice_chroma |
| Maximum transform unit size | sps_max_luma_transform_size_64_flag |
| Scaling list | sps_explicit_scaling_list_enabled_flag |
| Long terms reference picture prediction | sps_long_term_ref_pics_flag |
| Weighted prediction | sps_weighted_pred_flag sps_weighted_bipred_flag |

This contribution proposes adding a constraint flag for each of these tools. This leads to easier definition of profiles/sub-profile that do not employ any of tools. This is especially useful for MTT and weighted prediction since 3 and 2 SPS flags are respectively needed to deactivate each of them.

# Proposed Method

The change to the current specification is provided below (added part in blue):

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| general_one_picture_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| single_layer_constraint_flag | u(1) |
| all_layers_independent_constraint_flag | u(1) |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_mtt_constraint_flag | u(1) |
| max_luma_transform_size_32_constraint_flag | u(1) |
| no_scaling_list_constraint_flag | u(1) |
| no_long_term_ref_pic_constraint_flag | u(1) |
| no_weighted_pred_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_chroma_qp_offset_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

The semantics of these flags are:

no_mtt_constraint_flag equal to 1 specifies that sps_max_mtt_hierarchy_depth_intra_slice_luma, sps_max_mtt_hierarchy_depth_inter_slice and sps_max_mtt_hierarchy_depth_intra_slice_chroma shall be equal to 0. no_mtt_constraint_flag equal to 0 does not impose such a constraint.

max_luma_transform_size_32_constraint_flag equal to 1 specifies that sps_max_luma_transform_size_64_flag shall be equal to 0. max_luma_transform_size_32_constraint_flag equal to 0 does not impose such a constraint.

no_scaling_list_constraint_flag equal to 1 specifies that sps_explicit_scaling_list_enabled_flag shall be equal to 0. no_scaling_list_constraint_flag equal to 0 does not impose such a constraint.

no_long_term_ref_pic_constraint_flag equal to 1 specifies that sps_long_term_ref_pics_flag shall be equal to 0. no_long_term_ref_pic_constraint_flag equal to 0 does not impose such a constraint.

no_weighted_pred_constraint_flag equal to 1 specifies that sps_weighted_pred_flag and sps_weighted_bipred_flag shall be equal to 0. no_weighted_pred_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).