| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0059 |
| --- | --- |

| Title: | AHG9: Fixing the Semantics of no_bdpcm_constraint_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Fabrice Le Leannec Tangi Poirier Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes imposing the value of no_bdpcm_constraint_flag to 1 when no_transform_skip_constraint_flag is 1, where BDPCM is currently disabled if transform skip is disabled. This is to be inline with other constraint flags and to reduce the conformance testing effort.

# Introduction

In VVC draft 9, there are many examples where certain constraint flags are set to one according to the value of another flag. For example, the chroma related general constraint flags are set to one when max_chroma_format_constraint_idc is zero, which indicates monochrome coding. Similarly, when intra_only_constraint_flag is set to one, all inter-related constraint flags are set to 1. A more closer example is no_ccalf_constraint_flag, whose value is set to one if no_alf_constraint_flag is one:

no_ccalf_constraint_flag equal to 1 specifies that sps_ccalf_enabled_flag shall be equal to 0. no_ccalf_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0 or no_alf_constraint_flag is equal 1, the value of no_ccalf_constraint_flag shall be equal to 1.

However, the transform skip and BDPCM constraint flags are not related, although BDPCM is only enabled when transform skip is enabled. Therefore, it is proposed here to constrain the value of no_bdpcm_constraint_flag to one when no_transform_skip_constraint_flag is one.

# Proposed Method

The proposed changes are (added part in blue):

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is is equal 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).