| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0062-r1 |
| --- | --- |

| Title: | AHG9: On no_tsrc_constraint_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam NASER Fabrice Le Leannec Tangi Poirier Philippe de Lagrange | Tel: Email: | karam.naser@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

In VVC draft 9, a new constraint flag “no_tsrc_constraint_flag” for defining a profile/sub-profiles where only regular residual coding (RRC) is used, i.e., transform skip residual coding (TSRC) is disabled. It is observed that when transform skip is disabled (sps_transform_skip_enabled_flag is 0), this constraint flag cannot be 1 since its corresponding SH flag (sh_ts_residual_coding_disable_flag) is inferred to be 0. Thus, although transform skip is never used, TSRC cannot be disabled by its SH flag. When “no_tsrc_constraint_flag” is one, both dependent quantization and sign data hiding cannot be used, but they can still be activated by their SPS flag. Therefore, this contribution proposes methods to well-define “no_tsrc_constraint_flag” and properly signal SPS values for such a profile. Finally, it proposes to set the value of no_tsrc_constraint_flag to one when no_transform_skip_constraint_flag is 1.

# Introduction

In JVET-R0483, it was proposed to condition sh_ts_residual_coding_disable_flag on transform skip SPS flag (sps_transform_skip_enabled_flag) and also to disallow the combination of RRC with either dependent quantization or sign data hiding. The corresponding specification text is:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| … |  |

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. sh_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When sh_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.

When transform skip is disabled (sps_transform_skip_enabled_flag = 0), sh_ts_residual_coding_disabled_flag is not signaled but inferred to zero. That is, TSRC is virtually enabled. That is, even when it is inferred to zero, TSRC is never used because transform skip is not used. The corresponding specification text is:

| transform_unit( x0, y0, tbWidth, tbHeight, treeType, subTuIndex, chType ) { | Descriptor |
| --- | --- |
| … |  |
| if( tu_y_coded_flag[ x0 ][ y0 ] && treeType != DUAL_TREE_CHROMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 0 ] && tbWidth <= MaxTsSize && tbHeight <= MaxTsSize && ( IntraSubPartitionsSplitType = = ISP_NO_SPLIT ) && !cu_sbt_flag ) |  |
| transform_skip_flag[ x0 ][ y0 ][ 0 ] | ae(v) |
| if( !transform_skip_flag[ x0 ][ y0 ][ 0 ] ¦ ¦ sh_ts_residual_coding_disabled_flag ) |  |
| residual_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| else |  |
| residual_ts_coding( x0, y0, Log2( tbWidth ), Log2( tbHeight ), 0 ) |  |
| } |  |
| if( tu_cb_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 1 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 1 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 1 ] ¦ ¦ sh_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 1 ) |  |
| } |  |
| if( tu_cr_coded_flag[ xC ][ yC ] && treeType != DUAL_TREE_LUMA && !( tu_cb_coded_flag[ xC ][ yC ] && tu_joint_cbcr_residual_flag[ xC ][ yC ] ) ) { |  |
| if( sps_transform_skip_enabled_flag && !BdpcmFlag[ x0 ][ y0 ][ 2 ] && wC <= MaxTsSize && hC <= MaxTsSize && !cu_sbt_flag ) |  |
| transform_skip_flag[ xC ][ yC ][ 2 ] | ae(v) |
| if( !transform_skip_flag[ xC ][ yC ][ 2 ] ¦ ¦ sh_ts_residual_coding_disabled_flag ) |  |
| residual_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| else |  |
| residual_ts_coding( xC, yC, Log2( wC ), Log2( hC ), 2 ) |  |
| } |  |
| } |  |

TSRC, represented by the coding structure “residual_ts_coding” is never used when sps_transform_skip_enabled_flag is set to 0. Therefore, even if sh_ts_residual_coding_disabled_flag is inferred to one, it actually behaves like if it is inferred to zero. However, the general constraint flag “no_tsrc_constraint_flag” cannot be correctly set as it depends on sh_ts_residual_coding_disabled_flag.

It is also observed that when a profile/sub-profile with “no_tsrc” is used, both dependent quantization and sign data hiding cannot be used. However, their SPS flags can remain to be one. This is because of the lack of an SPS flag to control TSRC. Therefore, this contribution proposes adding an SPS flag for TSRC, where dependent quantization and sign-data hiding is disabled if any TSRC is disabled.

Finally, when transform skip is disabled at the general constraint flag level (no_transform_skip_constraint_flag = 1), it is clear that TSRC is not used. Therefore, no_tsrc_constraint_flag shall be to 1 when no_transform_skip_constraint_flag is 1. This is similar to no_sbtmvp_constraint_flag whose value is set to one if no_temporal_mvp_constraint_flag is one:

no_sbtmvp_constraint_flag equal to 1 specifies that sps_sbtmvp_enabled_flag shall be equal to 0. no_sbtmvp_constraint_flag equal to 0 does not impose such a constraint. When no_temporal_mvp_constraint_flag is equal to 1, the value of no_sbtmvp_constraint_flag shall be equal to 1.

# Proposed Method 1: Modified Inference Role of sh_ts_residual_coding_disable_flag

In this method, the inference role for sh_ts_residual_coding_disable_flag is modified such that when transform skip is disabled, TSRC is disabled as well. The corresponding specification changes are:

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. sh_ts_residual_coding_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure is used to parse the residual samples of a transform skip block for the current slice. When sh_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0 !sps_transform_skip_enabled_flag.

# Proposed Method 2: SPS flag for TSRC

Adding an SPS for TSRC solves both the problem of setting the constraint flag as well as it forces the encoder to disable both dependent quantization and sign-data hiding for when TSRC is disabled. The corresponding specification changes are:

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| sps_transform_skip_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag ) { |  |
| log2_transform_skip_max_size_minus2 | ue(v) |
| sps_tsrc_disabled_flag | u(1) |
| … |  |
| } |  |
| … |  |
| } |  |
| … |  |
| if (!sps_tsrc_disabled_flag) |  |
| sps_dep_quant_enabled_flag | u(1) |
| if( !sps_dep_quant_enabled_flag && !sps_tsrc_disabled_flag) |  |
| sps_sign_data_hiding_enabled_flag | u(1) |

The semantics of this flag is:

sps_tsrc_disabled_flag equal to 1 specifies that the residual_coding( ) syntax structure is used to parse the residual samples of a transform skip block for pictures referring to the SPS. sps_tsrc_disabled_flag equal to 0 specifies that the residual_ts_coding( ) syntax structure may be used to parse the residual samples of a transform skip block for pictures referring to the SPS. When not present, the value of sps_tsrc_disabled_flag is inferred to be zero.

The semantics of no_tsrc_constraint_flag are modified as f

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag sps_tsrc_disabled_flag shall be equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

The SH signaling is modified as follows:

| slice_header( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_enabled_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_enabled_flag ) |  |
| sh_sign_data_hiding_enabled_flag | u(1) |
| if( sps_transform_skip_enabled_flag!sps_tsrc_disabled_flag && !sh_dep_quant_enabled_flag && !sh_sign_data_hiding_enabled_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| … |  |

# Proposed Method 3: Constraint on “no_tsrc_constraint_flag”

As described earlier, when no_transform_skip_constraint_flag is one, no_tsrc_constraint_flag shall be one as TSRC is not used if transform skip is disabled. The corresponding specification changes are:

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is 1, the value of no_tsrc_constraint_flag shall be one.

Further, disabling dependent quantization and sign data hiding, their constraint flag semantics are modified as follows:

no_dep_quant_constraint_flag equal to 1 specifies that sps_dep_quant_enabled_flag shall be equal to 0. no_dep_quant_constraint_flag equal to 0 does not impose such a constraint. When no_tsrc_constraint_flag is equal to 1, the value of no_dep_quant_constraint_flag shall be 1.

no_sign_data_hiding_constraint_flag equal to 1 specifies that sps_sign_data_hiding_enabled_flag shall be equal to 0. no_sign_data_hiding_constraint_flag equal to 0 does not impose such a constraint. When no_tsrc_constraint_flag is equal to 1, the value of no_sign_data_hiding_constraint_flag shall be 1.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).