| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0066 |
| --- | --- |

| Title: | AHG9: On constraint flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | C. Rosewarne J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 chris.rosewarne@canon.com.au jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

The VVC draft 9 specification provides a set of constraint flags to deactivate certain coding tools, pictures types, image formats, and other categories of constraint flags. It is asserted that adding flags to also constrain certain aspects of block structure would be beneficial and in keeping with the specificity with which other coding tools are constrained. This contribution proposes adding constraint flags for CTU size and 64-pt transform availability. Also proposed are a scaling list constraint flags, and adding constraints on aps_params_type according to the constraint flags.

# Proposal

Additional constraints in general_constraint_info:

- Maximum CTU size is proposed to be constrained by a max_log2_ctu_size_constraint_idc syntax element, coded using u(2).

- Use of the 64-pt transform is proposed to be constrained by no_luma_transform_size_64_constraint_flag.

- Use of scaling lists is proposed to be constrained by no_explicit_scaling_list_constraint_flag.

The corresponding SPS syntax elements (sps_log2_ctu_size_minus5,  sps_max_luma_transform_size_64_flag, and sps_explicit_scaling_list_enabled_flag) are constrained accordingly.

Constraints within the APS to prohibit specific contents according to coding tool constraints:

- aps_params_type equal to 0 (ALF) is prohibited when no_alf_constraint_flag is 1.

- aps_params_type equal to 1 (LMCS) is prohibited when no_lmcs_constraint_flag is 1.

- aps_params_type equal to 2 (scaling lists) is prohibited when no_explicit_scaling_list_constraint_flag is 1.

# References

- B. Bross, J. Chen, S. Liu, “Versatile Video Coding (Draft 8)”, JVET-R2001, Alpbach, AT, April 2020.

# WD changes (on JVET-R2001-vA)

#### 7.3.3.2  General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| general_one_picture_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| single_layer_constraint_flag | u(1) |
| all_layers_independent_constraint_flag | u(1) |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| max_log2_ctu_size_constraint_idc | u(2) |
| no_luma_transform_size_64_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |
| no_explicit_scaling_list_constraint_flag | u(1) |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_chroma_qp_offset_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

#### 7.4.4.2   General constraint information semantics

…

max_log2_ctu_size_constraint_idc specifies that sps_log2_ctu_size_minus5 shall be in the range of 0 to max_log2_ctu_size_constraint_idc, inclusive.

no_luma_transform_size_64_constraint_flag equal to 1 specifies that sps_max_luma_transform_size_64_flag shall be equal to 0. no_luma_transform_size_64_constraint_flag equal to 0 does not impose such a constraint. no_explicit_scaling_list_constraint_flag equal to 1 specifies that sps_explicit_scaling_list_enabled_flag shall be equal to 0 and aps_params_type shall not be equal to 2.  no_explicit_scaling_list_constraint_flag equal to 0 does not impose such a constraint.

…

no_alf_constraint_flag equal to 1 specifies that sps_alf_enabled_flag shall be equal to 0. And aps_params_type shall not be equal to 0. no_alf_constraint_flag equal to 0 does not impose such a constraints.

…

no_lmcs_constraint_flag equal to 1 specifies that sps_lmcs_enabled_flag shall be equal to 0 and aps_params_type shall not be equal to 1. no_lmcs_constraint_flag equal to 0 does not impose such a constraints.

# Patent rights declaration(s)

Canon Inc may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Canon Australia Pty Ltd may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).