| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0074 |
| --- | --- |

| Title: | AHG9: On SPS, PH, SH syntax order |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Mohammed Golam Sarwer Yan Ye 525 Almanor Ave, Sunnyvale, CA | Tel: Email: | +1-408-858-6780 m.sarwer@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

# Abstract

This contribution proposes two aspects.

The first aspect proposes to rearrange the SPS syntax elements so that the syntax elements of similar coding tools are grouped together. In the proposed method, all of the transform related SPS syntax are placed together. Similarly, sps_lmcs_enabled_flag is signaled right after signaling of sps_ccalf_enabled_flag.

In a similar sprit of aspect 1, aspect 2 proposes to rearrange the PH and SH syntax elements so that the syntax elements related to all in-loop filters are placed together.

# Introduction

In VVC draft 9, the SPS syntax elements of the transform related syntax are scattered in the different places of the syntax table. Similarly, in the picture header (PH) and the slice header (SH), some of the syntax elements of in-loop filters are in the top part of the syntax table whereas, while others are on the bottom part of the syntax table.

# Proposed method-aspect 1: SPS syntax rearrange

In the aspect 1 of the proposed method, the syntax elements of the SPS syntax table are rearranged in such a way that all transform related syntax elements are placed together. The syntax elements related to maximum transform size, transform-skip, low frequency non-separable transform (LFNST) and multiple transform sets (MTS) are signaled in the same place of the syntax table.

Similarly, in the proposed method, syntax related to in-loop filters are grouped together. The sps_lmcs_enabled_flag is signaled right after signaling of sps_ccalf_enabled_flag. Table 1 shows the SPS syntax table of the proposed method after rearranging. The yellow highlighted portion shows addition and red stripe part shows the removal as compared to VVC draft 9.

Table 1: SPS syntax table of the aspect 1

| seq_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| sps_seq_parameter_set_id | u(4) |  |
| sps_video_parameter_set_id | u(4) |  |
| sps_max_sublayers_minus1 | u(3) |  |
| sps_reserved_zero_4bits | u(4) |  |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |  |
| sps_gdr_enabled_flag | u(1) |  |
| sps_chroma_format_idc | u(2) |  |
| if( sps_chroma_format_idc = = 3 ) |  |  |
| sps_separate_colour_plane_flag | u(1) |  |
| sps_ref_pic_resampling_enabled_flag | u(1) |  |
| if( sps_ref_pic_resampling_enabled_flag ) |  |  |
| sps_res_change_in_clvs_allowed_flag | u(1) |  |
| sps_pic_width_max_in_luma_samples | ue(v) |  |
| sps_pic_height_max_in_luma_samples | ue(v) |  |
| sps_conformance_window_flag | u(1) |  |
| if( sps_conformance_window_flag ) { |  |  |
| sps_conf_win_left_offset | ue(v) |  |
| sps_conf_win_right_offset | ue(v) |  |
| sps_conf_win_top_offset | ue(v) |  |
| sps_conf_win_bottom_offset | ue(v) |  |
| } |  |  |
| sps_log2_ctu_size_minus5 | u(2) |  |
| sps_subpic_info_present_flag | u(1) |  |
| if( sps_subpic_info_present_flag ) { |  |  |
| sps_num_subpics_minus1 | ue(v) |  |
| if( sps_num_subpics_minus1 > 0 ) |  |  |
| sps_independent_subpics_flag | u(1) |  |
| for( i = 0; sps_num_subpics_minus1 > 0 && i <= sps_num_subpics_minus1; i++ ) { |  |  |
| if( i > 0 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |  |
| sps_subpic_ctu_top_left_x[ i ] | u(v) |  |
| if( i > 0 && sps_pic_height_max_in_luma_samples > CtbSizeY ) { |  |  |
| sps_subpic_ctu_top_left_y[ i ] | u(v) |  |
| if( i < sps_num_subpics_minus1 && sps_pic_width_max_in_luma_samples > CtbSizeY ) |  |  |
| sps_subpic_width_minus1[ i ] | u(v) |  |
| if( i < sps_num_subpics_minus1 && sps_pic_height_max_in_luma_samples > CtbSizeY ) |  |  |
| sps_subpic_height_minus1[ i ] | u(v) |  |
| if( !sps_independent_subpics_flag) { |  |  |
| sps_subpic_treated_as_pic_flag[ i ] | u(1) |  |
| sps_loop_filter_across_subpic_enabled_flag[ i ] | u(1) |  |
| } |  |  |
| } |  |  |
| sps_subpic_id_len_minus1 | ue(v) |  |
| sps_subpic_id_mapping_explicitly_signalled_flag | u(1) |  |
| if( sps_subpic_id_mapping_explicitly_signalled_flag ) { |  |  |
| sps_subpic_id_mapping_present_flag | u(1) |  |
| if( sps_subpic_id_mapping_present_flag ) |  |  |
| for( i = 0; i <= sps_num_subpics_minus1; i++ ) |  |  |
| sps_subpic_id[ i ] | u(v) |  |
| } |  |  |
| } |  |  |
| sps_bit_depth_minus8 | ue(v) |  |
| sps_entropy_coding_sync_enabled_flag | u(1) |  |
| sps_entry_point_offsets_present_flag | u(1) |  |
| sps_log2_max_pic_order_cnt_lsb_minus4 | u(4) |  |
| sps_poc_msb_cycle_flag | u(1) |  |
| if( sps_poc_msb_cycle_flag ) |  |  |
| sps_poc_msb_cycle_len_minus1 | ue(v) |  |
| sps_num_extra_ph_bits_bytes | u(2) |  |
| extra_ph_bits_struct( sps_num_extra_ph_bits_bytes ) |  |  |
| sps_num_extra_sh_bits_bytes | u(2) |  |
| extra_sh_bits_struct( sps_num_extra_sh_bits_bytes ) |  |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |  |
| sps_sublayer_dpb_params_flag | u(1) |  |
| dpb_parameters( sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |  |
| } |  |  |
| if( ChromaArrayType != 0 ) |  |  |
| sps_qtbtt_dual_tree_intra_flag | u(1) |  |
| sps_log2_min_luma_coding_block_size_minus2 | ue(v) |  |
| sps_partition_constraints_override_enabled_flag | u(1) |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |  |
| sps_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |  |
| if( sps_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |  |
| sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |  |
| } |  |  |
| sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |  |
| sps_max_mtt_hierarchy_depth_inter_slice | ue(v) |  |
| if( sps_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |  |
| sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |  |
| sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |  |
| } |  |  |
| if( sps_qtbtt_dual_tree_intra_flag ) { |  |  |
| sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |  |
| sps_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |  |
| if( sps_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |  |
| sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |  |
| sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |  |
| } |  |  |
| } |  |  |
| if( CtbSizeY > 32 ) |  |  |
| sps_max_luma_transform_size_64_flag | u(1) |  |
| sps_transform_skip_enabled_flag | u(1) |  |
| if( sps_transform_skip_enabled_flag ) { |  |  |
| sps_log2_transform_skip_max_size_minus2 | ue(v) |  |
| sps_bdpcm_enabled_flag | u(1) |  |
| } |  |  |
| sps_mts_enabled_flag | u(1) |  |
| if( sps_mts_enabled_flag ) { |  |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |  |
| } |  |  |
| sps_lfnst_enabled_flag | u(1) |  |
| if( ChromaArrayType != 0 ) { |  |  |
| sps_joint_cbcr_enabled_flag | u(1) |  |
| sps_same_qp_table_for_chroma_flag | u(1) |  |
| numQpTables = sps_same_qp_table_for_chroma_flag ? 1 : ( sps_joint_cbcr_enabled_flag ? 3 : 2 ) |  |  |
| for( i = 0; i < numQpTables; i++ ) { |  |  |
| sps_qp_table_start_minus26[ i ] | se(v) |  |
| sps_num_points_in_qp_table_minus1[ i ] | ue(v) |  |
| for( j = 0; j <= sps_num_points_in_qp_table_minus1[ i ]; j++ ) { |  |  |
| sps_delta_qp_in_val_minus1[ i ][ j ] | ue(v) |  |
| sps_delta_qp_diff_val[ i ][ j ] | ue(v) |  |
| } |  |  |
| } |  |  |
| } |  |  |
| sps_sao_enabled_flag | u(1) |  |
| sps_alf_enabled_flag | u(1) |  |
| if( sps_alf_enabled_flag && ChromaArrayType != 0 ) |  |  |
| sps_ccalf_enabled_flag | u(1) |  |
| sps_lmcs_enabled_flag | u(1) |  |
| sps_transform_skip_enabled_flag | u(1) |  |
| if( sps_transform_skip_enabled_flag ) { |  |  |
| sps_log2_transform_skip_max_size_minus2 | ue(v) |  |
| sps_bdpcm_enabled_flag | u(1) |  |
| } |  |  |
| sps_weighted_pred_flag | u(1) |  |
| sps_weighted_bipred_flag | u(1) |  |
| sps_long_term_ref_pics_flag | u(1) |  |
| if( sps_video_parameter_set_id > 0 ) |  |  |
| sps_inter_layer_ref_pics_present_flag | u(1) |  |
| sps_idr_rpl_present_flag | u(1) |  |
| sps_rpl1_same_as_rpl0_flag | u(1) |  |
| for( i = 0; i < sps_rpl1_same_as_rpl0_flag ? 1 : 2; i++ ) { |  |  |
| sps_num_ref_pic_lists[ i ] | ue(v) |  |
| for( j = 0; j < sps_num_ref_pic_lists[ i ]; j++) |  |  |
| ref_pic_list_struct( i, j ) |  |  |
| } |  |  |
| sps_ref_wraparound_enabled_flag | u(1) |  |
| sps_temporal_mvp_enabled_flag | u(1) |  |
| if( sps_temporal_mvp_enabled_flag ) |  |  |
| sps_sbtmvp_enabled_flag | u(1) |  |
| sps_amvr_enabled_flag | u(1) |  |
| sps_bdof_enabled_flag | u(1) |  |
| if( sps_bdof_enabled_flag ) |  |  |
| sps_bdof_control_present_in_ph_flag | u(1) |  |
| sps_smvd_enabled_flag | u(1) |  |
| sps_dmvr_enabled_flag | u(1) |  |
| if( sps_dmvr_enabled_flag) |  |  |
| sps_dmvr_control_present_in_ph_flag | u(1) |  |
| sps_mmvd_enabled_flag | u(1) |  |
| if( sps_mmvd_enabled_flag ) |  |  |
| sps_mmvd_fullpel_only_flag | u(1) |  |
| sps_six_minus_max_num_merge_cand | ue(v) |  |
| sps_sbt_enabled_flag | u(1) |  |
| sps_affine_enabled_flag | u(1) |  |
| if( sps_affine_enabled_flag ) { |  |  |
| sps_five_minus_max_num_subblock_merge_cand | ue(v) |  |
| sps_affine_type_flag | u(1) |  |
| if( sps_amvr_enabled_flag ) |  |  |
| sps_affine_amvr_enabled_flag | u(1) |  |
| sps_affine_prof_enabled_flag | u(1) |  |
| if( sps_affine_prof_enabled_flag ) |  |  |
| sps_prof_control_present_in_ph_flag | u(1) |  |
| } |  |  |
| sps_bcw_enabled_flag | u(1) |  |
| sps_ciip_enabled_flag | u(1) |  |
| if( MaxNumMergeCand >= 2 ) { |  |  |
| sps_gpm_enabled_flag | u(1) |  |
| if( sps_gpm_enabled_flag && MaxNumMergeCand >= 3 ) |  |  |
| sps_max_num_merge_cand_minus_max_num_gpm_cand | ue(v) |  |
| } |  |  |
| sps_log2_parallel_merge_level_minus2 | ue(v) |  |
| sps_isp_enabled_flag | u(1) |  |
| sps_mrl_enabled_flag | u(1) |  |
| sps_mip_enabled_flag | u(1) |  |
| if( ChromaArrayType != 0 ) |  |  |
| sps_cclm_enabled_flag | u(1) |  |
| if( sps_chroma_format_idc = = 1 ) { |  |  |
| sps_chroma_horizontal_collocated_flag | u(1) |  |
| sps_chroma_vertical_collocated_flag | u(1) |  |
| } |  |  |
| sps_mts_enabled_flag | u(1) |  |
| if( sps_mts_enabled_flag ) { |  |  |
| sps_explicit_mts_intra_enabled_flag | u(1) |  |
| sps_explicit_mts_inter_enabled_flag | u(1) |  |
| } |  |  |
| sps_palette_enabled_flag | u(1) |  |
| if( ChromaArrayType = = 3 && !sps_max_luma_transform_size_64_flag ) |  |  |
| sps_act_enabled_flag | u(1) |  |
| if( sps_transform_skip_enabled_flag ¦ ¦ sps_palette_enabled_flag ) |  |  |
| sps_internal_bit_depth_minus_input_bit_depth | ue(v) |  |
| sps_ibc_enabled_flag | u(1) |  |
| if( sps_ibc_enabled_flag ) |  |  |
| sps_six_minus_max_num_ibc_merge_cand | ue(v) |  |
| sps_lmcs_enabled_flag | u(1) |  |
| sps_lfnst_enabled_flag | u(1) |  |
| sps_ladf_enabled_flag | u(1) |  |
| if( sps_ladf_enabled_flag ) { |  |  |
| sps_num_ladf_intervals_minus2 | u(2) |  |
| sps_ladf_lowest_interval_qp_offset | se(v) |  |
| for( i = 0; i < sps_num_ladf_intervals_minus2 + 1; i++ ) { |  |  |
| sps_ladf_qp_offset[ i ] | se(v) |  |
| sps_ladf_delta_threshold_minus1[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| sps_explicit_scaling_list_enabled_flag | u(1) |  |
| if( sps_lfnst_enabled_flag && sps_explicit_scaling_list_enabled_flag ) |  |  |
| sps_scaling_matrix_for_lfnst_disabled_flag | u(1) |
| if( sps_act_enabled_flag && sps_explicit_scaling_list_enabled_flag ) |  |
| sps_scaling_matrix_for_alternative_colour_space_disabled_flag | u(1) |
| if( sps_scaling_matrix_for_alternative_colour_space_disabled_flag ) |  |
| sps_scaling_matrix_designated_colour_space_flag | u(1) |
| sps_dep_quant_enabled_flag | u(1) |  |
| if( !sps_dep_quant_enabled_flag ) |  |  |
| sps_sign_data_hiding_enabled_flag | u(1) |  |
| sps_virtual_boundaries_enabled_flag | u(1) |  |
| if( sps_virtual_boundaries_enabled_flag ) { |  |  |
| sps_virtual_boundaries_present_flag | u(1) |  |
| if( sps_virtual_boundaries_present_flag ) { |  |  |
| sps_num_ver_virtual_boundaries | u(2) |  |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |  |
| sps_virtual_boundary_pos_x[ i ] | ue(v) |  |
| sps_num_hor_virtual_boundaries | u(2) |  |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |  |
| sps_virtual_boundary_pos_y[ i ] | ue(v) |  |
| } |  |  |
| } |  |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |  |
| sps_general_hrd_params_present_flag | u(1) |  |
| if( sps_general_hrd_params_present_flag ) { |  |  |
| general_hrd_parameters( ) |  |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |  |
| sps_sublayer_cpb_params_present_flag | u(1) |  |
| firstSubLayer = sps_sublayer_cpb_params_present_flag ? 0 : sps_max_sublayers_minus1 |  |  |
| ols_hrd_parameters( firstSubLayer, sps_max_sublayers_minus1 ) |  |  |
| } |  |  |
| } |  |  |
| sps_field_seq_flag | u(1) |  |
| sps_vui_parameters_present_flag | u(1) |  |
| if( sps_vui_parameters_present_flag ) |  |  |
| vui_parameters( ) /* Specified in ITU-T H.SEI ¦ ISO/IEC 23002-7 */ |  |  |
| sps_extension_flag | u(1) |  |
| if( sps_extension_flag ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| sps_extension_data_flag | u(1) |  |
| rbsp_trailing_bits( ) |  |  |
| } |  |  |

# Proposed method-aspect 2: PH and SH syntax order

In the aspect 2, picture header (PH) and slice header (SH) syntax elements related to SAO and de-blocking process are signaled right after ALF and LMCS related syntax. Table 2 and Table 3 show the proposed PH and SH syntax table. It is shown that the syntax elements related to de-blocking and SAO are moved from bottom part to the top part of the syntax table so that all in-loop filter related syntax are placed together.

Table 2: PH syntax table of the aspect 2

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag | u(1) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ph_pic_order_cnt_lsb | u(v) |
| if( ph_gdr_or_irap_pic_flag ) |  |
| ph_no_output_of_prior_pics_flag | u(1) |
| if( ph_gdr_pic_flag ) |  |
| ph_recovery_poc_cnt | ue(v) |
| for( i = 0; i < NumExtraPhBits; i++ ) |  |
| ph_extra_bit[ i ] | u(1) |
| if( sps_poc_msb_cycle_flag ) { |  |
| ph_poc_msb_cycle_present_flag | u(1) |
| if( ph_poc_msb_cycle_present_flag ) |  |
| ph_poc_msb_cycle_val | u(v) |
| } |  |
| if( sps_alf_enabled_flag && pps_alf_info_in_ph_flag ) { |  |
| ph_alf_enabled_flag | u(1) |
| if( ph_alf_enabled_flag ) { |  |
| ph_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < ph_num_alf_aps_ids_luma; i++ ) |  |
| ph_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) { |  |
| ph_alf_cb_flag | u(1) |
| ph_alf_cr_flag | u(1) |
| } |  |
| if( ph_alf_cb_flag ¦ ¦ ph_alf_cr_flag ) |  |
| ph_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| ph_cc_alf_cb_enabled_flag | u(1) |
| if( ph_cc_alf_cb_enabled_flag ) |  |
| ph_cc_alf_cb_aps_id | u(3) |
| ph_cc_alf_cr_enabled_flag | u(1) |
| if( ph_cc_alf_cr_enabled_flag ) |  |
| ph_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |
| if( sps_lmcs_enabled_flag ) { |  |
| ph_lmcs_enabled_flag | u(1) |
| if( ph_lmcs_enabled_flag ) { |  |
| ph_lmcs_aps_id | u(2) |
| if( ChromaArrayType != 0 ) |  |
| ph_chroma_residual_scale_flag | u(1) |
| } |  |
| } |  |
| if( sps_sao_enabled_flag && pps_sao_info_in_ph_flag ) { |  |
| ph_sao_luma_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| ph_sao_chroma_enabled_flag | u(1) |
| } |  |
| if( pps_deblocking_filter_override_enabled_flag && pps_dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_luma_beta_offset_div2 | se(v) |
| ph_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( sps_explicit_scaling_list_enabled_flag ) { |  |
| ph_explicit_scaling_list_enabled_flag | u(1) |
| if( ph_explicit_scaling_list_enabled_flag ) |  |
| ph_scaling_list_aps_id | u(3) |
| } |  |
| if( sps_virtual_boundaries_enabled_flag && !sps_virtual_boundaries_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_x[ i ] | ue(v) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_y[ i ] | ue(v) |
| } |  |
| } |  |
| if( pps_output_flag_present_flag && !ph_non_ref_pic_flag ) |  |
| ph_pic_output_flag | u(1) |
| if( pps_rpl_info_in_ph_flag ) |  |
| ref_pic_lists( ) |  |
| if( sps_partition_constraints_override_enabled_flag ) |  |
| ph_partition_constraints_override_flag | u(1) |
| if( ph_intra_slice_allowed_flag ) { |  |
| if( ph_partition_constraints_override_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_qtbtt_dual_tree_intra_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| ph_max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| ph_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| if( pps_cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_intra_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_intra_slice | ue(v) |
| } |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| if( ph_partition_constraints_override_flag ) { |  |
| ph_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| ph_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( ph_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| ph_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| ph_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| if( pps_cu_qp_delta_enabled_flag ) |  |
| ph_cu_qp_delta_subdiv_inter_slice | ue(v) |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| ph_cu_chroma_qp_offset_subdiv_inter_slice | ue(v) |
| if( sps_temporal_mvp_enabled_flag ) { |  |
| ph_temporal_mvp_enabled_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && pps_rpl_info_in_ph_flag ) { |  |
| if( num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) |  |
| ph_collocated_from_l0_flag | u(1) |
| if( ( ph_collocated_from_l0_flag && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( !ph_collocated_from_l0_flag && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) |  |
| ph_collocated_ref_idx | ue(v) |
| } |  |
| } |  |
| if( sps_mmvd_fullpel_only_flag ) |  |
| ph_mmvd_fullpel_only_flag | u(1) |
| if( !pps_rpl_info_in_ph_flag ¦ ¦ num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 ) { |  |
| ph_mvd_l1_zero_flag | u(1) |
| if( sps_bdof_control_present_in_ph_flag ) |  |
| ph_bdof_disabled_flag | u(1) |
| if( sps_dmvr_control_present_in_ph_flag ) |  |
| ph_dmvr_disabled_flag | u(1) |
| } |  |
| if( sps_prof_control_present_in_ph_flag ) |  |
| ph_prof_disabled_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_wp_info_in_ph_flag ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( pps_qp_delta_info_in_ph_flag ) |  |
| ph_qp_delta | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| ph_joint_cbcr_sign_flag | u(1) |
| if( sps_sao_enabled_flag && pps_sao_info_in_ph_flag ) { |  |
| ph_sao_luma_enabled_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| ph_sao_chroma_enabled_flag | u(1) |
| } |  |
| if( pps_deblocking_filter_override_enabled_flag && pps_dbf_info_in_ph_flag ) { |  |
| ph_deblocking_filter_override_flag | u(1) |
| if( ph_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| ph_deblocking_filter_disabled_flag | u(1) |
| if( !ph_deblocking_filter_disabled_flag ) { |  |
| ph_luma_beta_offset_div2 | se(v) |
| ph_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| ph_cb_beta_offset_div2 | se(v) |
| ph_cb_tc_offset_div2 | se(v) |
| ph_cr_beta_offset_div2 | se(v) |
| ph_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| if( pps_picture_header_extension_present_flag ) { |  |
| ph_extension_length | ue(v) |
| for( i = 0; i < ph_extension_length; i++) |  |
| ph_extension_data_byte[ i ] | u(8) |
| } |  |
| } |  |

Table 3: SH syntax table of aspect 2

| slice_header( ) { | Descriptor |
| --- | --- |
| sh_picture_header_in_slice_header_flag | u(1) |
| if( sh_picture_header_in_slice_header_flag ) |  |
| picture_header_structure( ) |  |
| if( sps_subpic_info_present_flag ) |  |
| sh_subpic_id | u(v) |
| if( ( pps_rect_slice_flag && NumSlicesInSubpic[ CurrSubpicIdx ] > 1 ) ¦ ¦ ( !pps_rect_slice_flag && NumTilesInPic > 1 ) ) |  |
| sh_slice_address | u(v) |
| for( i = 0; i < NumExtraShBits; i++ ) |  |
| sh_extra_bit[ i ] | u(1) |
| if( !pps_rect_slice_flag && NumTilesInPic − sh_slice_address > 1 ) |  |
| sh_num_tiles_in_slice_minus1 | ue(v) |
| if( ph_inter_slice_allowed_flag ) |  |
| sh_slice_type | ue(v) |
| if( sps_alf_enabled_flag && !pps_alf_info_in_ph_flag ) { |  |
| sh_alf_enabled_flag | u(1) |
| if( sh_alf_enabled_flag ) { |  |
| sh_num_alf_aps_ids_luma | u(3) |
| for( i = 0; i < sh_num_alf_aps_ids_luma; i++ ) |  |
| sh_alf_aps_id_luma[ i ] | u(3) |
| if( ChromaArrayType != 0 ) { |  |
| sh_alf_cb_flag | u(1) |
| sh_alf_cr_flag | u(1) |
| } |  |
| if( sh_alf_cb_flag ¦ ¦ sh_alf_cr_flag ) |  |
| sh_alf_aps_id_chroma | u(3) |
| if( sps_ccalf_enabled_flag ) { |  |
| sh_cc_alf_cb_enabled_flag | u(1) |
| if( sh_cc_alf_cb_enabled_flag ) |  |
| sh_cc_alf_cb_aps_id | u(3) |
| sh_cc_alf_cr_enabled_flag | u(1) |
| if( sh_cc_alf_cr_enabled_flag ) |  |
| sh_cc_alf_cr_aps_id | u(3) |
| } |  |
| } |  |
| } |  |
| if( ph_lmcs_enabled_flag && !sh_picture_header_in_slice_header_flag ) |  |
| sh_lmcs_used_flag | u(1) |
| if( sps_sao_enabled_flag && !pps_sao_info_in_ph_flag ) { |  |
| sh_sao_luma_used_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sh_sao_chroma_used_flag | u(1) |
| } |  |
| if( pps_deblocking_filter_override_enabled_flag && !pps_dbf_info_in_ph_flag ) |  |
| sh_deblocking_filter_override_flag | u(1) |
| if( sh_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| sh_deblocking_filter_disabled_flag | u(1) |
| if( !sh_deblocking_filter_disabled_flag ) { |  |
| sh_luma_beta_offset_div2 | se(v) |
| sh_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| sh_cb_beta_offset_div2 | se(v) |
| sh_cb_tc_offset_div2 | se(v) |
| sh_cr_beta_offset_div2 | se(v) |
| sh_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( ph_explicit_scaling_list_enabled_flag && !sh_picture_header_in_slice_header_flag ) |  |
| sh_explicit_scaling_list_used_flag | u(1) |
| if( sps_separate_colour_plane_flag = = 1 ) |  |
| sh_colour_plane_id | u(2) |
| if( !pps_rpl_info_in_ph_flag && ( ( nal_unit_type != IDR_W_RADL && nal_unit_type != IDR_N_LP ) ¦ ¦ sps_idr_rpl_present_flag ) ) |  |
| ref_pic_lists( ) |  |
| if( ( ( ( sh_slice_type != I && num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 1 ) ¦ ¦ ( sh_slice_type = = B && num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 1 ) ) ) { |  |
| sh_num_ref_idx_active_override_flag | u(1) |
| if( sh_num_ref_idx_active_override_flag ) |  |
| for( i = 0; i < ( sh_slice_type = = B ? 2: 1 ); i++ ) |  |
| if( num_ref_entries[ i ][ RplsIdx[ i ] ] > 1 ) |  |
| sh_num_ref_idx_active_minus1[ i ] | ue(v) |
| } |  |
| if( sh_slice_type != I ) { |  |
| if( pps_cabac_init_present_flag ) |  |
| sh_cabac_init_flag | u(1) |
| if( ph_temporal_mvp_enabled_flag && !pps_rpl_info_in_ph_flag ) { |  |
| if( sh_slice_type = = B ) |  |
| sh_collocated_from_l0_flag | u(1) |
| if( ( sh_collocated_from_l0_flag && NumRefIdxActive[ 0 ] > 1 ) ¦ ¦ ( ! sh_collocated_from_l0_flag && NumRefIdxActive[ 1 ] > 1 ) ) |  |
| sh_collocated_ref_idx | ue(v) |
| } |  |
| if( !pps_wp_info_in_ph_flag && ( ( pps_weighted_pred_flag && sh_slice_type = = P ) ¦ ¦ ( pps_weighted_bipred_flag && sh_slice_type = = B ) ) ) |  |
| pred_weight_table( ) |  |
| } |  |
| if( !pps_qp_delta_info_in_ph_flag ) |  |
| sh_qp_delta | se(v) |
| if( pps_slice_chroma_qp_offsets_present_flag ) { |  |
| sh_cb_qp_offset | se(v) |
| sh_cr_qp_offset | se(v) |
| if( sps_joint_cbcr_enabled_flag ) |  |
| sh_joint_cbcr_qp_offset | se(v) |
| } |  |
| if( pps_cu_chroma_qp_offset_list_enabled_flag ) |  |
| sh_cu_chroma_qp_offset_enabled_flag | u(1) |
| if( sps_sao_enabled_flag && !pps_sao_info_in_ph_flag ) { |  |
| sh_sao_luma_used_flag | u(1) |
| if( ChromaArrayType != 0 ) |  |
| sh_sao_chroma_used_flag | u(1) |
| } |  |
| if( pps_deblocking_filter_override_enabled_flag && !pps_dbf_info_in_ph_flag ) |  |
| sh_deblocking_filter_override_flag | u(1) |
| if( sh_deblocking_filter_override_flag ) { |  |
| if( !pps_deblocking_filter_disabled_flag ) |  |
| sh_deblocking_filter_disabled_flag | u(1) |
| if( !sh_deblocking_filter_disabled_flag ) { |  |
| sh_luma_beta_offset_div2 | se(v) |
| sh_luma_tc_offset_div2 | se(v) |
| if( pps_chroma_tool_offsets_present_flag ) { |  |
| sh_cb_beta_offset_div2 | se(v) |
| sh_cb_tc_offset_div2 | se(v) |
| sh_cr_beta_offset_div2 | se(v) |
| sh_cr_tc_offset_div2 | se(v) |
| } |  |
| } |  |
| } |  |
| if( sps_dep_quant_enabled_flag ) |  |
| sh_dep_quant_used_flag | u(1) |
| if( sps_sign_data_hiding_enabled_flag && !sh_dep_quant_used_flag ) |  |
| sh_sign_data_hiding_used_flag | u(1) |
| if( sps_transform_skip_enabled_flag && !sh_dep_quant_used_flag && !sh_sign_data_hiding_used_flag ) |  |
| sh_ts_residual_coding_disabled_flag | u(1) |
| if( pps_slice_header_extension_present_flag ) { |  |
| sh_slice_header_extension_length | ue(v) |
| for( i = 0; i < sh_slice_header_extension_length; i++) |  |
| sh_slice_header_extension_data_byte[ i ] | u(8) |
| } |  |
| if( NumEntryPoints > 0 ) { |  |
| sh_entry_offset_len_minus1 | ue(v) |
| for( i = 0; i < NumEntryPoints; i++ ) |  |
| sh_entry_point_offset_minus1[ i ] | u(v) |
| } |  |
| byte_alignment( ) |  |
| } |  |

# Conclusion

This contribution proposes to rearrange the SPS, PH, and SH syntax elements so that the syntax elements related to similar coding tools are grouped together. In SPS, all transform related syntax are grouped together. In addition, the sps_lmcs_enabled_flag is signaled right after signaling of sps_ccalf_enabled_flag. Aspect 2 proposes to re-order the PH and SH syntax elements so that the syntax elements related to all in-loop filters are placed together.

# Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).