| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0076-r1 |
| --- | --- |

| Title: | AHG9: On syntax elements in the beginning of picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution asserted that the signalling of syntax elements in the beginning of picture header may need clean-ups. The proposed clean-ups is asserted help system to access syntax elements in the beginning of picture header.

The following changes are proposed:

- Move ph_non_ref_pic_flag to earlier location in picture header before any other syntax elements whose presence is conditioned (i.e., earlier than ph_gdr_pic_flag and ph_intra_slice_allowed_flag). It is asserted that ph_non_ref_pic_flag is needed for system use so it should be located in earlier location that is fixed.

- Change the coding of ph_pic_parameter_set_id from ue(v) to u(6).

- Move ph_pic_parameter to earlier location in picture header before any other syntax elements whose presence is conditioned.

- Move ph_gdr_pic_flag to be after ph_pic_parameter_set_id and condition the presence of ph_gdr_pic_flag such that it may be present only when sps_gdr_enabled_flag is equal to 1.

In the first revision, proposal item d) was updated.

# Introduction

The signalling of syntax elements in picture header may need some clean-ups. The clean-ups are in the following aspects:

- The signalling of ph_non_ref_pic_flag. This flag is designed to be used by the system; consequently, it should be signalled in the earlier position in the picture header that is fixed. Currently, after adoptions of several other syntax elements, this flag is now signalled in lower position and after some syntax elements whose presence are conditioned, i.e., ph_gdr_pic_flag and ph_intra_slice_allowed_flag. It would be better to move the signalling of the flag to earlier location and before any syntax elements whose presence is conditioned.

- In the 18th JVET meeting, it was agreed to change the coding of pps_pic_parameter_set_id from ue(v) to u(6). However, the syntax element ph_pic_parameter_set_id is still coded as ue(v). It may be a good and consistent design to also change the coding of ph_pic_parameter_set_id to u(6) as well.

- If the above point is agreed, since the coding of ph_pic_parameter_set_id has a fixed number of bits, it may also be desirable to move the signalling of this syntax element to earlier position and before any syntax elements whose presence is conditioned.

- The syntax element ph_gdr_pic_flag is asserted not for system use but for conditioning the presence of ph_recovery_poc_cnt. For system use, it is ph_gdr_or_irap_pic_flag, not ph_gdr_pic_flag. Furthermore, if ph_gdr_pic_flag is moved to be present after the syntax element pps_pic_parameter_set_id then it is possible to not signal ph_gdr_pic_flag when GDR is not enabled as it is not needed.

# Proposal changes and spec text

The proposed clean-ups to address the aspects described above are as follows:

- Move ph_non_ref_pic_flag to earlier location in picture header before any other syntax elements whose presence is conditioned (i.e., earlier than ph_gdr_pic_flag and ph_intra_slice_allowed_flag).

- Change the coding of ph_pic_parameter_set_id from ue(v) to u(6)

- Move ph_pic_parameter to earlier location in picture header before any other syntax elements whose presence is conditioned.

- Move ph_gdr_pic_flag to be after ph_pic_parameter_set_id and condition the presence of ph_gdr_pic_flag such that it may be present only when sps_gdr_enabled_flag is equal to 1.

The spec text changes for implementing the above proposed changes is as follows (yellow-highlighted text are added text and italic font text are removed text):

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| ph_gdr_or_irap_pic_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| ph_pic_parameter_set_id | u(6) |
| if( ph_gdr_or_irap_pic_flag && sps_gdr_enabled_flag ) |  |
| ph_gdr_pic_flag | u(1) |
| ph_inter_slice_allowed_flag | u(1) |
| if( ph_inter_slice_allowed_flag ) |  |
| ph_intra_slice_allowed_flag | u(1) |
| ph_non_ref_pic_flag | u(1) |
| ph_pic_parameter_set_id | ue(v) |
| ... |  |
| } |  |

...

ph_gdr_or_irap_pic_flag equal to 1 specifies that the current picture is a GDR or IRAP picture. ph_gdr_or_irap_pic_flag equal to 0 specifies that the current picture is not a GDR picture and may or may not be an IRAP picture.

ph_non_ref_pic_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. ph_non_ref_pic_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture.

ph_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of ph_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

It is a requirement of bitstream conformance that the value of TemporalId of the PH shall be greater than or equal to the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to ph_pic_parameter_set_id.

ph_gdr_pic_flag equal to 1 specifies the picture associated with the PH is a GDR picture. ph_gdr_pic_flag equal to 0 specifies that the picture associated with the PH is not a GDR picture. When not present, the value of ph_gdr_pic_flag is inferred to be equal to 0. When sps_gdr_enabled_flag is equal to 0, the value of ph_gdr_pic_flag shall be equal to 0. [Ed. (YK): Should the semantics of ph_gdr_pic_flag also be specified "one way"? That'd be consistent with the other flags herein, but then there is some issue for using it to condition the presence of ph_recovery_poc_cnt.]

NOTE 1 – When ph_gdr_or_irap_pic_flag is equal to 1 and ph_gdr_pic_flag is equal to 0, the picture associated with the PH is an IRAP picture.

...

ph_non_ref_pic_flag equal to 1 specifies the picture associated with the PH is never used as a reference picture. ph_non_ref_pic_flag equal to 0 specifies the picture associated with the PH may or may not be used as a reference picture.

ph_pic_parameter_set_id specifies the value of pps_pic_parameter_set_id for the PPS in use. The value of ph_pic_parameter_set_id shall be in the range of 0 to 63, inclusive.

It is a requirement of bitstream conformance that the value of TemporalId of the PH shall be greater than or equal to the value of TemporalId of the PPS that has pps_pic_parameter_set_id equal to ph_pic_parameter_set_id.

...

# Patent rights declaration(s)

LG Electronics may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).