| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0083 |
| --- | --- |

| Title: | AHG8/AHG9: On signaling of ILRP layer index using delta value |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Hyeongmun Jang Junghak Nam Seung Hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com hm.jang@lge.com junghak.nam@lge.com seunghwan3.kim@lge.com jaehyun.lim@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to change the signalling of ILRP layer index in RPL structure from signalling the layer index to signalling the delta of the layer index between the layer where the RPL structure belongs to and the reference layer.

The proposed change is asserted to give two benefits as follows:

- The number of bits required for signalling may be less for common use cases such as spatial scalability and / or quality scalability. In those such cases, inter-layer prediction is done by referring to one layer below the current layer.

- Signalling ILRP using delta value is friendlier for SPS sharing. It is asserted that current SPS cannot be shared across dependent layers since the layer index of ILRPs signalled in the SPS would mostly different. By signalling the layer index of ILRPs using delta value, it improves the chance for SPS to be shared across dependent layers.

# Introduction

In the current VVC specification text, the signalling method for identifying the layer index of inter-layer reference picture (ILRP) directly signals the value of the layer index. This is different from the method for signalling short-term reference picture (STRP) in which the delta value is used, instead of the POC value. It may be beneficial to align the way to signal layer index for ILRP similar to the way STRP is signalled.

There are at least two benefits to align the way to signal layer index for ILRP similar to the way STRP is signalled, they are:

- The number of bits required for signalling may be less for common use cases such as spatial scalability and / or quality scalability. In those such cases, inter-layer prediction is done by referring to one layer below the current layer.

- Signalling ILRP using delta value is friendlier for SPS sharing. In current signalling, it is difficult to share SPS for two or more different dependent layers since each layer would have different reference layer. For example, in the scalable bitstream with 3 layers (layer 0 is independent, layer 1 depends on layer 0, and layer 2 depends on layer 1). In the current signalling, the SPS for layer 1 cannot be shared with layer 2 since the SPS in layer 1 would contains ILRP with layer index 0, whereas for layer 2, the SPS needs to contain ILRP with layer index 1. If we signal the ILPR with delta layer index, instead, the SPS for layer 1 can be shared with layer 2.

In this contribution, it is proposed to change the signalling of ilrp_idx[ listIdx ][ rplsIdx ][ i ]) to use delta value.

# Proposal and spec text changes

The following changes are proposed:

- Change the current syntax element from ilrp_idx[ listIdx ][ rplsIdx ][ i ] to delta_ilrp_idx_minus1[ listIdx ][ rplsIdx ][ i ].

- The delta value is the difference between the layer index of slice or picture that contains the RPL structure.

- The layer index of the ILRP for the current slice is derived by subtracting the layer index of the current slice by the delta value.

The spec text changes for implementing the above proposed changes is as follows:

| ref_pic_list_struct( listIdx, rplsIdx ) { | Descriptor |
| --- | --- |
| num_ref_entries[ listIdx ][ rplsIdx ] | ue(v) |
| if( sps_long_term_ref_pics_flag && rplsIdx < sps_num_ref_pic_lists[ listIdx ] ) |  |
| ltrp_in_header_flag[ listIdx ][ rplsIdx ] | u(1) |
| for( i = 0, j = 0; i < num_ref_entries[ listIdx ][ rplsIdx ]; i++) { |  |
| if( sps_inter_layer_ref_pics_present_flag ) |  |
| inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( !inter_layer_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| if( sps_long_term_ref_pics_flag ) |  |
| st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| if( st_ref_pic_flag[ listIdx ][ rplsIdx ][ i ] ) { |  |
| abs_delta_poc_st[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| if( AbsDeltaPocSt[ listIdx ][ rplsIdx ][ i ] > 0 ) |  |
| strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] | u(1) |
| } else if( !ltrp_in_header_flag[ listIdx ][ rplsIdx ] ) |  |
| rpls_poc_lsb_lt[ listIdx ][ rplsIdx ][ j++ ] | u(v) |
| } else |  |
| delta_ilrp_idx_minus1[ listIdx ][ rplsIdx ][ i ] | ue(v) |
| } |  |
| } |  |

delta_ilrp_idx_minus1[ listIdx ][ rplsIdx ][ i ] plus 1 specifies the delta of index that is used to derive the layer index of the ILRP of the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of delta_ilrp_idx_minus1[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to vps_max_layers_minus1, inclusive.

ilrp_idx[ listIdx ][ rplsIdx ][ i ] specifies the index, to the list of the direct reference layers, of the ILRP of the i-th entry in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure. The value of ilrp_idx[ listIdx ][ rplsIdx ][ i ] shall be in the range of 0 to NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] − 1, inclusive.

The RPL construction process is updated as follows:

The reference picture lists RefPicList[ 0 ] and RefPicList[ 1 ], the reference picture scaling ratios RefPicScale[ i ][ j ][ 0 ] and RefPicScale[ i ][ j ][ 1 ], and the reference picture scaled flags RprConstraintsActive[ 0 ][ j ] and RprConstraintsActive[ 1 ][ j ] are derived as follows:

for( i = 0; i < 2; i++ ) {
	for( j = 0, k = 0, pocBase = PicOrderCntVal; j < num_ref_entries[ i ][ RplsIdx[ i ] ]; j++) {
		if( !inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] ) {
			...
		} else {
			ilrpIdx = GeneralLayerIdx[ nuh_layer_id ] – ( delta_ilrp_idx_minus1[ i ][ RplsIdx ][ j ] + 1 )
			layerIdx = 				DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrpIdxilrp_idx[ i ][ RplsIdx ][ j ] ]
			refPicLayerId = vps_layer_id[ layerIdx ]
			if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
						the same PicOrderCntVal as the current picture )
				RefPicList[ i ][ j ] = picA
			else
				RefPicList[ i ][ j ] = "no reference picture"
		} 
		...
	}
}

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).