| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0088 |
| --- | --- |

| Title: | AHG9: Clean-up on derivation of the MaxNumSubblockCand |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Naeri Park Junghak Nam Hyeongmun Jang Jaehyun Lim 19 Yangjaedaero-11gil Seocho-gu, Seoul 06772 South Korea Seung Hwan Kim 10225 Willow Creek Rd, San Diego CA 92131, USA | Tel: Email: | +82-2-6912-6561 {naeri.park, junghak.nam, hm.jang, jaehyun.lim, seunghwan3.kim}@lge.com |
| Source: | LG Electronics Inc. |  |  |

_____________________________

- Abstract

In the previous meeting, JVET-R0371 was adopted to avoid the disallowing the SbTMVP when both Affine and SbTMVP are enabled. It was suggested to set the sps_five_minus_max_num_subblock_merge_cand in range of 0 to 5 – sps_sbtmvp_enabled_flag so that the number of the subblock merge candidate is set to be 1 at least when sps_sbtmvp_enabled_flag is equal to 1. However, SbTMVP is controlled by not only sps_sbtmvp_enabled_flag but ph_temporal_mvp_enabled_flag in PH. Hence, the intention of the JVET-R0371 for ensuring the minimum one candidate for SbTMVP was not precisely reflected. Therefore, in this proposal, the derivation method of the number of the subblock merge candidates is further cleaned-up by reflecting sps_sbtmvp_enabled_flag and ph_temporal_mvp_enabled_flag simultaneously.

Problem Statement

In VVC draft 9, sps_five_minus_max_num_subblock_merge_cand is signalled in SPS when sps_affine_enabled_flag is equal to 1 as follows.

| if( sps_affine_enabled_flag ) |  |
| --- | --- |
| sps_five_minus_max_num_subblock_merge_cand | ue(v) |

sps_five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction candidates supported in the SPS subtracted from 5. The value of sps_five_minus_max_num_subblock_merge_cand shall be in the range of 0 to 5 − sps_sbtmvp_enabled_flag, inclusive.

Also, final maximum number of the subblock candidates, MaxNumSubblockCand, which is derived in PH are shown as follows.

The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand, is derived as follows: if( sps_affine_enabled_flag ) MaxNumSubblockMergeCand = 5 − sps_five_minus_max_num_subblock_merge_cand (88) else MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag && ph_temporal_mvp_enabled_flag The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

According to the derivation process of the MaxNumSubblockMergeCand, the value of the MaxNumSubblockMergeCand is in range of 1 to 5 when sps_affine_enabled_flag is equal to 1 and sps_sbtmvp_enabled_flag is equal to 1 even though the SbTMVP can be disallowed by the ph_tempora_mvp_enabled_flag

Proposed Method

In this proposal, the range of MaxNumSubblockMergeCand is set by considering sps_sbtmvp_enabled_flag and ph_temporal_mvp_enabled_flag together as follow.

The proposed modified texts are marked as highlighted yellow and stricken-red-font text.

<In SPS > sps_five_minus_max_num_subblock_merge_cand specifies the maximum number of subblock-based merging motion vector prediction candidates supported in the SPS subtracted from 5. The value of sps_five_minus_max_num_subblock_merge_cand shall be in the range of 0 to 5 − sps_sbtmvp_enabled_flag, inclusive. <In PH> The maximum number of subblock-based merging MVP candidates, MaxNumSubblockMergeCand, is derived as follows: if( sps_affine_enabled_flag && sps_five_minus_max_num_subblock_merge_cand < 5 ) MaxNumSubblockMergeCand = 5 − sps_five_minus_max_num_subblock_merge_cand (88) else MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag && ph_temporal_mvp_enabled_flag The value of MaxNumSubblockMergeCand shall be in the range of 0 to 5, inclusive.

Following table shows the behavior of the current VVC draft 9 and proposed method.

| sps_affine_enabled_flag | sps_sbtmvp_enabled_flag | ph_temporal_mvp_enabled_flag | MaxNumSubblockMergeCand |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  |  | VVC Draft 8 | VVC Draft 9 | Proposed |
| 1 | 1 | 1 | 0~5 | 1~5 | 1~5 |
|  | 1 | 0 | 0~5 | 1~5 | 0~5 |
|  | 0 | 1 | 0~5 | 0~5 | 0~5 |
|  | 0 | 0 | 0~5 | 0~5 | 0~5 |
| 0 | 1 | 1 | 1 | 1 | 1 |
|  | 1 | 0 | 0 | 0 | 0 |
|  | 0 | 1 | 0 | 0 | 0 |
|  | 0 | 0 | 0 | 0 | 0 |

Conclusion

Since the intention of the JVET-R0371 for ensuring the minimum one candidate for SbTMVP was not precisely reflected, the clean-up on derivation process for the number of the subblock merge candidate are proposed.

Reference

- B. Bross, et al., “Versatile Video Coding (Draft 9)”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-R2001(JVET-Q2001-vA), 18th Meeting: teleconference, 15–24 April 2020.

- H. Han, et al., “AHG2/9: On max num of subblock merge candidates”, Joint Video Exploration Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC1/SC 29/WG 11 JVET-R0371, 18th Meeting: teleconference, 15–24 April 2020.

Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).