| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0091 |
| --- | --- |

| Title: | AHG8/AHG9: On constraints of still picture |  |  |
| --- | --- | --- | --- |
| Status: | Input Document |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Kiyofumi Abe Takahiro Nishi Tadamasa Toma Virginie Drugeon 3-1-1 Yagumo-naka-machi, Moriguchi, Osaka, Japan | Tel: Email: | +81-80-3083-1171 abe.kiyo@jp.panasonic.com nishi.takahiro@jp.panasonic.com toma.tadamasa@jp.panasonic.com virginie.drugeon@eu.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

- Abstract

According to the specification of the still picture profile in Annex A, general_one_picture_only_constraint_flag is constrained to be set equal to 1. As a consequence, intra_only_constraint_flag and single_layer_constraint_flag shall also be equal to 1. If it is appropriate for VVC to support still pictures with multi layers, the description regarding these constraints should be changed. This contribution proposes to modify these constraint flags to enable the support of still pictures with multi layers.

Introduction

In VVC Draft 9, bitstreams conforming to the still picture profile shall obey the constraint that the bitstream contains only one picture, and general_one_picture_only_constraint_flag has to be set equal to 1. As a result, intra_only_constraint_flag and single_layer_constraint_flag are also constrained to be set equal to 1. However, the authors of this document think there is no issue to support a multi layer structure in still picture, and it is necessary to clarify whether to prohibit a multi layer as the current description of these constraints or to allow it. A still picture with multi layers would achieve progressively scalable picture coding as progressive JPEG. If it is appropriate for VVC to support it, the description regarding these constraints should be changed. This contribution proposes following modifications;

- Modify the definition of still picture to one AU instead of one picture.

- Change general_one_picture_only_constraint_flag to general_one_au_only_constraint_flag.

- Remove the constraints in the semantics of intra_only_constraint_flag and single_layer_constraint_flag which set the value equal to 1 based on general_one_picture_only_constraint_flag.

Proposed changes

The suggested specification changes are described on top of VVC draft of JVET-R2001[1]. The newly added parts are highlighted in yellow and the text to be removed is marked as red.

7.3.3.2  General constraint information syntax

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| general_one_aupicture_only_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| single_layer_constraint_flag | u(1) |
| . . . |  |

7.4.4.2  General constraint information semantics

general_one_aupicture_only_constraint_flag equal to 1 specifies that there is only one AUcoded picture in the bitstream. general_one_aupicture_only_constraint_flag equal to 0 does not impose such a constraint.

intra_only_constraint_flag equal to 1 specifies that sh_slice_type shall be equal to I. intra_only_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of intra_only_constraint_flag shall be equal to 1.

single_layer_constraint_flag equal to 1 specifies that sps_video_parameter_set_id shall be equal to 0. single_layer_constraint_flag equal to 0 does not impose such a constraint. When general_one_picture_only_constraint_flag is equal to 1, the value of single_layer_constraint_flag shall be equal to 1.

A.3.1  Main 10 and Main 10 Still Picture profiles

Bitstreams conforming to the Main 10 or Main 10 Still Picture profile shall obey the following constraints:

- In a bitstream conforming to the Main 10 Still Picture profile, the bitstream shall contain only one AUpicture.

- Referenced SPSs shall have sps_chroma_format_idc equal to 0 or 1.

. . .

Conformance of a bitstream to the Main 10 profile is indicated by general_profile_idc being equal to 1.

Conformance of a bitstream to the Main 10 Still Picture profile is indicated by general_one_aupicture_only_constraint_flag being equal to 1 together with general_profile_idc being equal to 1.

. . .

A.3.2  Main 4:4:4 10 and Main 4:4:4 10 Still Picture profiles

Bitstreams conforming to the Main 4:4:4 10 or Main 4:4:4 10 Still Picture profile shall obey the following constraints:

- In a bitstream conforming to the Main 4:4:4 10 Still Picture profile, the bitstream shall contain only one AUpicture.

- Referenced SPSs shall have sps_chroma_format_idc in the range of 0 to 3, inclusive.

. . .

Conformance of a bitstream to the Main 4:4:4 10 profile is indicated by general_profile_idc being equal to 2.

Conformance of a bitstream to the Main 4:4:4 10 Still Picture profile is indicated by general_one_aupicture_only_constraint_flag being equal to 1 together with general_profile_idc being equal to 2.

. . .

Conclusion

This contribution proposes to modify the constraints of general_one_picture_only_constraint_flag, intra_only_constraint_flag, and single_layer_constraint_flag in the still picture profile. The proposed changes can enable the support of still pictures with multi layers in the still picture profile. The authors of this document believe that the proposed modifications should be adopted into VVC.

References

- B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001, JVET 18th Meeting: by teleconference, 15–24 April 2020.

Patent rights declaration(s)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).