| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0092 |  |
| --- | --- | --- |

| Title: | On constraint info signalling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution presents a proposal for a modification to the constraint info signalling of the VVC draft. It is proposed to move the extension length indicator from last to first in the constraint info syntax structure in order to allow skipping of signalling constraint info when all constraint flags are zero. The full specification text integrated in JVET-R2001-vA.docx and a software patch for VTM master branch (commit 8d9573f1ec214868e1a4e61e38991c5a07b86449) are attached to this contribution.

# Introduction

The VVC draft includes a mechanism for a variable-length general_constraint_info() syntax structure in order to allow additional constraints to be introduced in future versions of VVC. This is realized by an extension length indicator (called “gci_num_reserved_bytes” in the first version of VVC). It is asserted that it would be desirable to move this length indicator from last in the general_constraint_info() syntax structure to first in the general_constraint_info() syntax structure.

## Proposal

It is proposed to move the length indicator from last in the general_constraint_info() syntax structure to first in the general_constraint_info() syntax structure. The motivation for moving it is to be able to set the length equal to zero for encoders that do not want to set any constraint flags, for example because they produce bitstream that do not apply any constraints beyond what the defined profiles allow. If the length indicator is first in the general_constraint_info() syntax structure it can be set equal to 0 so that the general_constraint_info() syntax structure would only include the length indicator itself. This means that for the currently defined constraints, it would be possible to signal a single 1-byte syntax element (the length indicator) instead of having to signal 10 bytes of only zeros.

Figure 1 illustrates the position of the length indication in the current VVC draft. In the example in Figure 1, a 2-byte extension has been added in a future version of VVC.

Figure 1. Illustration of general_constraint_info() extension length indication in current VVC draft.

Figure 2 illustrates the position of the length indication in the proposal. Also in this example, a 2-byte extension has been added in a future version of VVC. Thus, the total length indicated will be 11.

Figure 2. Illustration of proposed general_constraint_info() length.

With the proposed change it is possible to set the length to be equal 0, meaning that there will be no bytes of data in the general_constraint_info() syntax structure except for the length indicator itself. This is illustrated in Figure 3.

Figure 3. Illustration of proposed length indication when no constraints are signalled.

The proposed move is show in the table below:

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| gci_num_constraint_bytes | u(8) |
| if( gci_num_constraint_bytes > 8 ) |  |
| { |  |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| … /* 61 more syntax elements */ |  |
| no_aps_constraint_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| } |  |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reservedconstraint_bytes − 9; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

gci_num_constraint_bytes specifies the length in bytes of the general constraint info structure, not including the byte used for signalling gci_num_constraint_bytes itself. The value of gci_num_constraint_bytes shall be equal to 0 or equal to 9. Other values of gci_num_constraint_bytes are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

gci_num_reserved_bytes specifies the number of the reserved constraint bytes. The value of gci_num_reserved_bytes shall be 0. Other values of gci_num_reserved_bytes are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification.

# Full specification text

The complete text integrated in the VVC draft text of JVET-R2001-vA.docx is attached to the contribution.

# Software

A complete software patch relative to VTM master branch (commit 8d9573f1ec214868e1a4e61e38991c5a07b86449) is attached to this contribution.

# Patent rights declaration(s)

Sharp Labs of America, Inc.may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).