| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0093 |  |
| --- | --- | --- |

| Title: | On slice header flags related to residual coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson Sachin Deshpande Frank Bossen Andrew Segall | Email: | samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution presents a proposal for a modification to the High-Level Syntax of the VVC draft to avoid signalling redundant information in slice header in many scenarios that are envisioned to be common use cases of VVC. The proposal is to bring back picture level control of Dependent Quantization (DQ) and Sign Data Hiding (SDH) and add an SPS level flag (sps_residual_control_present_in_sh_flag) to control the presence of the three slice header syntax elements sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag. It is further proposed to change the gating conditions for the slice header elements and adjust the inference rule to disable support for undesired combinations at the slice level. No changes are proposed to the decoding process or to any syntax tables below slice header level. The complete suggested text changes are included as an attachment to this contribution. A slide deck with further details of the proposal and highlighted text changes is also provided as an attachment to this contribution as well as a software patch for the proposal relative to VTM master (commit 8d9573f1ec214868e1a4e61e38991c5a07b86449).

# Introduction

At the 18th JVET meeting it was agreed to move control of Dependent Quantization (DQ) and Sign Data Hiding (SDH) from Picture Header (PH) to Slice Header (SH). This move was motivated by a desire to avoid that DQ or SDH is used in the same slice as Transform Skip (TS) with Regular Residual Coding (RRC) while at the same time enable efficient coding of a mixture of lossy slices and lossless slices within the same picture. However, a side-effect of moving the DQ and SDH flags to SH instead of PH is that all encoders that use any of these features would now have to repeatedly indicate in each slice of each picture that the tool is enabled for the slice. The authors of this contribution asserts that it is undesirable to introduce redundant signalling for common cases (i.e. general lossy coding with multiple slices) in the most expensive part of the High-Level Syntax, i.e. the slice header, in order to enable more efficient coding for less common cases (i.e. coding with multiple slices where some are coded lossy and some are coded lossless within the same picture). It is proposed to bring back PH control of DQ and SDH (as in VVC draft 7 and 8) and introduce an SPS level flag to indicate if SH control of DQ/SDH/TSRC is present. It is asserted that these two changes, combined with changed inference rules for the SH flags, brings back the ability to use DQ and SDH without redundant signalling in SH, and still makes it possible to do efficient coding of combined lossy/lossless slices. No changes are proposed to the decoding process or to any syntax tables below slice header level.

# Proposal

The proposal includes four modifications to the VVC draft:

- Addition of the SPS flag sps_residual_control_present_in_sh_flag.

- Addition of picture header flags for DQ and SDH.

- Modification of gating conditions for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to ensure that at most one of these flag is signalled at the slice level.

- Modifications of inference rules for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to match the desired behaviour.

These changes keep the property that DQ and SDH cannot be enabled in a slice that disables TSRC. The changes also ensure that no information needs to be signalled at slice header level for encoders that are not configured to do lossless encoding. At the same time, the proposed changes ensure that at most one of the three flags is signalled in any slice header.

None of the three slice header flags are signalled when sps_residual_control_present_in_sh_flag is set equal to 0.

When sps_residual_control_present_in_sh_flag is set equal to 1 the following three cases can occur:

If both ph_dep_quant_enabled_flag and ph_sign_data_hiding_enabled_flag are equal to 0, then only sh_ts_residual_coding_disabled_flag is signalled to determine if TSRC is used or not.

If ph_dep_quant_enabled_flag is equal to 1, then only sh_dep_quant_enabled_flag is signalled to determine if TSRC is used or not.

If ph_sign_data_hiding_enabled_flag is equal to 1, then only sh_sign_data_hiding_enabled_flag is signalled to determine if TSRC is used or not.

For all of these cases it applies that whenever TSRC is disabled, both DQ and SDH are disabled.

## Proposed text

The suggested text changes are included as an attachment to this contribution and additionally included in the slide deck accompanying this contribution.

## Software

The software for the proposal is attached as a patch relative to VTM master commit 8d9573f1ec214868e1a4e61e38991c5a07b86449.

# Patent rights declaration(s)

Sharp Labs of America, Inc.may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).