| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0094 |
| --- | --- |

| Title: | AHG9: Profile-level constraints on chroma format and bit depth constraint flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karl Sharman Adrian Browne Steve Keating | Email: | karl.sharman@sony.com adrian.browne@sony.com steve.keating@sony.com |
| Source: | Sony Europe |  |  |

_____________________________

# Abstract

In the current draft of VVC, the bit depth (“bit_depth_minus8”) and the chroma format (“chroma_format_idc”) are constrained by the constraint flags “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc”. In addition, they are constrained in the specification of each profile.

However, there is no constraint of specification of what “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc” should be for each profile. This means that it is possible for a stream to indicate maximums of 16 bit 4:4:4, but only be a 4:2:0 10-bit profile.

The constraint flags are present in the VPS, describing each layer, and optionally present in the SPS.

Four options are explored, and it is proposed that a change is made to remove this ambiguity.

# Problem statement

In the current draft of VVC, the bit depth (“bit_depth_minus8”) and the chroma format (“chroma_format_idc”) are constrained by the constraint flags “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc”. In addition, they are constrained in the specification of each profile.

However, there is no constraint of specification of what “max_bitdepth_constraint_idc” and “max_chroma_format_constraint_idc” should be for each profile. This means that it is possible for a stream to indicate maximums of 16 bit 4:4:4, but only be a 4:2:0 10-bit profile.

The constraint flags are present in the VPS, describing each layer, and optionally present in the SPS.

It is proposed that the constraint flags are constrained by the profile to prevent such unusual conditions.

The options are as follows:

- The constraint flags may be set to any value less than or equal to that implied by the profile; and add these constraints to the profile definition.

- The constraint flags are set exactly to maximum value implied by the profile; and add these constraints to the profile definition.

- The constraint flags may be set to any value less than or equal to that implied by the profile; replace the bit_depth_minus8 and chroma_format_idc constraints with these constraints to the profile definition. The original constraints are not necessary, as the is already a constraint on chroma_format_idc and bit_depth_minus8.

- The constraint flags are set exactly to maximum value implied by the profile; replace the bit_depth_minus8 and chroma_format_idc with these constraints to the profile definition. The original constraints are not necessary, as the is already a constraint on chroma_format_idc and bit_depth_minus8.

# Draft text.

The draft text of each of these is shown in the following sections, relative to JVET-R2001-vA.docx.

They illustrate the text for one profile, “Main 10”. However, corresponding changes would be needed for the other profiles described.

## Option 1: Addition of non-specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 0 or 1.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present in the range of 0 to 2, inclusive.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 2: Addition of specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 1.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present equal to 2.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 3: Replacement of non-specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 0 or 1.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present in the range of 0 to 2, inclusive.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

## Option 4: Replacement of specific constraints to profiles

…

  - Profiles

    - Main 10 profile

Bitstreams conforming to the Main 10 profile shall obey the following constraints:

- Referenced SPSs shall have chroma_format_idc equal to 0 or 1.

- Referenced VPSs and SPSs shall have max_chroma_format_constraint_idc when present equal to 1.

- Referenced SPSs shall have bit_depth_minus8 in the range of 0 to 2, inclusive.

- Referenced VPSs and SPSs shall have max_bitdepth_constraint_idc when present equal to 2.

- Referenced SPSs shall have sps_palette_enabled_flag equal to 0.

…

# Conclusion

It is proposed that one of these changes is adopted to remove the ambiguity of these constraint flags.

# Patent rights declaration(s)

Sony Corporation or Sony Group companies may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).