| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0095-v1 |
| --- | --- |

| Title: | AHG12: On tile and slice partitioning related syntax and semantics |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jie Chen, Yan Ye, Ru-Ling Liao Radiance Jin Hui Tower, Building No.6, The 4 District Wangjing Dong Yuan, Chaoyang District, Beijing, China | Tel: Email: | jiechen.cj@alibaba-inc.com |
| Source: | Alibaba Inc. |  |  |

_____________________________

- Abstract

This contribution includes six proposals related to tile and slice partitioning as follows

- Proposal 1: it is proposed to remove the signaling redundancy of rectangular slice within a tile. Two options are provided. Option 1 is to replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ] and option 2 is to change the upper limit of pps_exp_slice_height_in_ctus_minus1[i][j] from RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 1 to  RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 2;

- Proposal 2: Considering the fact that the sum of explicitly signaled heights of rectangular slice within a tile should be equal to or less than the height of the tile, a more precise range of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] which is from 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − pps_num_exp_slices_in_tile[ i ] is proposed.

- Proposal 3: When the number of tiles in a picture is less than 4, the layout of raster-scan slice can be also indicated by rectangular slice mode. Thus, it is proposed to only signal of pps_rect_slice_flag when NumTilesInPic is greater than 3 to reduce unnecessary signaling.

- Proposal 4: Two options are proposed to change the constraint that the value of pps_no_pic_partition_flag shall be the same within a CLVS. Option 1 is to signal sps_no_pic_partition_flag in SPS instead of signaling pps_no_pic_partition_flag in PPS since the value of the flag is already restricted to be constant in a CLVS; option 2 is to remove this constraint, since even with this constraint the mixture of pictures with tile/slice partitioning and pictures without tile/slice partitioning within a CLVS is still allowed.

- Proposal 5: The semantics of pps_rect_slice_flag is fixed to cover the case that rectangular slice is smaller than the tile.

- Proposal 6: The inference condition of pps_single_slice_per_subpic_flag is changed from “not present” to “the value of pps_no_pic_partition_flag is equal to 1” so that pps_single_slice_per_subpic_flag is inferred only when it is needed.

Proposal 1: On signaling of rectangular slice within a tile

In VVC draft 9, pps_num_exp_slices_in_tile[ i ] and pps_exp_slice_height_in_ctus_minus1[ i ][ j ] are signaled to specify the layout of rectangular slice within a tile. However, in the current spec, there can be two different signalings to indicate the case that there is only one slice as follows:

- pps_num_exp_slices_in_tile[ i ] is siganled to be 0 and pps_exp_slice_height_in_ctus_minus1[ i ][ j ]  is not signlaed

- pps_num_exp_slices_in_tile[ i ] is signaled to be 1 and pps_exp_slice_height_in_ctus_minus1[ i ][ j ] is signaled to be RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 1

In the first way, pps_num_exp_slices_in_tile[ i ] is equal to 0 and signalling of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] is skipped. In this case, the number of slice is derived as 1 and the height of the slice is derived as the height of tile in equation (30) in the spec. In the second case, there is only one slice of which the height is explicitly signalled, but the height is signalled to be equal to the height of tile, so there can be only one slice within the tile. Both ways indicate the case that there is only one slice within the current tile. So it is redundant.

Two options are provided in this proposal to remove the redundancy.

Option 1

Option 1 is to replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ] as what we do when we signal the number of explictly signaled tile coloum and tile row. In option 1, first way is removed,so encoder can only use the second way to indicate the case that one slice within one tile.

The spec. changes are as follows

Table 1. PPS syntax change of proposal 1

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if( pps_rect_slice_flag ) |  |
| pps_single_slice_per_subpic_flag | u(1) |
| if( pps_rect_slice_flag && !pps_single_slice_per_subpic_flag ) { |  |
| pps_num_slices_in_pic_minus1 | ue(v) |
| if( pps_num_slices_in_pic_minus1 > 1 ) |  |
| pps_tile_idx_delta_present_flag | u(1) |
| for( i = 0; i < pps_num_slices_in_pic_minus1; i++ ) { |  |
| if( SliceTopLeftTileIdx[ i ] % NumTileColumns != NumTileColumns − 1 ) |  |
| pps_slice_width_in_tiles_minus1[ i ] | ue(v) |
| if( SliceTopLeftTileIdx[ i ] / NumTileColumns != NumTileRows − 1 && ( pps_tile_idx_delta_present_flag ¦ ¦ SliceTopLeftTileIdx[ i ] % NumTileColumns = = 0 ) ) |  |
| pps_slice_height_in_tiles_minus1[ i ] | ue(v) |
| if( pps_slice_width_in_tiles_minus1[ i ] = = 0 && pps_slice_height_in_tiles_minus1[ i ] = = 0 && RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] > 1 ) { |  |
| pps_num_exp_slices_in_tile[ i ] pps_num_exp_slices_in_tile_minus1[ i ] | ue(v) |
| for( j = 0; j < pps_num_exp_slices_in_tile[ i ] <=pps_num_exp_slices_in_tile_minus1[ i ]; j++ ) |  |
| pps_exp_slice_height_in_ctus_minus1[ i ][ j ] | ue(v) |
| i += NumSlicesInTile[ i ] − 1 |  |
| } |  |
| if( pps_tile_idx_delta_present_flag && i < pps_num_slices_in_pic_minus1 ) |  |
| pps_tile_idx_delta_val[ i ] | se(v) |
| } |  |
| } |  |
| if( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) |  |
| pps_loop_filter_across_slices_enabled_flag | u(1) |
| } |  |
| … | u(1) |

pps_num_exp_slices_in_tile_minus1[ i ] plus 1 specifies the number of explicitly provided slice heights for the slices in the tile containing the i-th slice (i.e., the tile with tile index equal to SliceTopLeftTileIdx[ i ]). The value of pps_num_exp_slices_in_tile[ i ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 12, inclusive. When not present, the value of pps_num_exp_slices_in_tile[ i ] is inferred to be equal to 0.

pps_exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice, in units of CTU rows, for j in the range of 0 to pps_num_exp_slices_in_tile_minus1[ i ] − 1, inclusive, when pps_num_exp_slices_in_tile[ i ] is greater than 0. pps_exp_slice_height_in_ctus_minus1[ i ][ pps_num_exp_slices_in_tile_minus1[ i ] ] is also used to derive the heights of the rectangular slices in the tile containing the i-th slice with index greater than pps_num_exp_slices_in_tile_minus1 [ i ]  − 1 as specified in clause 6.5.1. The value of pps_exp_slice_height_in_ctus_minus1 [ i ][ j ]  shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1, inclusive.

The changes of equation (3) are as follows

if( sliceWidthInTiles[ i ]  = =  1  &&  sliceHeightInTiles[ i ]  = =  1 ) {	
			if( pps_num_exp_slices_in_tile[ i ]  = = 0 ) {
				NumSlicesInTile[ i ] = 1
				sliceHeightInCtus[ i ] = RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ]
			} else {
				remainingHeightInCtbsY = RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ]
				for( j = 0; j < =pps_num_exp_slices_in_tile_minus1[ i ]; j++ ) {
					sliceHeightInCtus[ i + j ] = pps_exp_slice_height_in_ctus_minus1[ i ][ j ] + 1
					remainingHeightInCtbsY   −=  sliceHeightInCtus[ i + j ]
				}
				uniformSliceHeight = sliceHeightInCtus[ i + j − 1 ]
				while( remainingHeightInCtbsY  >=  uniformSliceHeight ) {
					sliceHeightInCtus[ i + j ] = uniformSliceHeight
					remainingHeightInCtbsY  −=  uniformSliceHeight
					j++
				}
				if( remainingHeightInCtbsY > 0 ) {
					sliceHeightInCtus[ i + j ] = remainingHeightInCtbsY
					j++
				}
				NumSlicesInTile[ i ] = j
			}

Option 2

In option 2, the range of pps_exp_slice_height_in_ctus_minus1[i][j] is restricted so that way 2 is removed. In this option, the maximun value of pps_exp_slice_height_in_ctus_minus1[i][j] is set to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 2 so that if there is only one slice of which the height is explicitly signaled, the height cannot be euqal to the height of tile. Therefore, there must be at least 2 slices in this case.

The spec. changes of option 2 are as follows

pps_exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice, in units of CTU rows, for j in the range of 0 to pps_num_exp_slices_in_tile[ i ] − 1, inclusive, when pps_num_exp_slices_in_tile[ i ] is greater than 0. pps_exp_slice_height_in_ctus_minus1[ i ][ pps_num_exp_slices_in_tile[ i ] ] is also used to derive the heights of the rectangular slices in the tile containing the i-th slice with index greater than pps_num_exp_slices_in_tile[ i ] − 1 as specified in clause 6.5.1. The value of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 12, inclusive.

Proposal 2: On range of pps_exp_slice_height_in_ctus_minus1[ i ][ j ]

In VVC draft 9, pps_exp_slice_height_in_ctus_minus1[ i ][ j ] is to indicate the height of rectangular slices within a tile. The current range of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] is from 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – 1 (i.e., height of tile minus1). It is correct if we consider individual slice.

However, this is in the case that one or multiple slices within the tiles, so the the sum of all the tiles within a tile should be equal to the height of the tile. As the minimum value of slice height is 1, the maximum value of slice height should be tile height – the number of slices - 1.

As the we already know the number of explicitly signaled slice heights (pps_num_exp_slices_in_tile[i]), it is proposed to change the range of pps_exp_slice_height_in_ctus_minus1[i][j] to from 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − pps_num_exp_slices_in_tile[ i ].

The spec. changes of proposal 2 are as follows

pps_exp_slice_height_in_ctus_minus1[ i ][ j ] plus 1 specifies the height of the j-th rectangular slice in the tile containing the i-th slice, in units of CTU rows, for j in the range of 0 to pps_num_exp_slices_in_tile[ i ] − 1, inclusive, when pps_num_exp_slices_in_tile[ i ] is greater than 0. pps_exp_slice_height_in_ctus_minus1[ i ][ pps_num_exp_slices_in_tile[ i ] ] is also used to derive the heights of the rectangular slices in the tile containing the i-th slice with index greater than pps_num_exp_slices_in_tile[ i ] − 1 as specified in clause 6.5.1. The value of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] shall be in the range of 0 to RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] − 1 pps_num_exp_slices_in_tile[ i ], inclusive.

Please note that if combine this change with option 2 of proposal 1, the maximum value of pps_exp_slice_height_in_ctus_minus1[ i ][ j ] can be written as RowHeight[ SliceTopLeftTileIdx[ i ] / NumTileColumns ] – max(2, pps_num_exp_slices_in_tile[ i ])

Proposal 3: On signaling of pps_rect_slice_flag

pps_rect_slice_flag is a flag to indicate whether raster-scan slice or rectangular slice is used. In VVC draft 9, pps_rect_slice_flag is signaled when NumTilesInPic is greater than 1. That is because if there is only 1 tile in the picture, the paritioning of raster-scan slice can be implemented by rectangular slice mode. So pps_rect_slice_flag is not signaled but inferred to be 1.

However, since the tiles are paritioned by tile rows and tile columns, if the number of tile in a picture is less than 3, there must be only one tile row or one tile column. In that case, the partitioning of raster-scan slice can also be implemented by rectangular slice mode. So in this proposal, it is proposed skip the signaling of pps_rect_slice_flag is the numbe rof tiles in a picture is less than 4 and infer the value of pps_rect_slice_flag to be equal to 1.

The spec. changes are as follows

| pic_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(1) |
| if( !pps_no_pic_partition_flag ) { |  |
| pps_log2_ctu_size_minus5 | u(2) |
| pps_num_exp_tile_columns_minus1 | ue(v) |
| pps_num_exp_tile_rows_minus1 | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_columns_minus1; i++ ) |  |
| pps_tile_column_width_minus1[ i ] | ue(v) |
| for( i = 0; i <= pps_num_exp_tile_rows_minus1; i++ ) |  |
| pps_tile_row_height_minus1[ i ] | ue(v) |
| if( NumTilesInPic > 1 ) { |  |
| pps_loop_filter_across_tiles_enabled_flag | u(1) |
| if(NumTilesInPic > 3) |  |
| pps_rect_slice_flag | u(1) |
| } |  |
| ... |  |
| … | u(1) |

Proposal 4: on constraint on pps_no_pic_partition_flag

pps_no_pic_partition_flag is signaled in PPS to indicate whether the pictures referring to the PPS is partitioned into multiple tiles or slices. In VVC draft 9, there is a constraint that the value of pps_no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS. That is to say, although pps_no_pic_partition_flag is singled in PPS, it is a CLVS level flag. The value of this flag shall be constant within the whole CLVS.

As a general design principle, a CLVS level syntax element should be signaled in SPS rather than PPS. To solve this problems, two options are provided in proposal 3

Option 1

In the first option, it is proposed to signal sps_no_pic_parition_flag in SPS and change the constraint to that pps_no_pic_partition_flag shall be equal to sps_no_pic_parition_flag. By signaling sps_no_pic_parition_flag in SPS, the subpic information can be conditionally signaled on it.

The spec. changes are as follows

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … | u(2) |
| sps_no_pic_partition_flag | u(1) |
| if(!sps_no_pic_partition_flag) |  |
| sps_subpic_info_present_flag | u(1) |
| if( sps_subpic_info_present_flag ) { |  |
| sps_num_subpics_minus1 | ue(v) |
| if( sps_num_subpics_minus1 > 0 ) |  |
| sps_independent_subpics_flag | u(1) |
| … | u(1) |
| } |  |
| … |  |

sps_no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the SPS. pps_no_pic_partition_flag equal to 0 specifies that each picture referring to the SPS may be partitioned into more than one tile or slice.

sps_subpic_info_present_flag equal to 1 specifies that subpicture information is present for the CLVS and there may be one or more than one subpicture in each picture of the CLVS. sps_subpic_info_present_flag equal to 0 specifies that subpicture information is not present for the CLVS and there is only one subpicture in each picture of the CLVS. When not present, the value of sps_subpic_info_present_flag is inferred to be equal to 0

pps_no_pic_partition_flag equal to 1 specifies that no picture partitioning is applied to each picture referring to the PPS. pps_no_pic_partition_flag equal to 0 specifies that each picture referring to the PPS may be partitioned into more than one tile or slice.

It is a bitstream conformance constraint that the value of pps_no_pic_partition_flag shall be the equal to the value of sps_no_pic_partition_flag.

Option 2

However, it is noted that pps_no_pic_partition_flag has “one-way” semantics, which means even with this constraint, the partitioning behavior may be different in different pictures, i.e., there can be some pictures with no tile or slice partitioning and other pictures with tile or slice partitioning in a CLVS. Thus, this constraint doesn’t impose any actual restriction on tile/slice partitioning for pictures in a CLVS.

So in option 2, it is proposed to remove this constraint. By doing this, pps_no_pic_partition_flag becomes a real PPS level flag of which the value may change from one picture to another picture. Thus, the problem mentioned before is also solved.

It is a requirement of bitstream conformance that the value of pps_no_pic_partition_flag shall be the same for all PPSs that are referred to by coded pictures within a CLVS.

Proposal 5: on semantics of pps_rect_slice_flag

pps_rect_slice_flag is signaled in PPS to indicate whether raster-scan slice or rectangular slice is applied to the picture referring to the PPS. The semantics is as follows

pps_rect_slice_flag equal to 0 specifies that tiles within each slice are in raster scan order and the slice information is not signalled in PPS. pps_rect_slice_flag equal to 1 specifies that tiles within each slice cover a rectangular region of the picture and the slice information is signalled in the PPS. When not present, pps_rect_slice_flag is inferred to be equal to 1. When sps_subpic_info_present_flag is equal to 1 or pps_mixed_nalu_types_in_pic_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1.

However, for rectangular slice, it can be larger than a tile or smaller than a tile. The current semantics of pps_rect_slice_flag equal to 1 only specifies the case that a rectangular slice covers one or multiple tiles. But there also is a case that a tile covers multiple slices. So the current semantics of pps_rect_slice_flag is incomplete.

It is suggested to change the semantics of pps_rect_slice_flag as follows

pps_rect_slice_flag equal to 0 specifies that tiles within each slice are in raster scan order and the slice information is not signalled in PPS. pps_rect_slice_flag equal to 1 specifies that tiles within each slice cover a rectangular region of the picture or each slice within a tile covers one or more consecutive complete CTU rows of the tile, and the slice information is signalled in the PPS. When not present, pps_rect_slice_flag is inferred to be equal to 1. When sps_subpic_info_present_flag is equal to 1 or pps_mixed_nalu_types_in_pic_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1.

Alternatively, since section 6.3.1 of VVC draft 9 gives the clear definition of raster-scan mode and rectangular slice mode. Thus, it is further suggested to simplify the semantics as follows

pps_rect_slice_flag equal to 0 specifies that tiles within each slice are in raster scan order raster-san slice mode is applied to each picture referring to the PPS and the slice information is not signalled in PPS. pps_rect_slice_flag equal to 1 specifies that tiles within each slice cover a rectangular region of the picture reactangluar slice mode is applied to each picture referring to the PPS and the slice information is signalled in the PPS. When not present, pps_rect_slice_flag is inferred to be equal to 1. When sps_subpic_info_present_flag is equal to 1 or pps_mixed_nalu_types_in_pic_flag is equal to 1, the value of pps_rect_slice_flag shall be equal to 1.

Proposal 6: on semantics of pps_single_slice_per_subpic_flag

In VVC draft 9, the semantics of pps_single_slice_per_subpic_flag is as follows

pps_single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. pps_single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist of one or more rectangular slices. When not present, the value of pps_single_slice_per_subpic_flag is inferred to be equal to 1

According to the semantics, pps_single_slice_per_subpic_flag is only relevant in rectangular slice mode, since it specifies whether each subpicture consists of multiple rectangular slices. When pps_single_slice_per_subpic_flag is not present, it is inferred to be 1.

However, there are two cases that pps_single_slice_per_subpic_flag is not present according to the current syntax. The first case is that pps_no_pic_partition_flag is equal to 1 and the second case is that pps_rect_slice_flag is equal to 0. When pps_rect_slice_flag is equal to 0, it is raster-scan slice mode, so pps_single_slice_per_subpic_flag is irrelevant. Thus, inferring the value of pps_single_slice_per_subpic_flag when pps_rect_slice_flag is equal to 0 doesn’t make sense. So in this proposal, it is suggested to infer the value of pps_single_slice_per_subpic_flag only when pps_no_pic_partition_flag is equal to 1.

The spec. changes are as follows

pps_single_slice_per_subpic_flag equal to 1 specifies that each subpicture consists of one and only one rectangular slice. pps_single_slice_per_subpic_flag equal to 0 specifies that each subpicture may consist of one or more rectangular slices. When not present the value of pps_no_pic_partition_flag is equal to 1, the value of pps_single_slice_per_subpic_flag is inferred to be equal to 1.

Conclusion

Six proposals are proposed in this contribution. Proposal 1 is either to replace replace pps_num_exp_slices_in_tile[ i ] with pps_num_exp_slices_in_tile_minus1[ i ] or to change the range of pps_exp_slice_height_in_ctus_minus1[i][j] to remove the signaling redudancy; proposal 2 is to change the range pps_exp_slice_height_in_ctus_minus1[ i ][ j ] to a more precise range; proposal 3 is to skip the signaling of pps_rect_slice_flag when NumTilesInPic is less than 4 to reduce unnecessary signaling; proposal 4 is either to signal sps_no_pic_partition_flag in SPS or remove this constraint on pps_no_pic_partition_flag; proposal 5 is to fix the semantics of pps_rect_slice_flag; proposal 6 is to change the inference condition of pps_single_slice_per_subpic_flag.

It is suggested to adopt these proposals into next version of VVC draft.

Reference

B. Bross, J. Chen, S. Liu, Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001-vA, May 2020.

Patent rights declaration(s)

Alibaba Group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).