| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0099 |
| --- | --- |

| Title: | AHG12: SEI message handling in subpicture extraction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Sühring (HHI) Virginie Drugeon (Panasonic) | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de +49-6103-5996407 virginie.drugeon@eu.panasonic.com |
| Source: | Fraunhofer HHI, Panasonic |  |  |

_____________________________

# Abstract

This contribution compares the handling of scalable-nested SEI messages in the extraction processes of a target OLS (Annex C.6) and for a target subpicture (Annex C.7) and proposes additions to the subpicture-specific extraction process for alignment that are asserted to be on oversight and intended by design. It is proposed to add to Annex C.7:

- Removal of scalable-nested SEI messages for non-target subpictures

- Removal before replacing non-scalable-nested SEI messages

# Introduction

This contribution compares the handling of scalable-nested SEI messages in the extraction processes of a target OLS (Annex C.6) and for a target subpicture (Annex C.7) and proposes additions to the subpicture-specific extraction process for alignment that are asserted to be on oversight and intended by design.

Scalable-nested SEI messages for non-target subpictures

In the OLS-specific extraction process in Annex C.6 of the VVC draft specification, scalable-nested SEI messages that do are not associated to the extracted OLS, are removed from the bitstream as follows:

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_nesting_ols_flag equal to 1 and there is no value of i in the range of 0 to sn_nesting_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

However, the subpicture-specific extraction process in Annex C.7 does not include a corresponding step to remove the scalable-nested SEI messages that are not associated to the extracted subpicture.

It is asserted that the usability of the scalable-nested SEI message that do are not associated to the target subpicture is very limited after extraction and hence it is proposed to add removal of said messages to the subpicture extraction process. The related proposed specification changes are highlighted in yellow below.

Replaced non-scalable-nested SEI messages

The OLS-specific extraction process in Annex C.6 of the VVC draft specification removes non-scalable-nested SEI message related to HRD conformance before decapsulating applicable HRD-related SEI messages from the scalable-nesting SEI message as a replacement as follows:

–	When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all NAL units in the bitstream, the following applies:

–	Remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 0 (buffering period) or 130 (decoding unit information).

–	When general_same_pic_timing_in_all_ols_flag is equal to 0, remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 1 (picture timing).

However, the subpicture-specific extraction process in Annex C.7 only decapsulates SEI messages from the scalable-nesting SEI message without removing the corresponding non-scalable SEI messages first. It is asserted that this is an oversight and proposed to add a corresponding step. The related proposed specification changes are highlighted in green below.

# Proposed specification changes

C.7	Subpicture sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and an array of target subpicture index values for each layer subpicIdxTarget[ ].

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and subpicIdxTarget[ ] equal to a subpicture index present in the OLS, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and with sh_subpic_id equal to the value in SubpicIdVal[ subpicIdxTarget[ i ] ] for each i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive.

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

[…]

–	When sli_cbr_constraint_flag is equal to 1, remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages that are not associated with the VCL NAL units of a subpicture in subpicIdTarget[ ] and set cbr_flag[ tIdTarget ][ j ] equal to 1 of the j-th CPB in the vps_ols_hrd_idx[ MultiLayerOlsIdx[ targetOlsIdx ] ]-th ols_hrd_parameters( ) syntax structure in all the referenced VPS NAL units and SPS NAL units and j in the range of 0 to hrd_cpb_cnt_minus1. Otherwise, (sli_cbr_constraint_flag is equal to 0), remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages and set cbr_flag[ tIdTarget ][ j ] equal to 0.

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_nesting_ols_flag equal to 1 and sn_nesting_subpic_flag equal to 1 that are applicable to outBitstream and there is no value of i in the range of 0 to sn_nesting_num_subpics_minus1, inclusive, such that sn_nesting_subpic_id[ i ] is equal to SubpicIdVal[ subpicIdxTarget[ [ NumLayersInOls[ targetOlsIdx ] − 1 ] ]].

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with sn_nesting_ols_flag equal to 1 and sn_nesting_subpic_flag equal to 1 that are applicable to outBitstream, the following applies:

–	Remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 1 (picture timing), 130 (decoding unit information) or 132 (decoded picture hash).

–	Extract appropriate non-scalable-nested SEI message with payloadType equal to 1 (picture timing), 130 (decoding unit information), or 132 (decoded picture hash) from the scalable nesting SEI message and place the extracted SEI messages into outBitstream.

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)

Panasonic Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).