| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0109-v1 |
| --- | --- |

| Title: | AHG9: Signalling of picture rate |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Brian Heng Wade Wan | Email: | firstname.lastname@broadcom.com |
| Source: | Broadcom Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to enable signalling picture rate information (num_units_in_tick, time_scale, etc.), without requiring the additional syntax necessary to signal a complete HRD model (bitrate, buffer size, delivery model, etc.).  It is asserted there are applications that can benefit from signaling picture rate without the overhead of also requiring a complete HRD model and also noted that this capability was present in AVC as well as HEVC.

Specifically, this contribution recommends removing the current restriction that general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag shall not be both equal to 0.

Additionally, when general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag are both equal to 0, it is proposed that general HRD parameter syntax that is relevant only when signalling an HRD model can also be skipped.

# Introduction

This contribution asserts that many applications benefit from picture rate to be signalled within the bitstream. Even with formats where higher system layers carry timing information, the presence of picture rate encoded in the bitstream can be beneficial when operations such as demultiplexing/remultiplexing are performed and also ensure the intended picture rate (essential information needed to determine correct display format) is preserved.

In the VVC draft text, JVET-R2001-vA, the information relevant to picture timing (num_units_in_tick, time_scale, fixed_pic_rate_flag, and elemental_duration_in_tc_minus1) cannot be signalled without also sending a full HRD model. This includes several parameters that signal bitrate, buffer size, constant or variable bitrate delivery model, etc.

Section 7.3.5.1

| general_hrd_parameters( ) { | Descriptor |
| --- | --- |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| general_nal_hrd_params_present_flag | u(1) |
| general_vcl_hrd_params_present_flag | u(1) |
| general_same_pic_timing_in_all_ols_flag | u(1) |
| general_decoding_unit_hrd_params_present_flag | u(1) |
| if( general_decoding_unit_hrd_params_present_flag ) |  |
| tick_divisor_minus2 | u(8) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| if( general_decoding_unit_hrd_params_present_flag ) |  |
| cpb_size_du_scale | u(4) |
| hrd_cpb_cnt_minus1 | ue(v) |
| } |  |

As shown above, the num_units_in_tick and time_scale syntax elements that are used for indicating the picture rate are signalled in the general HRD syntax table.

The NAL and VCL HRD parameters present flags (general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag) are also found in this same table and these two flags are not allowed to both be equal to 0 (as described in Section 7.4.6.1 which is copied below for convenience).

Section 7.4.6.1

“It is a requirement of bitstream conformance that the values of general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag in each general_hrd_parameters( ) syntax structure shall not be both equal to 0.”

Therefore, in the current VVC draft text, picture timing information cannot be sent without sending a number of additional HRD model parameters and these model parameters may or may not be particularly useful for a given application. This was not the case in either AVC or HEVC which allows a bitstream to convey picture timing information without also requiring a complete HRD model.

Thus, this contribution proposes to enable the option of sending picture timing information without also requiring a complete HRD model.

# Proposed Changes

It is proposed to remove the requirement that general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag shall not be both equal to 0. In this way, timing information can be signalled without sending a full HRD model.

Additionally, when general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag are both equal to 0, general HRD parameter syntax that is relevant only when signalling an HRD model can be skipped.

Proposed changes to the draft text are provided below.

Section 7.4.6.1

“It is a requirement of bitstream conformance that the values of general_nal_hrd_params_present_flag and general_vcl_hrd_params_present_flag in each general_hrd_parameters( ) syntax structure shall not be both equal to 0.”

Section 7.3.5.1

| general_hrd_parameters( ) { | Descriptor |
| --- | --- |
| num_units_in_tick | u(32) |
| time_scale | u(32) |
| general_nal_hrd_params_present_flag | u(1) |
| general_vcl_hrd_params_present_flag | u(1) |
| general_same_pic_timing_in_all_ols_flag | u(1) |
| if( general_nal_hrd_params_present_flag ¦ ¦ general_vcl_hrd_params_present_flag ) { |  |
| general_decoding_unit_hrd_params_present_flag | u(1) |
| if( general_decoding_unit_hrd_params_present_flag ) |  |
| tick_divisor_minus2 | u(8) |
| bit_rate_scale | u(4) |
| cpb_size_scale | u(4) |
| if( general_decoding_unit_hrd_params_present_flag ) |  |
| cpb_size_du_scale | u(4) |
| hrd_cpb_cnt_minus1 | ue(v) |
| } |  |
| } |  |

Section 7.3.5.2

| ols_hrd_parameters( firstSubLayer, maxSubLayers ) { | Descriptor |
| --- | --- |
| for( i = firstSubLayer; i <= maxSubLayers; i++ ) { |  |
| fixed_pic_rate_general_flag[ i ] | u(1) |
| if( !fixed_pic_rate_general_flag[ i ] ) |  |
| fixed_pic_rate_within_cvs_flag[ i ] | u(1) |
| if( fixed_pic_rate_within_cvs_flag[ i ] ) |  |
| elemental_duration_in_tc_minus1[ i ] | ue(v) |
| else if( (general_nal_hrd_params_present_flag ¦ ¦ general_vcl_hrd_params_present_flag ) && hrd_cpb_cnt_minus1 = = 0 ) |  |
| low_delay_hrd_flag[ i ] | u(1) |
| if( general_nal_hrd_params_present_flag ) |  |
| sublayer_hrd_parameters( i ) |  |
| if( general_vcl_hrd_params_present_flag ) |  |
| sublayer_hrd_parameters( i ) |  |
| } |  |
| } |  |

# Patent rights declaration(s)

Broadcom Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).