| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0113v2 |
| --- | --- |

| Title: | AHG9: On new GCI flags |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yong He, Yao-Jen Chang, Nan Hu, Vadim Seregin, Muhammed Coban, Marta Karczewicz | Email: | yonghe@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes several new GCI flags as follows:

- max_layers_constraint_idc

- max_sublayers_constraint_idc

- max_subpics_constraint_idc

- no_pic_partition_constraint_flag

- no_rectangular_slice_constraint_flag

- no_raster_scan_slice_constraint_flag

- one_slice_per_subpicture_constraint_flag

- no_VUI_constraint_flag

- general_non_omnidirectional_video_constraint_flag

- general_non_dependent_RAP_constraint_flag

- general_non_HRD_constraint_flag

# Proposals

Several new GCI flags are proposed in

Table 1 Proposed GCI syntax elements

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| general_non_packed_constraint_flag | u(1) |
| general_frame_only_constraint_flag | u(1) |
| general_non_projected_constraint_flag | u(1) |
| general_one_picture_only_constraint_flag | u(1) |
| general_non_omnidirectional_video_constraint_flag | u(1) |
| general_non_dependent_RAP_constraint_flag | u(1) |
| general_non_HRD_constraint_flag | u(1) |
| intra_only_constraint_flag | u(1) |
| max_bitdepth_constraint_idc | u(4) |
| max_chroma_format_constraint_idc | u(2) |
| max_layers_constraint_idc | u(6) |
| max_sublayers_constraint_idc | u(3) |
| single_layer_constraint_flag | u(1) |
| all_layers_independent_constraint_flag | u(1) |
| no_ref_pic_resampling_constraint_flag | u(1) |
| no_res_change_in_clvs_constraint_flag | u(1) |
| no_pic_partition_constraint_flag | u(1) |
| one_tile_per_pic_constraint_flag | u(1) |
| pic_header_in_slice_header_constraint_flag | u(1) |
| one_slice_per_pic_constraint_flag | u(1) |
| one_subpic_per_pic_constraint_flag | u(1) |
| max_subpics_constraint_idc | u(10) |
| one_slice_per_subpicture_constraint_flag | u(1) |
| no_rectangular_slice_constraint_flag | u(1) |
| no_raster_scan_slice_constraint_flag | u(1) |
| no_qtbtt_dual_tree_intra_constraint_flag | u(1) |
| no_partition_constraints_override_constraint_flag | u(1) |
| no_sao_constraint_flag | u(1) |
| no_alf_constraint_flag | u(1) |
| no_ccalf_constraint_flag | u(1) |
| no_joint_cbcr_constraint_flag | u(1) |
| no_mrl_constraint_flag | u(1) |
| no_isp_constraint_flag | u(1) |
| no_mip_constraint_flag | u(1) |
| no_ref_wraparound_constraint_flag | u(1) |
| no_temporal_mvp_constraint_flag | u(1) |
| no_sbtmvp_constraint_flag | u(1) |
| no_amvr_constraint_flag | u(1) |
| no_bdof_constraint_flag | u(1) |
| no_dmvr_constraint_flag | u(1) |
| no_cclm_constraint_flag | u(1) |
| no_mts_constraint_flag | u(1) |
| no_sbt_constraint_flag | u(1) |
| no_lfnst_constraint_flag | u(1) |
| no_affine_motion_constraint_flag | u(1) |
| no_mmvd_constraint_flag | u(1) |
| no_smvd_constraint_flag | u(1) |
| no_prof_constraint_flag | u(1) |
| no_bcw_constraint_flag | u(1) |
| no_ibc_constraint_flag | u(1) |
| no_ciip_constraint_flag | u(1) |
| no_gpm_constraint_flag | u(1) |
| no_ladf_constraint_flag | u(1) |
| no_transform_skip_constraint_flag | u(1) |
| no_bdpcm_constraint_flag | u(1) |
| no_palette_constraint_flag | u(1) |
| no_act_constraint_flag | u(1) |
| no_lmcs_constraint_flag | u(1) |
| no_cu_qp_delta_constraint_flag | u(1) |
| no_chroma_qp_offset_constraint_flag | u(1) |
| no_dep_quant_constraint_flag | u(1) |
| no_sign_data_hiding_constraint_flag | u(1) |
| no_tsrc_constraint_flag | u(1) |
| no_mixed_nalu_types_in_pic_constraint_flag | u(1) |
| no_VUI_constraint_flag | u(1) |
| no_trail_constraint_flag | u(1) |
| no_stsa_constraint_flag | u(1) |
| no_rasl_constraint_flag | u(1) |
| no_radl_constraint_flag | u(1) |
| no_idr_constraint_flag | u(1) |
| no_cra_constraint_flag | u(1) |
| no_gdr_constraint_flag | u(1) |
| no_aps_constraint_flag | u(1) |
| while( !byte_aligned( ) ) |  |
| gci_alignment_zero_bit | f(1) |
| gci_num_reserved_bytes | u(8) |
| for( i = 0; i < gci_num_reserved_bytes; i++ ) |  |
| gci_reserved_byte[ i ] | u(8) |
| } |  |

The semantics of the proposed GCI syntax elements are specified as follows:

A new GCI flag is proposed to constrain the number of layers as follows:

max_layers_constraint_idc specifies that vps_max_layers_minus1 shall be in the range of 0 to max_layers_constraint_idc, inclusive. When single_layer_constraint_flag equal to 1, max_layers_constraint_idc shall be equal to 0.

A new GCI flag is proposed to constrain the number of sublayers as follows:

max_sublayers_constraint_idc specifies that vps_max_sublayers_minus1 shall be in the range of 0 to max_sublayers_constraint_idc, inclusive.

A new GCI flag is proposed to constrain the number of subpictures as follows:

max_subpics_constraint_idc specifies that sps_num_subpics_minus1 shall be in the range of 0 to max_subpics_idc -1, inclusive. When one_subpic_per_pic_constraint_flag is equal to 1, max_subpics_constraint_idc shall be equal to 0.

A new GCI flag is proposed to constrain the picture partition as follows:

no_pic_partition_constraint_flag equal to 1 specifies that pps_no_pic_partition_flag shall be equal to 1. no_pic_partition_constraint_flag equal to 0 does not impose such constraint. When no_pic_partition_constraint_flag is equal to 1, the value of one_tile_per_pic_constraint_flag and one_slice_per_pic_constraint_flag shall be equal to 1.

A new GCI flag is proposed to constrain the rectangular slice partition as follows:

no_rectangular_slice_constraint_flag equal to 1 specifies that pps_rect_slice_flag shall be equal to 0, no_rectangular_slice_constraint_flag equal to 0 does not impose such a constraint.

A new GCI flag is proposed to constrain the raster scan slice partition as follows:

no_raster_scan_slice_constraint_flag equal to 1 specifies that pps_rect_slice_flag shall be equal to 1, no_raster_scan_slice_constraint_flag equal to 0 does not impose such a constraint. When one_tile_per_pic_constraint_flag  is 1 or one_subpic_per_pic_constraint_flag is 1, the value of no_raster_scan_slice_constraint_flag shall be equal to 1.

A new GCI flag is proposed to constrain single slice per subpicture as follows:

one_slice_per_subpicture_constraint_flag equal to 1 specifies that the value of pps_single_slice_per_subpic_flag shall be equal to 1, one_slice_per_subpicture_constraint_flag equal to 0 does not impose such a constraint.

A new GCI flag is proposed to constrain the VUI information presence as follows:

no_VUI_constraint_flag equal to 1 specifies sps_vui_parameters_present_flag is equal to 0, no_VUI_constraint_flag equal to 0 does not impose such a constraint.

A new GCI flag is proposed to constrain the omnidirectional video specific SEI messages including equirectangular projection, generalized cubemap projection, sphere rotation, region-wise packing and omnidirectional viewport SEI messages.

general_non_omnidirectional_video_constraint_flag equal to 1 specifies that there shall not be any omnidirectional video specific SEI messages present in the bitstream of the OlsInScpoe. general_non_omnidirectional_video_constraint_flag equal to 0 does not impose such a constraint.

A new GCI flag is proposed to constrain the DRAP indication SEI message as follows:

general_non_dependent_RAP_constraint_flag equal to 1 specifies that there shall not be any Dependent Random Access Point indication SEI message present in the bitstream of the OlsInScpoe. General_non_dependent_RAP_constraint_flag equal to 0 does not impose such a constraint.

A new GCI flag is proposed to constrain the HRD relevant SEI message as follows:

general_non_HRD_constraint_flag equal to 1 specifies that there shall not be any HRD related SEI messages (BP, PT, DU information SEI messages) present in the bitstream of the OlsInScpoe. general_non_HRD_constraint_flag equal to 0 does not impose such a constraint.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).