| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0120 |
| --- | --- |

| Title: | AHG9: Constraints on sh_picture_header_in_slice_header_flag |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Vadim Seregin, Yong He, Marta Karczewicz 5775 Morehouse Dr, San Diego, CA, 92121, USA | Tel: Email: | +1(858)651-3483 nanh@qti.qualcomm.com |
| Source: | Qualcomm Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to modify constraints on the syntax element sh_picture_header_in_slice_header_flag as follows:

- When pps_no_pic_partition_flag is equal to 1 and sps_subpic_info_present_flag is equal to 1, there is no need to constrain the value of sh_picture_header_in_slice_header_flag to be equal to 0.

- When pps_alf_info_in_ph_flag is equal to 1, the value of sh_picture_header_in_slice_header_flag shall be equal to 0.

- An alternative proposal of item 2 is to remove the constraint of sh_picture_header_in_slice_header_flag dependent on the value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag and pps_qp_delta_info_in_ph_flag.

# Proposal

When picture partitioning is not allowed, a PH syntax structure can be present in SH. Therefore, there is no need to constrain the value of sh_picture_header_in_slice_header_flag to be equal to 0 as there is only one slice in this case. In other words, the value of sh_picture_header_in_slice_header_flag shall be constrained to be equal to 0 only when picture partitioning is allowed.

In VVC draft 9, when information of tools is present in PH but not present in SH, sh_picture_header_in_slice_header_flag shall be equal to 0. For example, when the value of pps_sao_info_in_ph_flag is equal to 1, the value of sh_picture_header_in_slice_header_flag shall be equal to 0. However, when the value of pps_alf_info_in_ph_flag is equal to 1, there is no such constraint on the value of sh_picture_header_in_slice_header_flag.

When the value of pps_alf_info_in_ph_flag is equal to 1, it is proposed that the value of sh_picture_header_in_slice_header_flag shall be equal to 0.

Semantics changes are highlighted below.

When any of the following conditions is true, the value of sh_picture_header_in_slice_header_flag shall be equal to 0:

- The value of sps_subpic_info_present_flag is equal to 1 and pps_no_pic_partition_flag is equal to 0.

- The value of sps_separate_colour_plane_flag is equal to 1.

- The value of pps_rect_slice_flag is equal to 0

- The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, pps_alf_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1

An alternative to adding pps_alf_info_in_ph_flag as an additional condition is to remove the constraint dependent on the value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag and pps_qp_delta_info_in_ph_flag.

When there is one slice in a picture and the value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1, a PH cannot be present in an SH. However, there is no disadvantage to allow PH to be present in an SH. It is not necessary to have an additional bitstream conformance.

Semantics changes are highlighted below.

When any of the following conditions is true, the value of sh_picture_header_in_slice_header_flag shall be equal to 0:

- The value of sps_subpic_info_present_flag is equal to 1 and pps_no_pic_partition_flag is equal to 0.

- The value of sps_separate_colour_plane_flag is equal to 1.

- The value of pps_rect_slice_flag is equal to 0

- The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1.

# Patent rights declaration(s)

Qualcomm Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).