| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0124 |
| --- | --- |

| Title: | AHG9: On unavailable reference pictures |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Vadim Seregin, Yong He, Muhammed Coban, Adarsh K. Ramasubramonian, Marta Karczewicz | Tel: Email: | vseregin@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

In VVC, there is a decoding process for generating unavailable reference pictures when reference picture is equal to “no reference picture”. This contribution proposes:

- Add TemporalId, ph_non_reference_picture_flag, and parameter sets to the decoding process for generating unavailable reference pictures.

- Invoke decoding process for generating unavailable reference pictures for inter-layer reference picture or constrain that inter-layer reference picture shall not be equal to “no reference picture”.

- Remove the constraint that each inter-layer reference picture entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

# Proposal

## Aspect 1. TemporalId, ph_non_reference_picture_flag, and parameter set for “no reference pictures”

In the reference picture list derivation process, pictures not present in DPB are set equal to “no reference picture”. There are constraints imposed on reference pictures and to check such constraints for “no reference picture”, there is a process to generate unavailable reference pictures.

In that process, TemporalId, ph_non_reference_picture_flag, and parameter sets (scaling windows, sub-picture information) are not defined.

It is proposed to set TemporalId equal to the TemporalId of the current picture, set ph_non_reference_picture_flag equal to 0, and parameter sets are set equal to those of the current picture.

The proposed changes are highlighted in yellow below.

8.3.4.1	General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a CRA picture with NoOutputBeforeRecoveryFlag equal to 1 or a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

When this process is invoked, the following applies:

- For each 	RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

- Otherwise, when st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of ph_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

- The value of PicOutputFlag for the generated reference picture is set equal to 0.

- RefPicList[ i ][ j ] is set to be the generated reference picture.

- The value of TemporalId for the generated picture is set equal to TemporalId of the current picture.

- The value of ph_non_reference_picture_flag for the generated picture is set equal to 0.

- Parameter set information is set equal to those of the current picture.

## Aspect 2. Generating unavailable inter-layer reference pictures

In the reference picture list derivation process, pictures not present in DPB are set equal to “no reference picture”, this is also applied to inter-layer reference picture (ILRP) as shown in the next excerpt.

…

} else {
			layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
			refPicLayerId = vps_layer_id[ layerIdx ]
			if( there is a reference picture picA in the DPB with nuh_layer_id equal to refPicLayerId and
						the same PicOrderCntVal as the current picture )
				RefPicList[ i ][ j ] = picA
			else
				RefPicList[ i ][ j ] = "no reference picture"
		} 
…

There are constraints imposed on reference pictures and to check such constraints for “no reference picture”, there is a process to generate unavailable reference pictures, but ILRP is not included in this process.

Instead of adding ILRP to generating unavailable inter-layer reference pictures, it is proposed in method “a” to add a constraint as follows.

There shall be no entry equal to "no reference picture" for inter-layer reference picture.

In the method “b”, it is proposed to add decoding process for generating unavailable inter-layer reference pictures as follows, where modifications are provided on top of Aspect 1.

8.3.4.1	General decoding process for generating unavailable reference pictures

This process is invoked once per coded picture when the current picture is a CRA picture with NoOutputBeforeRecoveryFlag equal to 1 or a GDR picture with NoOutputBeforeRecoveryFlag equal to 1.

When this process is invoked, the following applies:

- For each RefPicList[ i ][ j ], with i in the range of 0 to 1, inclusive, and j in the range of 0 to num_ref_entries[ i ][ RplsIdx[ i ] ] − 1, inclusive, that is equal to "no reference picture", a picture is generated as specified in subclause 8.3.4.2 and the following applies:

- The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- If st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicPocList[ i ][ j ] and the generated picture is marked as "used for short-term reference".

The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- Otherwise, when st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0, the value of PicOrderCntVal for the generated picture is set equal to RefPicLtPocList[ i ][ j ], the value of ph_pic_order_cnt_lsb for the generated picture is inferred to be equal to ( RefPicLtPocList[ i ][ j ] & ( MaxPicOrderCntLsb – 1 ) ), and the generated picture is marked as "used for long-term reference".

The value of nuh_layer_id for the generated picture is set equal to nuh_layer_id of the current picture.

- Otherwise, when st_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 0 and inter_layer_ref_pic_flag[ i ][ RplsIdx[ i ] ][ j ] is equal to 1, the value of PicOrderCntVal for the generated picture is set equal to PicOrderCntVal of the current picture, and the generated picture is marked as "used for long-term reference".

The value of nuh_layer_id for the generated picture is set equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ] ].

- The value of PicOutputFlag for the generated reference picture is set equal to 0.

- RefPicList[ i ][ j ] is set to be the generated reference picture.

- The value of TemporalId for the generated picture is set equal to TemporalId of the current picture.

- The value of ph_non_reference_picture_flag for the generated picture is set equal to 0.

- Parameter set information is set equal to those of the current picture. nuh_layer_id of the PPS and SPS is set equal to vps_layer_id[ DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ] ] if the generated picture is ILRP.

## Aspect 3. Remove requirement that ILRP shall be an active entry

In the reference picture list derivation process, it is required that ILRP shall be an active entry.

- Each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

However, RPL can be signalled in a picture header or SPS, and then such common RPL may be used by all slices of a picture. ILRP picture may be added as a last entry in the RPL, as for example in SHVC CTC.

If then a slice specifies a shorter number of active entries by using override flag, then the common RPL can not be used since it violates the above constraint to have each ILRP as an active entry. In such scenario, another RPL needs to be signaled which introduces an extra overhead.

It is proposed to remove the above-mentioned constraint.

- Each ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice shall be an active entry.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).