| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0131 |
| --- | --- |

| Title: | AHG9: On general constraint information syntax |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hong-Jheng Jhu, Xiaoyu Xiu, Yi-Wen Chen, Tsung-Chuan Ma, Xianglin Wang | Email: | jhuhong-jheng@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following modifications to the syntax related to general constraint information in the VVC draft specification version 9:

- Add conformance constraints for existing control flags

- Constrain the inter prediction-related constraint flags to be equal to 1 when intra_only_constraint_flag is equal to 1 (i.e., when slice_type is I for all slices).

- Constrain the chroma-related constraint flags to be equal to 1 when max_chroma_format_constraint_idc is equal to 0 (i.e., when there are no chroma components).

- Constrain the subpicture-related constraint flags to be equal to 1 when one_subpic_per_pic_constraint_flag is equal to 1 (i.e., when there are no subpictures).

- Constrain the affine mode-related constraint flags to be equal to 1 when no_affine_motion_constraint_flag is equal to 1 (i.e., when there is no affine mode).

- Constrain the transform skip-related constraint flags to be equal to 1 when no_transform_skip_constraint_flag is equal to 1 (i.e., when there is no transform skip mode).

- Constrain the picture resampling-related constraint flags to be equal to 1 when no_ref_pic_resampling_constraint_flag is equal to 1 (i.e., when there is no picture resampling).

- Add new control flags

- Add general constraint flags for the coding tools.

# Proposal

In current general constraint syntax design, there are several bitstream conformance as follows:

- To constrain no_ref_wraparound_constraint_flag by intra_only_constraint_flag:

When intra_only_constraint_flag is equal to 1, the value of no_ref_wraparound_constraint_flag shall be equal to 1.

- To constrain no_cclm_constraint_flag by max_chroma_format_constraint_idc:

When max_chroma_format_constraint_idc is equal to 0, the value of no_cclm_constraint_flag shall be equal to 1.

Those constraint might be beneficial to avoid a product to set up a profile which causes confusion. However, several similar constraints are missing, more bitstream conformance shall be added into the general constraint flags to avoid the similar confusions. Furthermore, several functions are missing in the general constraint information syntax, those function flags should be added to provide the same general constraint controls as others. Therefore, this proposal provides following modifications to address the issue:

- Add conformance constraints for existing control flags

- Constrain the inter prediction-related constraint flags to be equal to 1 when intra_only_constraint_flag is equal to 1 (i.e., when slice_type is I for all slices).

- Constrain the chroma-related constraint flags to be equal to 1 when max_chroma_format_constraint_idc is equal to 0 (i.e., when there are no chroma components).

- Constrain the subpicture-related constraint flags to be equal to 1 when one_subpic_per_pic_constraint_flag is equal to 1 (i.e., when there are no subpictures).

- Constrain the affine mode-related constraint flags to be equal to 1 when no_affine_motion_constraint_flag is equal to 1 (i.e., when there is no affine mode).

- Constrain the transform skip-related constraint flags to be equal to 1 when no_transform_skip_constraint_flag is equal to 1 (i.e., when there is no transform skip mode).

- Constrain the picture resampling-related constraint flags to be equal to 1 when no_ref_pic_resampling_constraint_flag is equal to 1 (i.e., when there is no picture resampling).

- Add new control flags

- Add general constraint flags for the coding tools.

The changes to the VVC working draft are highlighted below. The added parts are highlighted in yellow.

## Constrain the inter prediction-related constraint flags

7.4.4.2	General constraint information semantics

no_sbt_constraint_flag equal to 1 specifies that sps_sbt_enabled_flag shall be equal to 0. no_sbt_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_sbt_constraint_flag shall be equal to 1.

no_ref_pic_resampling_constraint_flag equal to 1 specifies that sps_ref_pic_resampling_enabled_flag shall be equal to 0. no_ref_pic_resampling_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_ref_pic_resampling_constraint_flag shall be equal to 1.

## Constrain the chroma-related constraint flags

7.4.4.2	General constraint information semantics

no_act_constraint_flag equal to 1 specifies that sps_act_enabled_flag shall be equal to 0. no_act_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_act_constraint_flag shall be equal to 1.

no_chroma_qp_offset_constraint_flag equal to 1 specifies that pps_cu_chroma_qp_offset_list_enabled_flag shall be equal to 0. no_chroma_qp_offset_constraint_flag equal to 0 does not impose such a constraint. When max_chroma_format_constraint_idc is equal to 0, the value of no_chroma_qp_offset_constraint_flag shall be equal to 1.

## Constrain the subpicture-related constraint flags

7.4.4.2	General constraint information semantics

no_mixed_nalu_types_in_pic_constraint_flag equal to 1 specifies that it is a requirement of bitstream conformance that pps_mixed_nalu_types_in_pic_flag shall be equal to 0. no_mixed_nalu_types_in_pic_constraint_flag equal to 0 does not impose such a constraint. When one_subpic_per_pic_constraint_flag is equal to 1, the value of no_mixed_nalu_types_in_pic_constraint_flag shall be equal to 1.

## Constrain the affine mode-related constraint flags

7.4.4.2	General constraint information semantics

no_prof_constraint_flag equal to 1 specifies that sps_affine_prof_enabled_flag shall be equal to 0. no_prof_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_prof_constraint_flag shall be equal to 1. When no_affine_motion_constraint_flag is equal to 1, the value of no_prof_constraint_flag shall be equal to 1.

## Constrain the transform skip-related constraint flags

7.4.4.2	General constraint information semantics

no_bdpcm_constraint_flag equal to 1 specifies that sps_bdpcm_enabled_flag shall be equal to 0. no_bdpcm_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_bdpcm_constraint_flag shall be equal to 1.

## Constrain the picture resampling-related constraint flags

7.4.4.2	General constraint information semantics

no_res_change_in_clvs_constraint_flag equal to 1 specifies that sps_res_change_in_clvs_allowed_flag shall be equal to 0. no_res_change_in_clvs_constraint_flag equal to 0 does not impose such a constraint. When the value of no_ref_pic_resampling_constraint_flag is equal to one, the value of no_res_change_in_clvs_constraint_flag should be equal to one.

## Add general constraint flags for the coding tools

| general_constraint_info( ) { | Descriptor |
| --- | --- |
| no_conformance_window_constraint_flag | u(1) |
| no_weighted_pred _constraint_flag | u(1) |
| no_weighted_bipred_constraint_flag | u(1) |
| no_virtual_boundaries_constraint_flag | u(1) |

7.4.4.2	General constraint information semantics

no_conformance_window_constraint_flag equal to 1 specifies that sps_conformance_window_flag shall be equal to 0. no_conformance_window_constraint_flag equal to 0 does not impose such a constraint.

no_weighted_pred_constraint_flag equal to 1 specifies that sps_weighted_pred_flag shall be equal to 0. no_weighted_pred_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_weighted_pred_constraint_flag shall be equal to 1.

no_weighted_bipred_constraint_flag equal to 1 specifies that sps_weighted_bipred_flag shall be equal to 0. no_weighted_bipred_constraint_flag equal to 0 does not impose such a constraint. When intra_only_constraint_flag is equal to 1, the value of no_weighted_bipred_constraint_flag shall be equal to 1.

no_virtual_boundaries_constraint_flag equal to 1 specifies that sps_virtual_boundaries_enabled_flag shall be equal to 0. no_virtual_boundaries_constraint_flag equal to 0 does not impose such a constraint.

# Reference

B. Bross, J. Chen, S. Liu and Y.-K. Wang, “Versatile Video Coding (Draft 9)”, JVET-R2001-vA, the 18th JVET meeting, teleconference, 15–24 April 2020.

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).