| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0133 |
| --- | --- |

| Title: | AHG9: On partition constraints override in picture header |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yi-Wen Chen, Xiaoyu Xiu, Hong-Jheng Jhu, Tsung-Chuan Ma, Xianglin Wang | Email: | yiwenchen@kwai.com |
| Source: | Kwai Inc. |  |  |

_____________________________

# Abstract

When encoding one picture, the partition constrains are derived based on the partitioning used in the previously encoded pictures. In current VTM, when the derived partition constraints are identical to the values signaled in SPS, the derived values are still signaled in the PH to override the values signalled in the SPS which is obviously redundant. It is proposed to modify the VTM encoder to signal the override flag as zero when the derived partition constraints are identical to the ones in the SPS.

Moreover, it is observed that, the partition constraints signalled in the PH are usually similar to or identical to the ones signalled in the SPS. It is proposed, in the proposal, to signal the differences between the values of partitioning constraint in the PH and the corresponding values in SPS to have a more efficient override mechanism of partition constraints.

The results reportedly show that the encoder-only change to remove the redundant signalling introduces -0.12% (RA) and -0.06%(LD) BD-rate reduction for class D. On top of the encoder-only change, the proposed syntax modifications introduce -0.20%(RA), -0.19% (LD) BD-rate reduction for class D. In average, the encoder-only change introduces 0.00 (AI), -0.03% (RA) and -0.14%(LDB) BD-rate changes, and on top of the encoder-only change, the proposed syntax modification introduces 0.00(AI), -0.04% (RA) and -0.15%(LDB) BD-rate changes under CTC. It is noted that the results show no impact on AI because current VTM always disable the override of the partition constraints for intra pictures.

# Proposal

In current VTM, when encoding one picture, the partition constrains are derived based on the partitioning used in the previously encoded pictures. But in VTM-9.0, when the derived partition constraints are the same as the values signaled in the SPS, the derived values are still signaled in the picture header (PH) to override the values signalled in the SPS which is obviously redundant. In this proposal, the redundant overhead signalling is removed by signaling the override flag as zero when the derived partition constraints are the same as the ones in the SPS. This redundant overhead removal is integrated into the VTM encoder.

Moreover, the signaling mechanism of partition constraints override is not efficient when the override is activated frequently and some of the syntax elements are the same as the ones signalled in SPS. It is proposed, in the proposal, to signal the differences between the values of partitioning constraint in the PH and the values in SPS to have a more efficient signalling of partition constraints. The proposed changes to VVC are highlighted below. The added parts are highlighted as yellow and the deleted parts are highlighted as green.

| picture_header_structure( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_partition_constraints_override_enabled_flag ) |  |
| ph_partition_constraints_override_flag | u(1) |
| if( ph_intra_slice_allowed_flag ) { |  |
| if( ph_partition_constraints_override_flag ) { |  |
| ph_sps_log2_diff_min_qt_min_cb_intra_slice_luma | ue(v) |
| ph_sps_diff _max_mtt_hierarchy_depth_intra_slice_luma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_luma != 0 ) { |  |
| ph_sps_log2_diff_max_bt_min_qt_intra_slice_luma | ue(v) |
| ph_sps_log2_diff_max_tt_min_qt_intra_slice_luma | ue(v) |
| } |  |
| if( sps_qtbtt_dual_tree_intra_flag ) { |  |
| ph_sps_log2_diff_min_qt_min_cb_intra_slice_chroma | ue(v) |
| ph_sps_diff _max_mtt_hierarchy_depth_intra_slice_chroma | ue(v) |
| if( ph_max_mtt_hierarchy_depth_intra_slice_chroma != 0 ) { |  |
| ph_sps_log2_diff_max_bt_min_qt_intra_slice_chroma | ue(v) |
| ph_sps_log2_diff_max_tt_min_qt_intra_slice_chroma | ue(v) |
| } |  |
| } |  |
| } |  |
| … |  |
| } |  |
| if( ph_inter_slice_allowed_flag ) { |  |
| if( ph_partition_constraints_override_flag ) { |  |
| ph_sps_log2_diff_min_qt_min_cb_inter_slice | ue(v) |
| ph_sps_diff_max_mtt_hierarchy_depth_inter_slice | ue(v) |
| if( ph_max_mtt_hierarchy_depth_inter_slice != 0 ) { |  |
| ph_sps_log2_diff_max_bt_min_qt_inter_slice | ue(v) |
| ph_sps_log2_diff_max_tt_min_qt_inter_slice | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

The semantic of the modified syntax elements are elaborated below:

ph_sps_log2_diff_min_qt_min_cb_intra_slice_luma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum coding block size in luma samples for luma CUs in the slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding minimum size derived in the SPS. The value of ph_log2_diff_min_qt_min_cb_intra_slice_luma shall be in the range of 0 to 6– MinQtLog2SizeInterY Min( 6, CtbLog2SizeY ) − MinCbLog2SizeY, inclusive. When not present, the value of ph_sps_log2_diff_min_qt_min_cb_luma is inferred to be equal to 0sps_log2_diff_min_qt_min_cb_intra_slice_luma.

ph_sps_diff_max_mtt_hierarchy_depth_intra_slice_luma specifies the differences between the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding maximum hierarchy depth derived in the SPS.  The value of ph_sps_diff_max_mtt_hierarchy_depth_intra_slice_luma shall be in the range of 0 to 2*( CtbLog2SizeY − MinCbLog2SizeY ) - SPS_MaxMttDepthY, inclusive. When not present, the value of ph_sps_diff_max_mtt_hierarchy_depth_intra_slice_luma is inferred to be equal to 0sps_max_mtt_hierarchy_depth_intra_slice_luma.

ph_sps_log2_diff_max_bt_min_qt_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding maximum size derived in the SPS. The value of ph_sps_log2_diff_max_bt_min_qt_intra_slice_luma shall be in the range of 0 to ( sps_qtbtt_dual_tree_intra_flag ? Min( 6, CtbLog2SizeY ) : CtbLog2SizeY ) − MaxBtSizeYMinQtLog2SizeIntraY, inclusive. When not present, the value of ph_sps_log2_diff_max_bt_min_qt_intra_slice_luma is inferred to be equal to 0sps_log2_diff_max_bt_min_qt_intra_slice_luma.

ph_sps_log2_diff_max_tt_min_qt_intra_slice_luma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding maximum size derived in the SPS. The value of ph_sps_log2_diff_max_tt_min_qt_intra_slice_luma shall be in the range of 0 to Min( 6, CtbLog2SizeY ) − MaxTtSizeYMinQtLog2SizeIntraY, inclusive. When not present, the value of ph_sps_log2_diff_max_tt_min_qt_intra_slice_luma is inferred to be equal to 0sps_log2_diff_max_tt_min_qt_intra_slice_luma.

ph_sps_log2_diff_min_qt_min_cb_intra_slice_chroma specifies the difference between the base 2 logarithm of the minimum size in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA and the base 2 logarithm of the minimum coding block size in luma samples for chroma CUs with treeType equal to DUAL_TREE_CHROMA in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding minimum size derived in the SPS. The value of ph_sps_log2_diff_min_qt_min_cb_intra_slice_chroma shall be in the range of 0 to 6– MinQtLog2SizeInterY Min( 6, CtbLog2SizeY ) -MinCbLog2SizeY, inclusive. When not present, the value of ph_sps_log2_diff_min_qt_min_cb_intra_slice_chroma is inferred to be equal to 0sps_log2_diff_min_qt_min_cb_intra_slice_chroma.

ph_sps_diff _max_mtt_hierarchy_depth_intra_slice_chroma specifies the differences between the maximum hierarchy depth for chroma coding units resulting from multi-type tree splitting of a chroma quadtree leaf with treeType equal to DUAL_TREE_CHROMA in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding maximum hierarchy depth derived in the SPS. The value of ph_sps_diff_max_mtt_hierarchy_depth_intra_slice_chroma shall be in the range of 0 to 2*( CtbLog2SizeY − MinCbLog2SizeY ) - SPS_MaxMttDepthY, inclusive. When not present, the value of ph_sps_diff_max_mtt_hierarchy_depth_intra_slice_chroma is inferred to be equal to 0sps_max_mtt_hierarchy_depth_intra_slice_chroma.

ph_sps_log2_diff_max_bt_min_qt_intra_slice_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding maximum size derived in the SPS. The value of ph_sps_log2_diff_max_bt_min_qt_intra_slice_chroma shall be in the range of 0 to Min( 6, CtbLog2SizeY ) − MaxBtSizeCMinQtLog2SizeIntraC, inclusive. When not present, the value of ph_sps_log2_diff_max_bt_min_qt_intra_slice_chroma is inferred to be equal to 0sps_log2_diff_max_bt_min_qt_intra_slice_chroma.

ph_sps_log2_diff_max_tt_min_qt_intra_slice_chroma specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a chroma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a chroma leaf block resulting from quadtree splitting of a chroma CTU with treeType equal to DUAL_TREE_CHROMA in slices with sh_slice_type equal to 2 (I) associated with the PH and the corresponding maximum size derived in the SPS. The value of ph_sps_log2_diff_max_tt_min_qt_intra_slice_chroma shall be in the range of 0 to Min( 6, CtbLog2SizeY ) − MaxTtSizeCMinQtLog2SizeIntraC, inclusive. When not present, the value of ph_sps_log2_diff_max_tt_min_qt_intra_slice_chroma is inferred to be equal to 0sps_log2_diff_max_tt_min_qt_intra_slice_chroma

ph_sps_log2_diff_min_qt_min_cb_inter_slice specifies the difference between the base 2 logarithm of the minimum size in luma samples of a luma leaf block resulting from quadtree splitting of a CTU and the base 2 logarithm of the minimum luma coding block size in luma samples for luma CUs in the slices with sh_slice_type equal to 0 (B) or 1 (P) associated with the PH and the corresponding minimum size derived in the SPS. The value of ph_sps_log2_diff_min_qt_min_cb_inter_slice shall be in the range of 0 to 6– MinQtLog2SizeInterY Min( 6, CtbLog2SizeY ) − MinCbLog2SizeY, inclusive. When not present, the value of ph_sps_log2_diff_min_qt_min_cb_luma is inferred to be equal to 0sps_log2_diff_min_qt_min_cb_inter_slice.

ph_sps_diff_max_mtt_hierarchy_depth_inter_slice specifies the differences between the maximum hierarchy depth for coding units resulting from multi-type tree splitting of a quadtree leaf in slices with sh_slice_type equal to 0 (B) or 1 (P) associated with the PH and the corresponding maximum hierarchy depth derived in the SPS. The value of ph_sps_diff_max_mtt_hierarchy_depth_inter_slice shall be in the range of 0 to 2*( CtbLog2SizeY − MinCbLog2SizeY ) - SPS_MaxMttDepthY, inclusive. When not present, the value of ph_sps_diff_max_mtt_hierarchy_depth_inter_slice is inferred to be equal to 0sps_max_mtt_hierarchy_depth_inter_slice.

ph_sps_log2_diff_max_bt_min_qt_inter_slice specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a binary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in the slices with sh_slice_type equal to 0 (B) or 1 (P) associated with the PH and the corresponding maximum size derived in the SPS. The value of ph_sps_log2_diff_max_bt_min_qt_inter_slice shall be in the range of 0 to CtbLog2SizeY − MaxBtSizeYMinQtLog2SizeInterY, inclusive. When not present, the value of ph_sps_log2_diff_max_bt_min_qt_inter_slice is inferred to be equal to 0sps_log2_diff_max_bt_min_qt_inter_slice.

ph_sps_log2_diff_max_tt_min_qt_inter_slice specifies the difference between the base 2 logarithm of the maximum size (width or height) in luma samples of a luma coding block that can be split using a ternary split and the minimum size (width or height) in luma samples of a luma leaf block resulting from quadtree splitting of a CTU in slices with sh_slice_type equal to 0 (B) or 1 (P) associated with the PH and the corresponding maximum size derived in the SPS. The value of ph_sps_log2_diff_max_tt_min_qt_inter_slice shall be in the range of 0 to Min( 6, CtbLog2SizeY ) − MaxTtSizeYMinQtLog2SizeInterY, inclusive. When not present, the value of ph_sps_log2_diff_max_tt_min_qt_inter_slice is inferred to be equal to 0sps_log2_diff_max_tt_min_qt_inter_slice.

After the above syntax elements related to partition constraints are received, the variables MinQtLog2SizeY, MinQtLog2SizeC, MinQtSizeY, MinQtSizeC, MaxBtSizeY, MaxBtSizeC, MinBtSizeY, MaxTtSizeY, MaxTtSizeC, MinTtSizeY, MaxMttDepthY and MaxMttDepthC are derived as follows:

- If sh_slice_type equal to 2 (I), the following applies:

MinQtLog2SizeY = MinQtLog2SizeY_SPSMinCbLog2SizeY + ph_log2_diff_min_qt_min_cb_intra_slice_luma	(119)

MinQtLog2SizeC = MinQtLog2SizeC_SPSMinCbLog2SizeY + ph_log2_diff_min_qt_min_cb_intra_slice_chroma	(120)

MaxBtSizeY =  1  <<  ( MaxBtSizeY_SPS -MinQtLog2SizeY + ph_log2_diff_max_bt_min_qt_intra_slice_luma )	(121)

MaxBtSizeC = 1  <<  ( MaxBtSizeC_SPS -MinQtLog2SizeC + ph_log2_diff_max_bt_min_qt_intra_slice_chroma )	(122)

MaxTtSizeY = 1  <<  ( MaxTtSizeY_SPS -MinQtLog2SizeY + ph_log2_diff_max_tt_min_qt_intra_slice_luma )	(123)

MaxTtSizeC = 1  <<  ( MaxTtSizeC_SPS -MinQtLog2SizeC + ph_log2_diff_max_tt_min_qt_intra_slice_chroma )	(124)

MaxMttDepthY = MaxMttDepthY_SPS -ph_max_mtt_hierarchy_depth_intra_slice_luma	(125)

MaxMttDepthC =  MaxMttDepthC_SPS -ph_max_mtt_hierarchy_depth_intra_slice_chroma	(126)

CuQpDeltaSubdiv = ph_cu_qp_delta_subdiv_intra_slice	(127)

CuChromaQpOffsetSubdiv = ph_cu_chroma_qp_offset_subdiv_intra_slice	(128)

- Otherwise (sh_slice_type equal to 0 (B) or 1 (P)), the following applies:

MinQtLog2SizeY = MinQtLog2SizeY_SPSMinCbLog2SizeY + ph_log2_diff_min_qt_min_cb_inter_slice	(129)

MinQtLog2SizeC = MinQtLog2SizeC_SPSMinCbLog2SizeY + ph_log2_diff_min_qt_min_cb_inter_slice	(130)

MaxBtSizeY = 1  <<  ( MaxBtSizeY_SPS -MinQtLog2SizeY + ph_log2_diff_max_bt_min_qt_inter_slice)	(131)

MaxBtSizeC = 1  <<  ( MaxBtSizeC_SPS -MinQtLog2SizeC + ph_log2_diff_max_bt_min_qt_inter_slice)	(132)

MaxTtSizeY = 1  <<  ( MaxTtSizeY_SPS -MinQtLog2SizeY + ph_log2_diff_max_tt_min_qt_inter_slice)	(133)

MaxTtSizeC = 1  <<  ( MaxTtSizeC_SPS -MinQtLog2SizeC + ph_log2_diff_max_tt_min_qt_inter_slice)	(134)

MaxMttDepthY = MaxMttDepthY_SPS -ph_max_mtt_hierarchy_depth_inter_slice	(135)

MaxMttDepthC = MaxMttDepthC_SPS -ph_max_mtt_hierarchy_depth_inter_slice	(136)

CuQpDeltaSubdiv = ph_cu_qp_delta_subdiv_inter_slice	(137)

CuChromaQpOffsetSubdiv = ph_cu_chroma_qp_offset_subdiv_inter_slice	(138)

- The following applies:

MinQtSizeY = 1  <<  MinQtLog2SizeY		(139)

MinQtSizeC = 1  <<  MinQtLog2SizeC		(140)

MinBtSizeY = 1  <<  MinCbLog2SizeY		(141)

MinTtSizeY = 1  <<  MinCbLog2SizeY                                               (142)

# Simulation Results

Both proposed changes are implemented based on the VTM-8.0 reference software [1] and tested using common test condition defined in [2]. The results of the proposed encoder-only change are illustrated in the Table 1 while the results of the proposed syntax modifications together with the encoder-only change are illustrated in the Table 2.

Table 1 Results of the proposed encoder-only change

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class A2 | -0.01% | -0.02% | -0.02% | 100% | 101% |
| Class B | -0.03% | -0.04% | -0.04% | 101% | 100% |
| Class C | -0.04% | -0.05% | -0.04% | 101% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.03% | -0.03% | -0.03% | 101% | 101% |
| Class D | -0.12% | -0.13% | -0.13% | 100% | 100% |
| Class F | -0.07% | -0.08% | -0.08% | 101% | 101% |
|  | Low delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.03% | -0.04% | -0.04% | 100% | 100% |
| Class C | -0.03% | -0.03% | -0.03% | 100% | 100% |
| Class E | -0.46% | -0.48% | -0.44% | 100% | 99% |
| Overall | -0.14% | -0.14% | -0.13% | 100% | 100% |
| Class D | -0.06% | -0.06% | -0.06% | 100% | 100% |
| Class F | -0.18% | -0.18% | -0.18% | 100% | 100% |

Table 2 Results of the proposed encoder-only change and the proposed syntax modifications

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | -0.02% | -0.02% | 100% | 100% |
| Class A2 | -0.02% | -0.02% | -0.02% | 99% | 100% |
| Class B | -0.03% | -0.04% | -0.04% | 101% | 99% |
| Class C | -0.07% | -0.07% | -0.07% | 102% | 101% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | -0.04% | -0.04% | 100% | 100% |
| Class D | -0.20% | -0.21% | -0.21% | 100% | 99% |
| Class F | -0.08% | -0.09% | -0.09% | 101% | 99% |
|  | Low delay B |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.04% | -0.04% | -0.04% | 99% | 100% |
| Class C | -0.07% | -0.07% | -0.07% | 100% | 100% |
| Class E | -0.46% | -0.48% | -0.44% | 101% | 100% |
| Overall | -0.15% | -0.16% | -0.15% | 100% | 100% |
| Class D | -0.19% | -0.19% | -0.19% | 101% | 100% |
| Class F | -0.20% | -0.21% | -0.20% | 100% | 100% |

# Reference

VTM-8.0, https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/tags/VTM-8.0.

A. Segall, E. François, W. Husak, S. Iwamura, D. Rusanovskyy, “ JVET common test conditions and evaluation procedures for HDR/WCG video”, JVET-R2011, 17th meeting, Teleconference, 15–24 April 2020

# Patent rights declaration(s)

Kwai Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).