| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0134-r1 |
| --- | --- |

| Title: | AHG9/AHG12: Relaxing an ALF APS constraint |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Alireza Aminlou , Miska M. Hannuksela Nokia Technologies, Tampere, Finland | Tel: Email: | firstname.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

VVC Draft 9 includes a constraint that the content of all APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a picture unit shall have the same content. Furthermore, number of ALF APSs is limited to 8. This contribution asserts that, because of this constraint, ALF needs to be turned off in many applications using adaptive subpicture-based delivery or decoding. To enable the use of ALF in such applications, the contribution proposes the following limitations in a specific case indicated by an SPS flag:

- Limit the number of ALF APSs to one for each slice.

- Limit the number of luma and chroma ALF filters to one for each ALF APS.

- Disable CCALF.

- Allow maximum of 115 ALF APSs

# Introduction / Problem Statement

Either of the following encoding strategies are possible for use cases involving subpicture extraction and merging:

- Encoding several versions of the content for different bitrates and/or resolutions, each version with multiple subpictures. Extraction of the coded subpicture sequences from the bitstreams for selective delivery and/or decoding.

- Splitting the uncompressed video into subpicture sequences before encoding, and encoding several versions of each subpicture sequence, each for different bitrates and/or resolutions. This approach has been discussed in more details in JVET-R0148.

In both encoding approaches, the content of ALF APSs is selected adaptively based on the content and it is typically not possible to avoid using the same ALF APS ID values in different bitstreams due to the limited value range (0..7). Consequently, to enable merging of subpictures originating from different bitstreams, ALF needs to be turned off, which may cause a remarkable loss in rate-distortion performance. For example, an average loss of 3.57% in YUV BD-bitrate was observed in the experiments for 360° video coded with CTC QP, discussed in Section ‎3. According to JVET-R0149, this average YUV loss is higher (i.e. 4.10%) for high quality encoding (QP:20, 23, 26, 29) which is more desirable for 360° video streaming.

# Proposal

The proposal is that in a special case, which is indicated by a flag in SPS, apply the following limitation and relations

- Only one ALF APS per slice

- Only one luma and chroma ALF filter per ALF APS

- Disable CCALF

- Allow maximum of 115 ALF APSs

The idea behind the above proposal is to guarantee the maximum memory requirement for storing ALF APSs and ALF filters. In the current VVC, there can be up to 8 ALF APSs, and each ALF APS can have up to 25 luma filters and 8 chroma filters. In a practical design memory is reserved to store 8 ALF APS. In test case of this proposal, encoding is constrained so that each slice can have only one ALF APS. So for the implementation purpose, one of those 8 ALF APS memory is reserved to storage the current/active ALF APS for the current slice. Then the remaining memory (i.e. 7 ALF APSs) can be used to store other ALF APSs. Since there is only one filter for each ALF APS according to this proposal, ALF filter of each ALF APS can be stored in the remaining memory without needing any additional data to be stored. For example, ALF filters can be stored at specific address of the memory where the address is calculated using ALF APS ID. For example, the memory address can be calculated as ALF_APS_ID+25 (+25 is the memory reserve for that one current/active ALF APS). The remaining memory is enough to store 175 (luma storage) + 56 (chroma storage) = 231 ALF filters. Chroma ALF filter has less coefficients that luma filters, so a part of luma storage can be used to store chroma filters. In this proposal, the number of ALF APSs is limited to 231/2=115.

# Spec text changes

The proposed changes are as below (removed and added part are highlighted):

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| if( sps_alf_enabled_flag ) |  |
| sps_one_aps_per_slice_one_filter_per_aps_flag | u(1) |

| adaptation_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| aps_adaptation_parameter_set_id | u(5) |
| aps_params_type | u(3) |
| aps_adaptation_parameter_set_id | u(v) |

| slice_header( ) { | Descriptor |
| --- | --- |
| for( i = 0; i < sh_num_alf_aps_ids_luma; i++ ) |  |
| sh_alf_aps_id_luma[ i ] | u(3v) |
| if( sh_alf_cb_flag ¦ ¦ sh_alf_cr_flag ) |  |
| sh_alf_aps_id_chroma | u(3v) |

sps_one_aps_per_slice_one_filter_per_aps_flag equal to 1 specifies that each ALF APS contains up to 1 luma filter, up to 1 chroma filter, and no cross-component filters, and each slice in the CLVS uses 0 or 1 ALF APSs for luma adaptive loop filtering, 0 or 1 ALF APSs for chroma adaptive loop filtering, and applies no cross-component adaptive loop filtering. sps_one_aps_per_slice_one_filter_per_aps_flag equal to 0 does not impose such constraints. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1 for any picture in a CVS, it shall be equal to 1 for all pictures in the CVS.

aps_adaptation_parameter_set_id provides an identifier for the APS for reference by other syntax elements.

When aps_params_type is equal to ALF_APS and sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 0, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive. When aps_params_type is equal to ALF_APS and sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1, the value of aps_adaptation_parameter_set_id shal be in the range of 0 to 114, inclusive. When aps_params_type is equal to ALF_APS, the length of the syntax element aps_adaptation_set_id is 7 bits.

When aps_params_type is equal to ALF_APS or SCALING_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 7, inclusive, and the length of the syntax element aps_adaptation_set_id is 3 bits.

When aps_params_type is equal to LMCS_APS, the value of aps_adaptation_parameter_set_id shall be in the range of 0 to 3, inclusive, and the length of the syntax element aps_adaptation_set_id is 2 bits.

alf_cc_cb_filter_signal_flag equal to 1 specifies that cross-component filters for the Cb colour component are signalled. alf_cc_cb_filter_signal_flag equal to 0 specifies that cross-component filters for Cb colour component are not signalled. When not present, the value of alf_cc_cb_filter_signal_flag is inferred to be equal to 0. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1, alf_cc_cb_filter_signal_flag shall be equal to 0.

alf_cc_cr_filter_signal_flag equal to 1 specifies that cross-component filters for the Cr colour component are signalled. alf_cc_cr_filter_signal_flag equal to 0 specifies that cross-component filters for the Cr colour component are not signalled. When not present, the value of alf_cc_cr_filter_signal_flag is inferred to be equal to 0. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1, alf_cc_cr_filter_signal_flag shall be equal to 0.

alf_luma_num_filters_signalled_minus1 plus 1 specifies the number of adpative loop filter classes for which luma coefficients can be signalled. The value of alf_luma_num_filters_signalled_minus1 shall be in the range of 0 to NumAlfFilters − 1, inclusive. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1 and alf_luma_num_filters_signalled_minus1 is present, the value of alf_luma_num_filters_signalled_minus1 shall be equal to 0.

alf_chroma_num_alt_filters_minus1 plus 1 specifies the number of alternative filters for chroma components. The value of alf_chroma_num_alt_filters_minus1 shall be in the range of 0 to 7, inclusive. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1 and alf_chroma_num_alt_filters_minus1 is present, the value of alf_chroma_num_alt_filters_minus1 shall be equal to 0.

sh_num_alf_aps_ids_luma specifies the number of ALF APSs that the slice refers to. When sh_alf_enabled_flag is equal to 1 and sh_num_alf_aps_ids_luma is not present, the value of sh_num_alf_aps_ids_luma is inferred to be equal to the value of ph_num_alf_aps_ids_luma. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1 and sh_num_alf_aps_ids_luma is present, the value of sh_num_alf_aps_ids_luma shall be equal to 0 or 1.

sh_alf_aps_id_luma[ i ] specifies the aps_adaptation_parameter_set_id of the i-th ALF APS that the luma component of the slice refers to. When sh_alf_enabled_flag is equal to 1 and sh_alf_aps_id_luma[ i ] is not present, the value of sh_alf_aps_id_luma[ i ] is inferred to be equal to the value of ph_alf_aps_id_luma[ i ]. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 0, the length of the syntax element sh_alf_aps_id_luma[ i ] is 3 bits. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1, the length of the syntax element sh_alf_aps_id_luma[ i ] is 7 bits.

sh_alf_aps_id_chroma specifies the aps_adaptation_parameter_set_id of the ALF APS that the chroma component of the slice refers to. When sh_alf_enabled_flag is equal to 1 and sh_alf_aps_id_chroma is not present, the value of sh_alf_aps_id_chroma is inferred to be equal to the value of ph_alf_aps_id_chroma. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 0, the length of the syntax element sh_alf_aps_id_chroma is 3 bits. When sps_one_aps_per_slice_one_filter_per_aps_flag is equal to 1, the length of the syntax element sh_alf_aps_id_chroma is 7 bits.

# Experimental results

Simulations were carried out using 300 frames for each of 360° video test sequences listed in Table 1. As described in JVET-M0870 ‎[2]‎[1], the 6K and 8K 360° video test sequences, defined in ‎[3], were first converted to the CMP format with a face size of 1536x1536 and 2048x2048, respectively, using JVET 360Lib-10.0 software ‎[4].

Table 1. Test sequences used in this study

| Class | Sequence name | ORG ERP Resolution@FPS | CMP Resolutions | Frame count | Bit depth |
| --- | --- | --- | --- | --- | --- |
| S1 | SkateboardInLot | 8192×4096@30 | 6144×4096 | 300 | 10 |
| S1 | ChairLift | 8192×4096@30 | 6144×4096 | 300 | 10 |
| S1 | KiteFlite | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | Harbor | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | Trolley | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | GasLamp | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S1 | Landing | 8192×4096@30 | 6144×4096 | 300 | 8 |
| S2 | Balboa | 6144×3072@60 | 4608×3072 | 300 | 8 |
| S2 | Broadway | 6144×3072@60 | 4608×3072 | 300 | 8 |
| S2 | Landing2 | 6144×3072@60 | 4608×3072 | 300 | 8 |
| S2 | BranCastle2 | 6144×3072@60 | 4608×3072 | 300 | 8 |

Original CMP was then partitioned to individual subpictures of size 512×512 before encoding and were encoded individually. In other words, each 1536×1536 cube face of a 6K sequence was split to 3×3 subpictures, while the 2048×2048 cube faces of 8K sequences were split to 4×4 subpictures, resulting in 54 and 96 individual subpictures, respectively. The subpicture sequences were then coded with VTM 9.0 ‎[1], using random access (RA) configuration. LMCS in SPS, scaling list in SPS, SAO and Joint Chroma sign flag were disabled for all test cases. This subpicture encoding approach has been discussed in more details and found valid in JVET-R0148.

Total bitrate for each test sequence was calculated as sum of the subpicture bitrates. To calculate the total PSNR, first MSE of each picture is calculated by taking the average of MSE of different subpicture, and then PSNR of sequence is calculated by taking the average of PSNR different pictures. YUV BDrate is calculated as weighted average of (Y, U, V) BDrates by weights of (8, 1, 1), respectively.

Different test cases were examined as define in Table 2, where number of ALF APSs, luma filters and chroma filters were limited to be 1 at the encoder side. Summary of results for different test cases are provided in Table 3, and details of the results are available in the contribution package.

Table 2. Different test cases (based on VTM9.0)

| Test case | Max # of ALF_APS/SubPic | Max # of luma filters | Max # of ch filters | Max # of cc filters | Decodable by VTM9.0 | Decodable by VTM9.0+ |
| --- | --- | --- | --- | --- | --- | --- |
| ALF_off (Anchor) | 0 | 0 | 0 | 0 | Yes | Yes |
| ALF_on | 8 | 25 | 8 | 4 | No | No |
| L1_Ch0_CC0 | 1 | 1 | 0 | 0 | No | Yes |
| L1_Ch1_CC0 | 1 | 1 | 1 | 0 | No | Yes |

VTM9.0+: VTM9.0 + proposed method

Table 3. BDRate results for different test cases in VTM9.0 (Anchor: ALF_off)

|  | ALF_on |  |  |  | L1_Ch0_CC0 |  |  |  | L1_Ch1_CC0 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | YUV | Y | U | V | YUV | Y | U | V | YUV | Y | U | V |
| skateboardInLot | -3.21 % | -2.78 % | -5.03 % | -4.80 % | -2.05 % | -2.72 % | 0.62 % | 0.59 % | -3.03 % | -2.62 % | -4.93 % | -4.42 % |
| ChairliftRide | -2.32 % | -1.95 % | -4.69 % | -2.92 % | -1.44 % | -1.99 % | 0.66 % | 0.81 % | -2.18 % | -1.89 % | -3.98 % | -2.77 % |
| KiteFlite | -3.33 % | -3.28 % | -3.58 % | -3.48 % | -2.49 % | -3.26 % | 0.57 % | 0.62 % | -3.25 % | -3.17 % | -3.63 % | -3.54 % |
| Harbor | -2.46 % | -2.53 % | -2.30 % | -2.08 % | -1.90 % | -2.53 % | 0.69 % | 0.58 % | -2.41 % | -2.44 % | -2.38 % | -2.16 % |
| Trolley | -2.63 % | -2.84 % | -2.26 % | -1.24 % | -2.18 % | -2.84 % | 0.49 % | 0.41 % | -2.60 % | -2.77 % | -2.40 % | -1.41 % |
| Gaslamp | -2.39 % | -2.38 % | -2.57 % | -2.31 % | -1.78 % | -2.46 % | 0.93 % | 0.96 % | -2.42 % | -2.36 % | -2.76 % | -2.51 % |
| Landing | -3.88 % | -3.56 % | -5.09 % | -5.19 % | -2.69 % | -3.44 % | 0.29 % | 0.39 % | -3.65 % | -3.36 % | -4.90 % | -4.73 % |
| Balboa | -3.69 % | -3.87 % | -3.19 % | -2.78 % | -2.93 % | -3.74 % | 0.35 % | 0.24 % | -3.51 % | -3.70 % | -3.18 % | -2.32 % |
| Broadway | -5.07 % | -5.41 % | -3.53 % | -3.87 % | -3.97 % | -5.11 % | 0.53 % | 0.62 % | -4.58 % | -5.05 % | -3.23 % | -2.14 % |
| Landing2 | -4.89 % | -4.51 % | -6.35 % | -6.47 % | -3.48 % | -4.38 % | -0.06 % | 0.25 % | -4.57 % | -4.29 % | -5.61 % | -5.77 % |
| BranCastle2 | -5.39 % | -5.05 % | -7.10 % | -6.37 % | -3.86 % | -4.81 % | -0.02 % | -0.12 % | -4.83 % | -4.76 % | -4.88 % | -5.34 % |
| Average | -3.57 % | -3.47 % | -4.15 % | -3.78 % | -2.62 % | -3.39 % | 0.46 % | 0.49 % | -3.37 % | -3.31 % | -3.81 % | -3.37 % |
|  | ALF_on |  |  |  | L1_Ch0_CC0 |  |  |  | L1_Ch1_CC0 |  |  |  |
|  | YUV | Y | U | V | YUV | Y | U | V | YUV | Y | U | V |
| skateboardInLot |  | -3.22 % |  | -2.82 % |  | -5.08 % |  | -4.49 % |  | -2.09 % |  | -2.71 % |  | 0.36 % |  | 0.39 % |  | -2.97 % |  | -2.60 % |  | -4.87 % |  | -3.96 % |  |
| ChairliftRide |  | -2.27 % |  | -1.96 % |  | -4.33 % |  | -2.67 % |  | -1.40 % |  | -1.94 % |  | 0.69 % |  | 0.79 % |  | -2.10 % |  | -1.86 % |  | -3.70 % |  | -2.46 % |  |
| KiteFlite |  | -3.30 % |  | -3.31 % |  | -3.29 % |  | -3.25 % |  | -2.48 % |  | -3.25 % |  | 0.55 % |  | 0.58 % |  | -3.19 % |  | -3.16 % |  | -3.33 % |  | -3.24 % |  |
| Harbor |  | -2.51 % |  | -2.62 % |  | -2.06 % |  | -2.05 % |  | -1.92 % |  | -2.53 % |  | 0.54 % |  | 0.51 % |  | -2.37 % |  | -2.46 % |  | -2.11 % |  | -1.99 % |  |
| Trolley |  | -2.64 % |  | -2.90 % |  | -2.05 % |  | -1.12 % |  | -2.20 % |  | -2.84 % |  | 0.44 % |  | 0.28 % |  | -2.56 % |  | -2.79 % |  | -2.10 % |  | -1.18 % |  |
| Gaslamp |  | -2.52 % |  | -2.54 % |  | -2.61 % |  | -2.25 % |  | -1.88 % |  | -2.55 % |  | 0.76 % |  | 0.81 % |  | -2.47 % |  | -2.46 % |  | -2.69 % |  | -2.33 % |  |
| Landing |  | -3.65 % |  | -3.92 % |  | -2.87 % |  | -2.29 % |  | -2.93 % |  | -3.73 % |  | 0.24 % |  | 0.29 % |  | -3.39 % |  | -3.68 % |  | -2.76 % |  | -1.74 % |  |
| Balboa |  | -5.03 % |  | -5.38 % |  | -3.34 % |  | -3.92 % |  | -3.96 % |  | -5.01 % |  | 0.17 % |  | 0.29 % |  | -4.51 % |  | -4.97 % |  | -3.14 % |  | -2.19 % |  |
| Broadway |  | -4.84 % |  | -4.46 % |  | -6.33 % |  | -6.41 % |  | -3.39 % |  | -4.29 % |  | 0.20 % |  | 0.20 % |  | -4.47 % |  | -4.23 % |  | -5.37 % |  | -5.50 % |  |
| Landing2 |  | -5.34 % |  | -5.01 % |  | -7.10 % |  | -6.15 % |  | -3.82 % |  | -4.76 % |  | -0.01 % |  | -0.13 % |  | -4.76 % |  | -4.71 % |  | -4.86 % |  | -5.12 % |  |
| BranCastle2 |  | -3.82 % |  | -3.50 % |  | -5.10 % |  | -5.09 % |  | -2.61 % |  | -3.35 % |  | 0.27 % |  | 0.40 % |  | -3.55 % |  | -3.28 % |  | -4.76 % |  | -4.50 % |  |
| Average |  | -3.56 % |  | -3.49 % |  | -4.01 % |  | -3.61 % |  | -2.61 % |  | -3.36 % |  | 0.38 % |  | 0.40 % |  | -3.30 % |  | -3.29 % |  | -3.61 % |  | -3.11 % |  |

# Software implementation in VTM9.0

# The changes in VTM9.0 (removed codes and added codes) are reported below. Please note that the changes at encoder side have been made for the case that each subpicture is encoded separately. After encoding each subpicture, required subpicture may be merged into a bitstream using the merge tool.

## Syntax change

## Slice.h

#if SubPicRD_ALF_Syntax

bool              m_oneAPSPerSliceOneFilterPerAPS;

...

bool                    getOneAPSPerSliceOneFilterPerAPS() const                                        { return m_oneAPSPerSliceOneFilterPerAPS; }

void                    setOneAPSPerSliceOneFilterPerAPS(bool b)                                        { m_oneAPSPerSliceOneFilterPerAPS = b; }

#endif

...

}

### VLCReader.cpp

void HLSyntaxReader::parseAPS( APS* aps )

{

#if ENABLE_TRACING

xTraceAPSHeader();

#endif

#if !SubPicRD_ALF_Syntax

READ_CODE(5, code, "adaptation_parameter_set_id");

aps->setAPSId(code);

#endif

READ_CODE(3, code, "aps_params_type");

aps->setAPSType( ApsType(code) );

#if SubPicRD_ALF_Syntax

uint32_t code2 = 0;

if (aps->getAPSType() == ALF_APS)

{

READ_CODE(7, code2, "adaptation_parameter_set_id");

}

else if (aps->getAPSType() == LMCS_APS)

{

READ_CODE(2, code2, "adaptation_parameter_set_id");

}

else if (aps->getAPSType() == SCALING_LIST_APS)

{

READ_CODE(3, code2, "adaptation_parameter_set_id");

}

aps->setAPSId(code2);

#endif

...

}

void HLSyntaxReader::parseSPS(SPS* pcSPS)

{

...

#if SubPicRD_ALF_Syntax

if (pcSPS->getALFEnabledFlag())

{

READ_FLAG(uiCode, "sps_one_aps_per_slice_one_filter_per_aps_flag");                      pcSPS->setOneAPSPerSliceOneFilterPerAPS(uiCode ? true : false);

}

else

{

pcSPS->setOneAPSPerSliceOneFilterPerAPS(false);

}

#endif

...

}

void HLSyntaxReader::parseSliceHeader (Slice* pcSlice, PicHeader* picHeader, ParameterSetManager *parameterSetManager, const int prevTid0POC)

{

...

for (int i = 0; i < numAps; i++)

{

#if SubPicRD_ALF_Syntax

if (sps->getOneAPSPerSliceOneFilterPerAPS())

READ_CODE(7, uiCode, "slice_alf_aps_id_luma");

else

#endif

READ_CODE(3, uiCode, "slice_alf_aps_id_luma");

apsId[i] = uiCode;

...

...

#if JVET_R0225_SEPERATE_FLAGS_ALF_CHROMA

if (alfCbEnabledFlag || alfCrEnabledFlag)

#else

if (alfChromaIdc)

#endif

{

#if SubPicRD_ALF_Syntax

if (sps->getOneAPSPerSliceOneFilterPerAPS())

READ_CODE(7, uiCode, "slice_alf_aps_id_chroma");

else

#endif

READ_CODE(3, uiCode, "slice_alf_aps_id_chroma");

pcSlice->setTileGroupApsIdChroma(uiCode);

...

}

### VLCWriter.cpp

void HLSWriter::codeAPS( APS* pcAPS )

{

#if ENABLE_TRACING

xTraceAPSHeader();

#endif

#if !SubPicRD_ALF_Syntax

WRITE_CODE(pcAPS->getAPSId(), 5, "adaptation_parameter_set_id");

#endif

WRITE_CODE( (int)pcAPS->getAPSType(), 3, "aps_params_type" );

#if SubPicRD_ALF_Syntax

if (pcAPS->getAPSType() == ALF_APS)

{

WRITE_CODE(pcAPS->getAPSId(), 7, "adaptation_parameter_set_id");

}

else if (pcAPS->getAPSType() == LMCS_APS)

{

WRITE_CODE(pcAPS->getAPSId(), 2, "adaptation_parameter_set_id");

}

else if (pcAPS->getAPSType() == SCALING_LIST_APS)

{

WRITE_CODE(pcAPS->getAPSId(), 3, "adaptation_parameter_set_id");

}

#endif

void HLSWriter::codeSPS( const SPS* pcSPS )

{

...

#if SubPicRD_ALF_Syntax

if (pcSPS->getALFEnabledFlag())

{

WRITE_FLAG(pcSPS->getOneAPSPerSliceOneFilterPerAPS(), "sps_one_aps_per_slice_one_filter_per_aps_flag");

}

#endif

...

}

void HLSWriter::codeSliceHeader         ( Slice* pcSlice )

{

...

for (int i = 0; i < pcSlice->getTileGroupNumAps(); i++)

{

#if SubPicRD_ALF_Syntax

if (pcSlice->getSPS()->getOneAPSPerSliceOneFilterPerAPS())

WRITE_CODE(apsId[i], 7, "slice_alf_aps_id_luma");

else

#endif

WRITE_CODE(apsId[i], 3, "slice_alf_aps_id_luma");

}

...

...

if (alfChromaIdc)

{

#if SubPicRD_ALF_Syntax

if (pcSlice->getSPS()->getOneAPSPerSliceOneFilterPerAPS())

WRITE_CODE(pcSlice->getTileGroupApsIdChroma(), 7, "slice_alf_aps_id_chroma");

else

#endif

WRITE_CODE(pcSlice->getTileGroupApsIdChroma(), 3, "slice_alf_aps_id_chroma");

}

...

}

## Encoder side and configuration change

## Restriction of number of ALF APS

To restrict the number of ALF_APS, “ALF_CTB_MAX_NUM_APS” is set to 2 (original value in VTM is 8). For some reason (small bug in encoder sude), the number of maximum number ALF APS is “ALF_CTB_MAX_NUM_APS-1”. This means when it is set to 2, maximum number of ALF_APS for each subpicture is 1.

static const int ALF_CTB_MAX_NUM_APS = 2;

## Restriction of number of Luma filters in each ALF APS

To restrict number of luma ALF filter in each ALF APS, the following change has been made in VTM9.0.

double EncAdaptiveLoopFilter::mergeFiltersAndCost( AlfParam& alfParam, AlfFilterShape& alfShape, AlfCovariance* covFrame, AlfCovariance* covMerged, int clipMerged[MAX_NUM_ALF_CLASSES][MAX_NUM_ALF_CLASSES][MAX_NUM_ALF_LUMA_COEFF], int& uiCoeffBits )

{

int numFiltersBest = 0;

#if SubPicRD_ALF_APS_filters

int numFilters = SubPicRD_ALF_APS_filters;

#else

int numFilters = MAX_NUM_ALF_CLASSES;

#endif

### Restriction on chroma and cross component ALF

The number of chroma alternative filter is restricted to one for each subpicture using and cross component has been disabled using the following parameters in the configuration file. These changes are applied only to proposed method.

MaxNumAlfAlternativesChroma : 1

CCALF : 0

# References

- VTM9.0, reference software for VVC. [Online]. Available: https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/tags/VTM-9.0

- M. Coban, R. Skupin, “AHG12: Proposed JVET common test conditions and evaluation procedures for MCTS and sub-pictures with boundary padding,” JVET-M0870, Marrakech, MA, Jan. 2019.

- P. Hanhart, J. Boyce, K. Choi, J.-L. Lin, “JVET common test conditions and evaluation procedures for 360°,” JVET-L1012, Oct. 2018.

- 360Lib-10.0. [Online]. Available: https://jvet.hhi.fraunhofer.de/svn/svn_360Lib/tags/360Lib-10.0/

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).