| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0140-v2 |
| --- | --- |

| Title: | AHG9: A summary of proposals on reference picture lists |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | yekui.wang@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 9 proposals on reference picture lists cleanups (the agenda item 3.1.6 in JVET-S0137 and JVET-S0237).

It is suggested that this summary, in terms of a list of proposed items, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposed items

- Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], such that it (S0045)

- swaps the meaning of positive and negative (consequently swaps the sign assignment of DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] in Equation 155, and change the subtraction to addition when assigning RefPicPocList[ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] in Equation 205.

- associates strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] with DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] instead of with an "i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )".

- When sps_inter_layer_ref_pics_present_flag is equal to 1, sps_idr_rpl_present_flag is skipped and inferred to be equal to 1. (S0049 aspect 2)

- Add a constraint: When vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_idr_rpl_present_flag to be equal to 1. (S0049 aspect 2)

- Remove sps_idr_rpl_present_flag (i.e., always signal RPLs for IDR slices, like for non-IDR I slices), and invoke generating unavailable reference picture for IDR pictures: (S0123 approach 1)

- As an alternative to 4), do the following and invoke generating unavailable reference picture for IDR pictures: (S0123 approach 2)

  - Do one of the following:

    - Change the SH syntax to signal RPL for each IDR slice in a non-independent layer.

    - Require sps_idr_rpl_present_flag or pps_rpl_info_in_ph_flag to be equal to 1 for an IDR slice when the current layer is a non-independent layer.

    - Require sh_slice_type to be equal to 2 for IDR slice when sps_idr_rpl_present_flag and pps_rpl_info_in_ph_flag are both equal to 0.

  - Change the process for deriving empty RPLs when sps_idr_rpl_present_flag is equal to 0 and nal_unit_type is equal to IDR_W_RADL or IDR_N_LP to involve pps_rpl_info_in_ph_flag.

- Add sps_ref_layer_idx_plus1 under the condition "if( sps_inter_layer_ref_pics_present_flag )" to enable skipping ilrp_idx[ listIdx ][ rplsIdx ][ i ] in the RPL syntax structures when the layer has only one direct reference layer, as follows: (S0082)

  - The value of ps_ref_layer_idx_plus1 shall be equal to

( NumDirectRefLayers[ GeneralLayerIdx[ nuh_layer_id ] ] > 1 ) ? 0:
		DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ 0 ] + 1

  - Consequently, when sps_ref_layer_idx_plus1 is greater than 0, ilrp_idx[ listIdx ][ rplsIdx ][ i ] is skipped and inferred to be equal to sps_ref_layer_idx_plus1 − 1.

- Change ilrp_idx[ listIdx ][ rplsIdx ][ i ] to delta_ilrp_idx_minus1[ i ][ RplsIdx ][ i ] as well as the semantics such that the following part of Equation 205 (S0083):

layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrp_idx[ i ][ RplsIdx ][ j ] ]
			refPicLayerId = vps_layer_id[ layerIdx ]

is changed to be as follows:

ilrpIdx = GeneralLayerIdx[ nuh_layer_id ] − ( delta_ilrp_idx_minus1[ i ][ RplsIdx ][ j ] + 1 )
			layerIdx = DirectRefLayerIdx[ GeneralLayerIdx[ nuh_layer_id ] ][ ilrpIdx ]
			refPicLayerId = vps_layer_id[ layerIdx ]

- Fix an asserted bug (when sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag is equal to 0, and rpl_idx[ 1 ] is less than sps_num_ref_pic_lists[ 0 ], the value of rpl_idx[ 1 ] may be out of range), with either of the following two options: (S0085 aspect 1)

  - Add a constraint: When sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag shall be equal to 1.

  - Add the following to the semantics of rpl_idx[ i ]: When pps_rpl1_idx_present_flag is equal to 0, the syntax element rpl_idx[ 0 ] shall be in the range of 0 to min(sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ]) − 1 and the syntax element rpl_idx[ 0 ] is represented by Ceil( Log2(min( sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ] ) ) ) bits, otherwise (pps_rpl1_idx_present_flag is equal to 1) the syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits.

- Fix an asserted bug in the syntax condition "if( !pps_rpl_info_in_ph_flag  | |  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0 )" in the PH syntax (when pps_rpl_info_in_ph_flag is equal to 0 the variable RplsIdx[ 1 ] in the context is used undefined), with the following changes: (S0085 aspect 2)

  - Add ",otherwise rpl_sps_flag[ i ] is inferred to be equal to 0" to the end of inference of rpl_sps_flag[ i ].

  - Add "num_ref_entries[ listIdx ][ sps_num_ref_pic_lists[ listIdx ] ] is infered to be equal to 0." to end of the semantics of num_ref_entries[ listIdx ][ rplsIdx ].

- Fix an asserted bug that the inference of RplsIdx[ 0 ] is missing, by adding the following inference rule (in addition to the existing inference of RplsIdx[ 1 ]): The value of rpl_idx[ 0 ] is inferred to be equal to 0 when sps_num_ref_pic_lists[ 0 ] is equal to 1 and rpl_sps_flag[ 0 ] is equal to 1. (S0085 aspect 3, S0096 aspect 3)

- Additionally check num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] for conditional signalling of ph_collocated_from_l0_flag and ph_mvd_l1_zero_flag to reduce the unnecessary signalling, by changing "num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0" in the syntax condition to "num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0  &&  num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] > 0". (S0096 aspect 1)

- Consider the number of active entries in the reference picture list when inferring the value of sh_collocated_ref_idx so that encoder can have more flexibility to change the number of active entries in slice headers, by changing "sh_collocated_ref_idx is inferred to be equal to ph_collocated_ref_idx" to "sh_collocated_ref_idx is inferred to be equal to min( ph_collocated_ref_idx, NumRefIdxActive[ !sh_collocated_from_l0_flag ] − 1 )". (S0096 aspect 2)

- Change the constraint that the collocated picture "shall be the same for all slices of a picture" to "shall be the same for all non-I slices of a picture". (S0096 aspect 2)

- Add a constraint such that when sps_idr_rpl_present_flag is equal to 0, pps_mixed_nalus_in_pic_flag is equal to 1, and pps_rpl_info_in_ph_flag is equal to 0, the value of nal_unit_type shall not be equal to IDR_W_RADL or IDR_N_LP. (S0159 aspect 1)

- Add a constraint such that when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 1. (S0159 aspect 2)

- Add GDR picture with ph_recovery_poc_cnt equal to 0 (in addition to IRAP picture) to the constraint in clause 8.3.2 on RPLs based on the value of vps_max_tid_il_ref_pics_plus1[ ][ ]. (S0159 aspect 3)

- Mark each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ], of a CLVSS picture as "used for long-term reference". (S0159 aspect 4)

- When num_ref_entries[ listIdx ][ rplsIdx ] in the ref_pic_list_struct( listIdx, rplsIdx ) syntax structure is equal to 0, ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is skipped and inferred to be equal to 0. (S0182)

# List of proposals

JVET-S0045 AHG9: On the sign of DeltaPocValSt [D. Liu, R. Sjöberg, Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, J. Ström, R. Yu(Ericsson)]

The authors claim that in the VVC draft specification v10, a sign flag value equal to 0 means that the corresponding value is positive and equal to 1 means that it is negative. However, for DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] and its sign flag strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], the use is asserted to be the opposite. Secondly, the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] associates the flag value with an “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )”. However, the “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )” is not necessarily a value, and associating a sign flag for that seems inaccurate to the proponents. Thirdly, RefPicPocList[ i ][ j ] is constructed by subtracting DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] from pocBase. It is claimed that it is cleaner to add a delta POC value to pocBase than to subtract it from pocBase.

This contribution proposes the following three modifications related to DeltaPocValSt:

- Modify the semantics of strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ], such that it a) swaps the meaning of positive and negative to align with the semantics of other sign flags and b) associates strp_entry_sign_flag[ listIdx ][ rplsIdx ][ i ] with DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] instead of with an “i-th entry in the syntax structure ref_pic_list_struct( listIdx, rplsIdx )”.

- Swap the sign assignment of DeltaPocValSt[ listIdx ][ rplsIdx ][ i ] in equation (155).

- Change the subtraction to addition when assigning RefPicPocList[ i ][ j ] using DeltaPocValSt[ i ][ RplsIdx[ i ] ][ j ] in equation (205).

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 2 of this contribution belongs to this category.

When sps_inter_layer_ref_pics_present_flag is equal to 1, sps_idr_rpl_present_flag is skipped and inferred to be equal to 1, and constrain the value of sps_idr_rpl_present_flag to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

JVET-S0123 AHG9: On reference picture list constraints [V. Seregin, Y. He, M. Coban, A. K. Ramasubramonian, M. Karczewicz (Qualcomm)]

In VVC, IDR VCL NAL may have reference picture list signalled in a picture header or a slice header, i.e, when pps_rpl_info_in_ph_flag is equal to 1 or sps_idr_rpl_present_flag is equal to 1, however there is no constraint to have those syntax elements equal to 1 for inter-layer prediction.

This contribution proposes to invoke generating unavailable reference picture for IDR and two approaches are proposed to handle inter-layer prediction:

- Remove sps_idr_rpl_present_flag and always signal RPL for IDR, which is like I-slice case.

- Require either sps_idr_rpl_present_flag or pps_rpl_info_in_ph_flag to be equal to 1 for a dependent layer or require having slice type equal to I for an independent layer. It is also proposed to set reference picture list to be empty only when sps_idr_rpl_present_flag is equal 0 and pps_rpl_info_in_ph_flag equal to 0.

JVET-S0082 AHG8/AHG9: On signaling of inter-layer reference picture layer index [H. Jang, Hendry, S. Paluri, J. Nam, S. Kim, J. Lim (LGE)]

This contribution asserted that the syntax element ilrp_idx[ listIdx ][ rplsIdx ][ i ] is not needed to be present when it is known that there is only one direct reference layer for the current layer.

In this contribution, it is proposed that when the value of sps_inter_layer_ref_pics_present_flag is equal to 1, a new syntax element called sps_ref_layer_idx_plus1 is signalled to specify the layer index of the direct reference layer of the inter-layer reference pictures of the pictures in the CLVS. The new syntax element is used as follows:

- If the CLVS has only one direct reference layer, the value of sps_ref_layer_idx_plus1 is greater than 0. Syntax element ilrp_idx[ listIdx ][ rplsIdx ][ i ] is not present when sps_ref_layer_idx_plus1 is not equal to 0 and its value is derived to be equal to sps_ref_layer_idx_plus1 − 1.

- Otherwise (i.e., the CLVS has multiple reference layers), the value of sps_ref_layer_idx_plus1 is equal to 0.

JVET-S0083 AHG8/AHG9: On signaling of ILRP layer index using delta value [Hendry, H. Jang, J. Nam, S. Kim, J. Lim (LGE)]

This contribution proposes to change the signalling of ILRP layer index in RPL structure from signalling the layer index to signalling the delta of the layer index between the layer where the RPL structure belongs to and the reference layer.

The proposed change is asserted to give two benefits as follows:

- The number of bits required for signalling may be less for common use cases such as spatial scalability and / or quality scalability. In those such cases, inter-layer prediction is done by referring to one layer below the current layer.

- Signalling ILRP using delta value is friendlier for SPS sharing. It is asserted that current SPS cannot be shared across dependent layers since the layer index of ILRPs signalled in the SPS would mostly be different. By signalling the layer index of ILRPs using delta value, it improves the chance for SPS to be shared across dependent layers.

JVET-S0085 AHG9: On semantics related to reference picture lists [T. Chujoh, E. Sasaki, T. Ikai (Sharp)]

- The first problem is that when sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag is equal to 0, and rpl_idx[ 1 ] is less than sps_num_ref_pic_lists[ 0 ], the value of rpl_idx[ 1 ] may be out of range.

Option 1: Add a constraint: When sps_num_ref_pic_lists[ 0 ] is greater than sps_num_ref_pic_lists[ 1 ], pps_rpl1_idx_present_flag shall not be equal to 0.

Option 2: Add the following to the semantics of rpl_idx[ i ]: When pps_rpl1_idx_present_flag is equal to 0, the syntax element rpl_idx[ 0 ] shall be in the range of 0 to min(sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ]) − 1 and the syntax element rpl_idx[ 0 ] is represented by Ceil( Log2(min( sps_num_ref_pic_lists[ 0 ], sps_num_ref_pic_lists[ 1 ] ) ) ) bits, otherwise (pps_rpl1_idx_present_flag is equal to 1) the syntax element rpl_idx[ i ] is represented by Ceil( Log2( sps_num_ref_pic_lists[ i ] ) ) bits.

- The second problem is that since the variable RplsIdx[ 1 ] before the derivation process is used, the value of RplsIdx[ 1 ] is undefined.

- The third problem is that when sps_num_ref_pic_lists[ 0 ] is equal to 1, the syntax element rpl_idx[ 0 ] may be undefined.

JVET-S0096 AHG9: On RPL syntax and semantics [J. Chen, Y. Ye, R.-L. Liao (Alibaba)]

Aspects 1 to 4 of this contribution belong to this category.

This contribution includes five proposals related to reference picture list (RPL) as follows

- Proposal 1: it is proposed to additionally check num_ref_entries[0][RplsIdx[0]] when signal ph_collocated_from_l0_flag and ph_mvd_l1_zero_flag to reduce the unnecessary signaling.

- Proposal 2: it is proposed to consider the number of active entries in the reference picture list when inferring the value of sh_collocated_ref_idx so that encoder can have more flexibility to change the number of active entries in slice header.

- Proposal 3: Currently the inference of rpl_idx[0] is missing in the spec. Thus, it is proposed to fix the inference rule of rpl_idx[i] so that both rpl_idx[0] and rpl_idx[1] have correct inferred value.

- Proposal 4: it is proposed to fix the constraint on collocated picture by removing the constraint on I slices since the constraint cannot be imposed on I slices.

- Proposal 5: ...

JVET-S0159 AHG9: Reference picture list (RPL) cleanups [Y.-K. Wang, L. Zhang, Z. Deng (Bytedance)]

This contribution proposes the following changes related to parameter sets and picture header:

- Add a constraint such that when sps_idr_rpl_present_flag is equal to 0, pps_mixed_nalus_in_pic_flag is equal to 1, and pps_rpl_info_in_ph_flag is equal to 0, the value of nal_unit_type shall not be equal to IDR_W_RADL or IDR_N_LP.

- Add a constraint such that when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0, the value of sps_inter_layer_ref_pics_present_flag shall be equal to 1.

- Add GDR picture with ph_recovery_poc_cnt equal to 0 (in addition to IRAP picture) to the constraint in clause 8.3.2 on RPLs based on the value of vps_max_tid_il_ref_pics_plus1[ ][ ].

- Mark each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ], of a CLVSS picture as "used for long-term reference".

JVET-S0182 AHG9: On Reference Picture List Sgnalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

The condition for signalling ltrp_in_slice_header_flag[ listIdx ][ rplsIdx ] is proposed to be modified such that it is signalled only when num_ref_entries[ listIdx ][ rplsIdx ] is greater than 0 in addition to other conditions.