| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0141-v2 |
| --- | --- |

| Title: | AHG9: A Summary of Proposals Related to HRD |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Sachin Deshpande | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This contribution intends to provide a summary of proposals on core aspects of HRD (including HRD operation, related SEI message signalling and sub-bitstream extraction).

Following proposals listed in JVET-S0137-v2 section 3.1.8 are covered in this summary: JVET-S0064, JVET-S0075 (aspect 2), JVET-S0097, JVET-S0080, JVET-S0086, JVET-S0098, JVET-S0099, JVET-S0101, JVET-S0102, JVET-S0117, JVET-S0118. Additionally JVET-S0049 Item 1 is covered.

It is suggested that this summary be used for the reviewing of these proposals, such that the discussions may be done in a more structured and efficient manner.

# List of design questions

Related to HRD Signalling and operation:

- Conditionally signal bp_sublayer_dpb_output_offsets_present_flag (and bp_dpb_output_tid_offset[ i ]) only when bp_max_sublayers_minus1 is greater than 0 and infer it otherwise?

Alternatively, add following constraint: when bp_max_sublayers_minus1 is equal to 0, bp_sublayer_dpb_output_offsets_present_flag shall be equal to 0. (JVET-S0064)

- When sps_video_paramater_set_id is equal to 0:

  - Infer TotalNumOlss to be equal to 1 (JVET-S0075, JVET-S0097)

  - Infer NumLayersInOls[ 0 ] to be equal to 1 (JVET-S0075, JVET-S0097)

  - Infer vps_layer_id[ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Require that parameter sets referred to shall have same nuh_layer_id value (JVET-S0075)

  - Infer LayerIdInOls[ 0 ][ 0 ] to be equal to the single value of nuh_layer_id of all the VCL NAL units. (JVET-S0097)

  - Infer NumSubLayersInLayerInOLS[ 0 ][ 0 ] to be equal to sps_max_sublayers_minus1 + 1. (JVET-S0097)

  - Modify the inference rule for vps_max_sub_layers_minus1 and range of values for sps_max_sublayers_minus1 by adding quoted text and not inferring vps_max_layers_minus1 to be equal to 6? (JVET-S0097)

“When sps_video_parameter_set_id is greater than 0,” the value of sps_max_sublayers_minus1 shall be in the range of 0 to vps_max_sublayers_minus1, inclusive. “When sps_video_parameter_set_id is equal to 0, the value of sps_max_sublayers_minus1 shall be in the range of 0 to 6, inclusive, and vps_max_sublayers_minus1 is inferred to be equal to sps_max_sublayers_minus1

OR

Modify only the inference for vps_max_sub_layers_minus1 (JVET-S0049 Item 1, JVET-S0097) as: When sps_video_parameter_set_id is equal to 0, the value of vps_max_sublayers_minus1 is inferred to be equal to “sps_max_sublayers_minus1” (instead of 6)

- Modify the semantics of bp_max_sublayers_minus1 to change the range of values from 0 to vps_max_sublayers_minus1 to instead require it to be equal to maxSubLayers (given as input to the ols_hrd_parameters() syntax structure)? (JVET-S0097)

- Modify the additional bumping process for DPB by doing following? (JVET-S0080)

  - Move the process for setting the value of PicLatencyCount from within additional bumping process to within the output and removal of pictures from the DPB (i.e., sub clause C.5.2.2) after the invocation of bumping process.

  - Remove the additional bumping process.

- Fix an asserted bug in Equation C.10 (also applicable to HEVC)? (JVET-S0101)

- Add a constraint on sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset as follows? (JVET-S0101)

When bp_concatenation_flag is equal to 0, the sum of InitCpbRemovalDelay[ Htid ][ ScIdx ] and InitCpbRemovalDelayOffset[ Htid ][ ScIdx ] shall be the same as in the previous buffering period.

- Include PH NAL unit into the bistream suject to HRD conformance checking using either of the following alternative options? (JVET-S0118)

  - Include PH NAL unit in Type I bitstream

  - Include PH NAL unit in Type II bitstream

Related to sub-bitstream extraction:

- Perform the asserted bug-fixes for sub-bitstream extraction process for determining which AUD NAL units of the input bitstream are kept in the output sub-bitstream as follows? (JVET-S0086)

  - Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value.

  - Add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction.

- Extend conformance for subpictures for the case where higher layers of an OLS contain multiple subpictures while lower layers of OLS do not use subpicture partitioning? (JVET-S0098)

  - Support extraction of subpictures which have no subpictures in reference layers?

  - Change the derivation of RprConstraintsActive[ i ][ j ] to incorporate the number of subpictures in reference picture and current picture and set RprConstraintsActive[ i ][ j ] to one if both values differ?

  - Signal a new syntax element non_subpic_layers_fraction[ i ] which specifies the fraction of the bitstream level limits associated with layers in the bitstream that have sps_num_subpics_minus1 equal to 0, for each i ?

  - Handle accumulated levels for subpicture sets by using one of the following options?

    - Option 1: Remove the accumulation and adjust equation D.11 to use a single subpicture.

    - Option 2: Modify equation D.10 (and D.11)

- Modify subpicture extraction process regarding handling of scalable-nested SEI messages by performing following? (JVET-S0099)

  - Removal of scalable-nested SEI messages for non-target subpictures

  - Removal before replacing non-scalable-nested SEI messages

- Modify OLS extraction process by performing following? (JVET-S0099)

  - Add a constraint that the BP/PT/DUI SEI messages for any two OLSs with the same layer set are the same

  - Modification regarding AUs that become IRAP or GDR AU after extraction by doing one of the two options:

    - Option 1: Add AUD writing to the extraction process for AUs that become IRAP AUs or GDR AUs.

    - Option 2: Mandate AUDs to be placed in all AUs that contain IRAP or GDR pictures and add rewriting of aud_irap_or_gdr_au_flag to extraction process.

  - Place PT SEI messages in individual SEI NAL units when general_same_pic_timing_in_all_ols_flag is equal to 1

  - Require that the scalable-nested and non-scalable-nested BP SEI messages in a CVS have the same values for the five syntax elements which specify lengths of various u(v) syntax elements

- Modify subpicture sub-bitstream extraction process by performing following? (JVET-S0117)

  - Remove rewriting of PPS conformance window syntax elements since pps_conformance_window_flag is asserted to be equal to 0

  - Remove subpicture ID mapping in SPS and PPS as follows:

    - Remove the syntax elements sps_subpic_id[ j ] in all the referenced SPS NAL units and for each j that is not equal to subpicIdx.

    - Remove the syntax elements pps_subpic_id[ j ] in all the referenced PPS NAL units and for each j that is not equal to subpicIdx.

  - Rewrite virtual boundary syntax elements in SPS

# List of proposals related to HRD

JVET-S0064 AHG9: On Buffering Period Message Signalling [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

Following is proposed as a cleanup:

It is proposed to conditionally signal bp_sublayer_dpb_output_offsets_present_flag, only when bp_max_sublayers_minus1 is greater than 0 and infer it otherwise.

JVET-S0075 AHG9: On constraints and inference values when VPS is not present [Hendry, S. Paluri, S. Kim (LGE)]

Aspect 2 of this contribution belongs to this category.

The following changes are proposed:

- When VPS is not present in the CVS, the CVS shall contain only one layer (i.e., all VCL NAL unit in the CVS and the parameter sets they referred to shall have the same value of nuh_layer_id).

- When VPS is not present in the CVS, the value of TotalNumOlss and NumLayersInOls[ 0 ] are inferred to be equal to 1.

JVET-S0097 AHG8/AHG9: Clarifications to HRD specification for single-layer and multi-layers bitstreams [V. Drugeon, T. Nishi, K. Abe, T. Toma (Panasonic)]

The authors of this document believe that current VVC Draft 9 is unclear for single-layer bitstreams that do not contain any VPS (i.e. for which sps_video_parameter_set_id is equal to 0), especially in the HRD related text (Annex C and Annex D for the related SEI messages). The following modifications are proposed:

- Inference of a number of VPS variables when sps_video_parameter_set_id is equal to 0;

- Modification of the inference of vps_max_sublayers_minus1 when sps_video_parameter_set_id is equal to 0;

- Constraint on the value of bp_max_sublayers_minus1 for HRD signalling consistency.

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 1 of this contribution belongs to this category.

- When sps_video_parameter_set_id is equal to 0, the value of vps_max_sublayers_minus1 is inferred to be equal to sps_max_sublayers_minus1 (instead of 6).

JVET-S0080 AHG9: On additional bumping process in the DPB [S. Paluri, Hendry, S. Kim (LGE)]

It is asserted that the DPB operation in the current VVC specification text can be simplified by removing the additional bumping process (Annex C.5.2.3). Currently, the bumping process is invoked in the beginning of decoding of the current picture but the additional bumping process is invoked after the decoding of the current picture. As there is overlap between the two processes, it is asserted that these processes can be simplification.

The following changes are proposed:

- Move the process for setting the value of PicLatencyCount from within additional bumping process to within the output and removal of pictures from the DPB (i.e., sub clause C.5.2.2) after the invocation of bumping process.

- Remove the additional bumping process.

JVET-S0086 AHG9: On AUD NAL units in the sub-bitstream extraction process [M. M. Hannuksela (Nokia)

The contribution proposes the following asserted bug fixes of the sub-bitstream extraction process (clause C.6) for determining which AUD NAL units of the input bitstream are kept in the output sub-bitstream:

- Add AUD_NUT among the NAL unit types that are kept in the output sub-bitstream regardless of its nuh_layer_id value.

- Add removal of an AUD NAL unit for which all VCL NAL units of an AU are removed by the sub-bitstream extraction.

JVET-S0098 AHG9/12: On subpicture conformance [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

At the last meeting, it was agreed to allow higher layers of an OLS to contain multiple subpictures while the lower layers of the OLS do not use subpicture partitioning. Extensions to the conformance testing related sections of the VVC draft specification are required to handle such bitstreams, especially when individual subpictures are extracted.

This contribution proposes fixes for the following items:

- Extraction of subpictures with non-subpicture reference layers and scaling windows

- RprConstraintsActive[ i ][ j ] derivation mismatches

- OLS-specific and layer-specific constraints

- SubPicSet accumulation

JVET-S0099 AHG12: SEI message handling in subpicture extraction [R. Skupin, Y. Sanchez, K. Suehring (HHI), V. Drugeon (Panasonic)]

This contribution compares the handling of scalable-nested SEI messages in the extraction processes of a target OLS (Annex C.6) and for a target subpicture (Annex C.7) and proposes additions to the subpicture-specific extraction process for alignment that are asserted to be on oversight and intended by design. It is proposed to add to Annex C.7:

- Removal of scalable-nested SEI messages for non-target subpictures

- Removal before replacing non-scalable-nested SEI messages

JVET-S0101 AHG9: On HRD [Y. Sanchez, R. Skupin, K. Suehring, T. Schierl (HHI)]

This contribution discusses two aspects related to HRD. This first one corresponds to an asserted bug in equation C-10 (also applicable to HEVC). The second aspect discusses about making the sum of InitialCpbRemovalDelay and InitialCpbRemovalOffset constant across CVSs unless signalled otherwise.

JVET-S0102 AHG9: On OLS extraction [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

This contribution reports asserted issues with respect to the OLS extraction process in Annex C.6 of the VVC draft specification and proposes respective fixes.

4 items are identified:

- Add a constraint that the BP/PT/DUI SEI messages that apply to any OLSs that correspond to the same bitstream are the same.

- Missing AUDs for AUs that turn IDR or GDR AU after extraction

- Place PT SEI messages in individual SEI NAL units when general_same_pic_timing_in_all_ols_flag is equal to 1.

- Further restrict HRD SEI message parsing dependencies in a buffering period

JVET-S0117 AHG12: On Subpicture sub-bitstream exaction [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes the following aspects for subpicture sub-bitstream extraction process

- Bugfix on PPS conformance rewriting process

- Rewrite subpicture ID mapping in SPS and PPS

- Rewrite virtual boundary in SPS

JVET-S0118 AHG9: On HRD bitstream [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

This contribution proposes to include PH NAL unit into the bitstream subject to HRD conformance checking.

----