| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0142-r2 |
| --- | --- |

| Title: | AHG9: A summary of proposals on SPS, PPS, and APS cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 8 proposals on SPS, PPS, and APS cleanups (the agenda item 3.1.4 in JVET-S0137).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

In the first revision of this document, the following changes were made:

- Remove JVET-S0049 aspecte 1 from the summary as it is included in JVET-S0141.

- Some editorial updates.

In the second revision of this document, summary of additional contributions in this category was added.

# List of changes proposed for SPS, PPS, and APS.

- It is asserted that when extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, it is allowed that no extension_data_flag syntax elements be present. The following options for change are proposed (JVET-S0042)

  - Option 1: When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, enforce the presence of at least one extension_data_flag syntax element in the syntax.

From the following design in the current spec:

| dci_extension_flag | u(1) |
| --- | --- |
| if( dci_extension_flag ) |  |
| while( more_rbsp_data( ) ) |  |
| dci_extension_data_flag | u(1) |

To the following:

| dci_extension_flag | u(1) |
| --- | --- |
| if( dci_extension_flag ) { |  |
| do |  |
| dci_extension_data_flag | u(1) |
| while( more_rbsp_data( ) ) |  |
| } |  |

  - Option 2: Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that specifies extension_data_flag syntax elements may be present.

  - Option 3: Do nothing

- Rearrange some syntax elements in SPS so that the syntax elements of similar coding tools are grouped together.

  - All of the transform related SPS syntax are placed together (JVET-S0074 aspect 1)

  - sps_lmcs_enabled_flag is signalled right after signalling of sps_ccalf_enabled_flag  (JVET-S0074 aspect 1)

  - Group the dual tree related syntax elements together in the SPS (JVET-S0132 aspect 1)

  - Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in the picture header (PH). More specifically, group such syntaxes in SPS based on whether they are intra or inter related (JVET-S0132 aspect 2)

- Not signalling sps_max_sublayers_minus1 when sps_ptl_dpb_hrd_params_present_flag is equal to 0 (JVET-S0079)

  - Move the signalling of sps_max_sublayers_minus1 and sps_reserved_zero_4bits immediately before the signalling of profile_tier_level( 1, sps_max_sublayers_minus1 )

  - Condition the presence of the two syntax elements such that they are present only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

- Add a constraint such that when sps_video_parameter_set_id is greater than 0 and vps_all_layers_same_num_sublayers_flag is equal to 1, the value of sps_max_sublayers_minus1 shall be equal to the value of vps_max_sublayers_minus1 (JVET-R0115 aspect 2).

- Add new flag sps_one_picture_only_constraint_flag and use the flag to skip signalling of some syntax elements in SPS. The skipped syntax elements includes inter prediction, RPL, and POC related syntax elements in both SPS and PH (JVET-R0129 aspect 2).

- Constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH (JVET-S0049 aspect 4)

- When pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, constrain the values of the 6 PPS flags (i.e., pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag) to be all equal to 0 (JVET-S0049 aspect 5).

The motivation / justification of the proposed changes is as follows:

  - To be better aligned with the inference of the value to 0 when pps_no_pic_partition_flag is equal to 1.

  - For more optimal syntax in the case of pps_rpl_info_in_ph_flag or pps_wp_info_in_ph_flag.

  - For the case of pps_alf_info_in_ph_flag, to enable the use of ALF for the same use case that applies independent parallel encoding of different subpictures as above.

- Rearrange the syntax elements related to inter slices in the picture parameter set (PPS) in a similar manner as the grouping of the inter-slice related syntax elements in the PH. More specifically, group syntax elements in PPS based on whether they are intra or inter related (JVET-S0132 aspect 3).

- Currently due to existing constraints, only 3 bits out of 5 bits of APS Id are needed. The following is proposed: split the u(5)-coded aps_adaptation_parameter_set_id into u(2)-coded aps_reserved_zero_2bits followed by u(3)-coded aps_adaptation_parameter_set_id (JVET-S0049 aspect 6)

- If aps_chroma_present_flag is equal to 0, due to an existing constraint, alf_luma_filter_signal_flag must be equal to 1. Based on that, it is proposed that when aps_chroma_present_flag is equal to 0, alf_luma_filter_signal_flag is skipped and inferred to be equal to 1. Otherwise, when alf_chroma_filter_signal_flag and alf_cc_cb_filter_signal_flag are both equal to 0, alf_cc_cr_filter_signal_flag is skipped and inferred to be equal to 1 (JVET-S0049 aspect 7).

- It is asserted that the memory to store APSs may exceed the max memory used to store the max number of APSs in a PU due to the fact that in a PU, prefix and suffix APS NAL units with particular APS identifier and type can have different contents. The following two constraints are proposed (JVET-S0122)

  - In a PU, for an APS type, a signalled suffix APS shall not have the same identifier as a referenced APS.

  - Any two suffix (prefix resp.) APS NAL units with particular APS identifier and type signalled in a PU cannot have the same APS identifier

Summary of additional contributions submitted after the 2 days AHG conf call

- Allow inferring some SPS syntax elements from SPS in the reference layer. It is asserted that in HEVC such mechanism is allowed (i.e., for scaling list data structure). If we allow this mechanism, the following syntax elements are proposed to be inferred from SPS in direct reference layer (JVET-S0166):

  - RPLs of the current SPS from the SPS referred to by the direct reference layer.

  - Subpicture information of the current SPS from the SPS referred to by the direct reference layer

Detail mechanism: for each category of syntax elements that can be inferred, add a new flag to specify whether the syntax elements are inferred or not. If inferred, signal additional syntax element to specify the index of the direct reference layer.

- It is asserted that the current semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag are inaccurate. It currently says that it control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements. However, it actually controls the presence of those syntax elements for temporal sub-layers that are not the highest. It is proposed to update the semantics to be  more accurate (JVET-S0169 aspect 1).

sps_sublayer_dpb_params_flag is used to control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements in the dpb_parameters( ) syntax strucure in the SPS for i in range from 0 to sps_max_sublayers_minus1 − 1, inclusive, when sps_max_sublayers_minus1 is larger than 0. When not present, the value of sps_sublayer_dpb_params_flag is inferred to be equal to 0.

vps_sublayer_dpb_params_present_flag is used to control the presence of max_dec_pic_buffering_minus1[ j ], max_num_reorder_pics[ j ], and max_latency_increase_plus1[ j ] syntax elements in the dpb_parameters( ) syntax strucure in the VPS for j in range from 0 to vps_dpb_max_tid[ i ] − 1, inclusive, when vps_dpb_max_tid[ i ] in VPS is larger than 0. When not present, vps_sub_dpb_params_info_present_flag is inferred to be equal to 0.

It is noted that j instead of i is used for specifying the range of syntax element array max_dec_pic_buffering_minus1[  ], max_num_reorder_pics[  ], and max_latency_increase_plus1[  ] in order to avoid confusion of the i in vps_dpb_max_tid[ i ]

- Move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits. (JVET-S0186 aspect 1)

- Reserve sps_seq_parameter_set_id value equal to 15 for future extensibility. Additionally, do the same for value 63 of pps_pic_parameter_set_id. (JVET-S0186 aspect 2)

- Move the location of decoded picture buffer (DPB) parameters syntax structure and hypothetical reference decoder (HRD) parameters syntax structure to directly follow sps_ptl_dpb_hrd_params_present_flag (JVET-S0199).

Currently only the signalling of PTL syntax structure directly follows sps_ptl_dpb_hrd_params_present_flag (i.e., early in SPS), the signalling of DPB syntax structure is in the middle of SPS, and HRD syntax structure is near the end of SPS. The proposal would move DPB and HRD syntax structures to early position in the SPS.

- Change the coding of syntax element sps_log2_transform_skip_max_size_minus2 from ue(v) to u(2). (JVET-S0204).

- Add additional conditions for the presence of six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag.

| ... |  |
| --- | --- |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) && pps_deblocking_filter_override_enabled_flag ) |  |
| pps_dbf_info_in_ph_flag | u(1) |
| if( !pps_deblocking_filter_disabled_flag ) { |  |

| ... |  |
| --- | --- |
| if( !pps_no_pic_partition_flag && ( !pps_rect_slice_flag ¦ ¦ pps_single_slice_per_subpic_flag ¦ ¦ pps_num_slices_in_pic_minus1 > 0 ) ) { |  |
| pps_rpl_info_in_ph_flag | u(1) |
| pps_sao_info_in_ph_flag | u(1) |
| pps_alf_info_in_ph_flag | u(1) |
| if( ( pps_weighted_pred_flag ¦ ¦ pps_weighted_bipred_flag ) && pps_rpl_info_in_ph_flag ) |  |
| pps_wp_info_in_ph_flag | u(1) |
| pps_qp_delta_info_in_ph_flag | u(1) |
| } |  |
| … |  |

- Enable removal of parameter sets (i.e., PPS and APS) in non-output layer that is not used as reference for decoding of pictures in output layers during extraction process (JVET-S0219 aspect 1)

- Modify the semantics of vps_max_tid_il_ref_pics_plus1[ i ][ j ] such that when it is greater than 0 it means for decoding pictures of the i-th layer, no parameter set and picture from the j-th layer with TemporalId greater than vps_max_tid_il_ref_pics_plus1[ i ][ j ] − 1 is used as reference.

- In extraction process, in addition to removing VCL NAL unit, also remove PPS and APS with TemporalId that is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

- Move the signalling of aps_params_type to earlier position and change the signalling of aps_adaptation_parameter_set_id from u(5) to u(v). The length of the syntax element depends on the type of the APS (JVET-S0219 aspect 2)

- Move the flag indicating the presence of sh_cabac_init_flag from PPS to SPS. (JVET-S0235).

# List of proposals on SPS, PPS, and APS clean-ups

JVET-S0042 JVET-S0042 AHG9: On the parameter set extension mechanism [M. M. Hannuksela, K. Kammachi-Sreedhar (Nokia)]

When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1:

- VVC Draft 9 syntax allows no extension_data_flag syntax elements be present.

- The semantics in VVC Draft 9 state that extension_data_flag syntax elements are present.

This contribution requests JVET to decide if any clarifications on this asserted contradiction between the syntax and semantics would be needed. The contribution suggests the following options:

- Option 1: When an extension_flag in DCI, VPS, SPS, PPS, or APS is equal to 1, enforce the presence of at least one extension_data_flag syntax element in the syntax.

- Option 2: Relax the semantics so that an extension_flag in DCI, VPS, SPS, PPS, or APS equal to 1 specifies that specifies extension_data_flag syntax elements may be present.

- Option 3: Do nothing.

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspects 4 to 7 of this contribution belong to this category.

This contribution proposes the following changes related to parameter sets and picture header:

- Constrain the value of pps_alf_info_in_ph_flag to be equal to 0 when the PH is in the SH.

- When pps_rect_slice_flag is equal to 1 and pps_num_slices_in_pic_minus1 is equal to 0, constrain the values of the 6 PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag to be all equal to 0.

- Split the u(5)-coded aps_adaptation_parameter_set_id into u(2)-coded aps_reserved_zero_2bits followed by u(3)-coded aps_adaptation_parameter_set_id.

- If aps_chroma_present_flag is equal to 0, alf_luma_filter_signal_flag is skipped and inferred to be equal to 1. Otherwise, when alf_chroma_filter_signal_flag and alf_cc_cb_filter_signal_flag are both equal to 0, alf_cc_cr_filter_signal_flag is skipped and inferred to be equal to 1.

JVET-S0074 AHG9: On SPS, PH, SH syntax order [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 1 of this contribution belongs to this category.

The first aspect proposes to rearrange the SPS syntax elements so that the syntax elements of similar coding tools are grouped together. In the proposed method, all of the transform related SPS syntax are placed together. Similarly, sps_lmcs_enabled_flag is signaled right after signaling of sps_ccalf_enabled_flag.

JVET-S0079 AHG9: On signalling of sps_max_sublayers_minus1 [Hendry (LGE)]

This contribution asserted that the signalling of sps_max_sublayers_minus1 is redundant when the value of sps_ptl_dpb_hrd_params_present_flag is equal to 0. Therefore, the following changes are proposed:

- Move the signalling of sps_max_sublayers_minus1 and sps_reserved_zero_4bits immediately before the signalling of profile_tier_level( 1, sps_max_sublayers_minus1 )

- Condition the presence of the two syntax elements (i.e., sps_max_sublayers_minus1 and sps_reserved_zero_4bits) such that they are present only when sps_ptl_dpb_hrd_params_present_flag is equal to 1.

The proposed changes preserve the fact that PTL signalling, when it present, starts from byte-aligned position.

JVET-S0115 AHG9: On SPS cleanups [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspects 2 of this contribution belong to this category.

This contribution proposes the following cleanup changes on SPS.

- Semantic constraint on sps_max_sublayers_minus1

JVET-S0129 AHG9: cleanup on parameter sets and GCI [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

Aspect 2 of this contribution belongs to this category.

Aspect 2: introduce sps_one_picture_only_constraint_flag in SPS to skip inter, RPL and POC related syntax elements in both SPS and PH

JVET-S0132 AHG9: On syntax signaling order in SPS and PPS [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, X. Wang (Kwai Inc.)]

This contribution proposes the following modifications to rearrange the syntax elements in the VVC draft specification version 9:

- Group the dual tree related syntax elements together in the sequence parameter set (SPS) to make the specification text cleaner.

- Rearrange the partition constraint related syntax elements in the SPS to be consistent with the ordering of the corresponding syntax elements in the picture header (PH). More specifically, it is proposed to also group such syntaxes in SPS based on whether they are intra or inter related.

- Rearrange the syntax elements related to inter slices in the picture parameter set (PPS) in a similar manner as the grouping of the inter-slice related syntax elements in the PH. More specifically, it is proposed to also group syntax elements in PPS based on whether they are intra or inter related.

JVET-S0122 AHG9: On APS memory constraint [N. Hu, V. Seregin, M. Karczewicz (Qualcomm)

In VVC draft 9, in a PU, prefix and suffix APS NAL units with particular APS identifier and type can have different contents. However, it is reported that the memory to store APSs may exceed the max memory used to store the max number of APSs in a PU. To solve this issue, constraints on referenced and suffix APSs are proposed: in a PU, for an APS type, a signalled suffix APS shall not have the same identifier as a referenced APS. Second, it is proposed that any two suffix (prefix resp.) APS NAL units with particular APS identifier and type signalled in a PU cannot have the same APS identifier.

# List of additional proposals on SPS, PPS, and APS clean-ups

JVET-S0164 AHG9: On signalling six PPS flags for indicating the presence of syntax in PH/SH [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

In this contribution it is proposed that the presence of six PPS flags pps_alf_info_in_ph_flag, pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, and pps_qp_delta_info_in_ph_flag is further conditioned on pps_rect_slice_flag, pps_single_slice_per_subpic_flag, and pps_num_slices_in_pic_minus1. When pps_rect_slice_flag is equal to 1, pps_single_slice_per_subpic_flag is equal to 0 and pps_num_slices_in_pic_minus1 is equal to 0, the six PPS flags are all inferred to be equal to 0.

JVET-S0166 AHG9: Inferring information of the SPS from the reference layer [S.-T. Hsiang, L. Chen, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

In HEVC, the syntax elements of scaling list data structure of the current SPS can be inferred to be equal to those of the SPS referred to by the selected direct reference layer. Based on similar mechanism, this contribution proposes for VVC to allow inferring information of the SPS from the corresponding syntax elements of the SPS referred to by the selected direct reference layer as follows:

- Propose to allow inferring the RPLs of the current SPS from the SPS referred to by the direct reference layer.

- Propose to allow inferring subpicture information of the current SPS from the SPS referred to by the direct reference layer.

JVET-S0169 AHG9: Miscellaneous Cleanups for HLS [B. Wang, S. Esenlik, H. Gao, E. Alshina (Huawei)]

Aspect 1 of this contribution belongs to this category.

The current semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag is inaccurate and in some corner cases would results in semantic bugs. Specifically, when there is only one sublayer in a coding layer, these two flags do not control the presence of syntax elements of decoded picture buffer, which is contradicted to their current definition. This contribution proposes corrected semantics for them.

it is proposed to change the semantics of sps_sublayer_dpb_params_flag and vps_sublayer_dpb_params_present_flag as follows, with addition highlighted, and deletion strike through:

sps_sublayer_dpb_params_flag is used to control the presence of max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] syntax elements in the dpb_parameters( ) syntax strucure in the SPS for i in range from 0 to sps_max_sublayers_minus1 - 1, inclusive, when sps_max_sublayers_minus1 is larger than 0. When not present, the value of sps_sublayer_dpb_params_flag is inferred to be equal to 0.

vps_sublayer_dpb_params_present_flag is used to control the presence of max_dec_pic_buffering_minus1[ j ], max_num_reorder_pics[ j ], and max_latency_increase_plus1[ j ] syntax elements in the dpb_parameters( ) syntax strucure in the VPS for j in range from 0 to vps_dpb_max_tid[ i ] - 1, inclusive, when vps_dpb_max_tid[ i ] in VPS is larger than 0. When not present, vps_sub_dpb_params_info_present_flag is inferred to be equal to 0. It is noted that j instead of i is used for specifying the range of syntax element array max_dec_pic_buffering_minus1[  ], max_num_reorder_pics[  ], and max_latency_increase_plus1[  ] in order to avoid confusion of the i in vps_dpb_max_tid[ i ]

JVET-S0186 AHG9: On SPS Cleanup [S. Deshpande, J. Samuelsson, A. Segall, P. Cowan (Sharp)]

Following is proposed as cleanups:

- Proposal 1: It is proposed to move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits.

- Proposal 2: It is proposed to keep sps_seq_parameter_set_id value equal to 15 as a reserved value for future extensibility. Additionally, the value 63 can be similarly kept reserved for pps_pic_parameter_set_id.

JVET-S0199 AHG9: On syntax grouping in SPS [H.-J. Jhu, Y.-W. Chen, X. Xiu, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

This contribution proposes to move the location of decoded picture buffer (DPB) parameters syntax structure and hypothetical reference decoder (HRD) parameters syntax structure to directly follow sps_ptl_dpb_hrd_params_present_flag. It is claimed that this modification makes the specification text cleaner as it groups together the DPB and HRD related syntax elements and reduces the condition checks in the SPS.

JVET-S0204 AHG9: On signalling of the maximum transform skip size [K. Unno, K. Kawamura (KDDI)]

In this contribution, it is proposed that the maximum transform skip size (sps_log2_transform_skip_max_size_minus2) is signalled by u(2) instead of ue(v). In the current spec, possible values of sps_log2_transform_skip_max_size_minus2 are only in the range of 0 to 3.

JVET-S0219 AHG8/AHG9: On APS NAL unit clean-up [H. Jang, J. Nam, J. Lim, S. Paluri, Hendry, S. Kim (LGE)]

This contribution proposes the following changes:

Proposal 1: Enable the removal of parameter sets (i.e., PPS and APS) in non-output layer that is not used as reference for decoding of pictures in output layers during extraction process.

Proposal 2: Move the signalling of aps_params_type to earlier position and change the signalling of aps_adaptation_parameter_set_id from u(5) to u(v). The length of the syntax element depends on the type of the APS.

JVET-S0235 AHG9: On pps_cabac_init_flag signalling [M. Coban, V. Seregin, Y. He, Y.-J. Chang, M. Karczewicz (Qualcomm)]

This contribution proposes to move the flag indicating the presence of sh_cabac_init_flag from PPS to SPS.