| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0143-r1 |
| --- | --- |

| Title: | AHG9: A summary of proposals on picture header and slice header cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 8 proposals on picture header and slice header cleanups (the agenda item 3.1.5 in JVET-S0137).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

In the first revision of this document, summary of additional contributions in this category was added.

# List of changes proposed for PH and SH.

- Add a constraint such that the value of ph_inter_slice_allowed_flag is equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 (JVET-S0049 aspect 8)

- It is asserted there may be issue with the signalling of ph_no_output_of_prior_pics_flag. When ph_gdr_or_irap_pic_flag is equal to 0, but the associated picture the associated picture may or may not be an IRAP picture but the ph_no_output_of_prior_pics_flag is not present but there is no inference value for it. The following options are proposed to fix this asserted issue (JVET-S0055 proposal 1, JVET-S0070 proposal 1)

  - Option 1: Change the current semantics of ph_gdr_or_irap_pic_flag to be a two-way (i.e., if ph_gdr_or_irap_pic_flag is equal 1 the associated picture is a GDR or an IRAP picture, otherwise, if ph_gdr_or_irap_pic_flag is equal to 0, the associated picture is not a GDR nor an IRAP picture) (JVET-S0055 proposal 1 option 1).

  - Option 2: Apply the following (JVET-S0070 proposal 1):

    - Replace the current flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag

    - Semantics of ph_irap_pic_flag and ph_gdr_pic_flag are two-way.

    - ph_no_output_of_prior_pics_flag is present when at least ph_irap_pic_flag or ph_gdr_pic_flag is equal to 1

  - Option 3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 0. (JVET-S0055 proposal 1 option 1)

- Add a constraint such that when aud_irap_or_gdr_au_flag is present and the value of aud_irap_or_gdr_au_flag is equal to 1, the value of ph_gdr_or_irap_pic_flag shall be equal to 1. (JVET-S0055 proposal 2)

- Regarding the signalling of ph_gdr_or_irap_pic_flag when pps_mixed_nalu_types_in_pic_flag is equal to 1 (JVET-S0135):

  - Option 1: add a constraint such that the value of ph_gdr_or_irap_pic_flag shall be equal to 0 when pps_mixed_nalu_types_in_pic_flag is equal to 1

  - Option 2: condition the presence of ph_gdr_or_irap_pic_flag on pps_mixed_nalu_types_in_pic_flag being equal to 0.

  - Option 3: add a constraint to the definition of GDR picture that pps_mixed_nalu_types_in_pic_flag is equal to 0.

For option 3, the proponent provided the text as a NOTE, instead of a constraint.

- Change coding of ph_pic_parameter_set_id from ue(v) to u(6) (JVET-S0076 item b), JVET-S0110 aspect 3)

- Rearrange the position of syntax elements in PH and SH.

  - Move syntax elements related to SAO and de-blocking process to earlier position in PH and in SH (i.e., signal them right after ALF and LMCS related syntax) JVET-S0074 aspect 2)

  - Move ph_non_ref_pic_flag to earlier position, i.e., before any syntax element whose presence is conditioned on the value of other syntax element(s). Immidiately after ph_gdr_or_irap_pic_flag (JVET-S0076 item a) )

  - Move ph_pic_parameter_set_id to earlier position (i.e., after ph_non_ref_pic_flag) (JVET-S0076 item c) )

  - Move down the location of syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag and condition the presence of ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture and non-gdr picture when ph_recovery_poc_cnt is equal to 0 (JVET-S0070 proposal 2)

- If item 5.c) is agreed, condition the presence of ph_gdr_pic_flag such that it may be present only when sps_gdr_enabled_flag is equal to 1 (JVET-S0076 item d) )

- Change the constraint on the value of sh_picture_header_in_slice_header_flag as follows: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when sps_subpic_info_present_flag is equal to 1 and and pps_no_pic_partition_flag is equal to 0 (JVET-S0120 aspect 1)

- Change the constraint on the value of sh_picture_header_in_slice_header_flag as follows: (JVET-S0120 aspect 2 and 3)

  - Option 1: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when  The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, pps_alf_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1.

  - Option 2: remove the following constraint: the value of sh_picture_header_in_slice_header_flag shall be equal to 0 when  The value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag, or pps_qp_delta_info_in_ph_flag is equal to 1

- Change VTM encoder to signal the override flag (i.e., ph_partition_constraints_override_flag) as zero when the derived partition constraints for a PH are identical to the ones in the SPS (JVET-S0133)

- Change the semantics of the signalled values of partition constraints in PH from values for overriding the corresponding values signalled in SPS to the delta values relative to the corresponding values signalled in the SPS (JVET-S0133)

The proposed changes have been implemened and simulation results is provided.

The results reportedly show that the encoder-only change to remove the redundant signalling introduces -0.12% (RA) and -0.06%(LD) BD-rate reduction for class D. On top of the encoder-only change, the proposed syntax modifications introduce -0.20%(RA), -0.19% (LD) BD-rate reduction for class D. In average, the encoder-only change introduces 0.00 (AI), -0.03% (RA) and -0.14%(LDB) BD-rate changes, and on top of the encoder-only change, the proposed syntax modification introduces 0.00(AI), -0.04% (RA) and -0.xx%(LDB) BD-rate changes under CTC.

It is noted that the results show no impact on AI because current VTM always disable the override of the partition constraints for intra pictures.

Summary of additional contributions submitted after the 2 days AHG conf call

- It is asserted that reference rule for the value of ph_collocated_from_l0_flag when L0 empty and L1 non-empty is missing. Update the current inference rule for ph_collocated_from_l0_flag (JVET-S0174 aspect 3)

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present and ph_temporal_mvp_enabled_flag and pps_rpl_info_in_ph_flag are both equal to 1, the value of ph_collocated_from_l0_flag is inferred to be equal to (num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]  = =  0 ? 1 : 0).

- Further study result for the item 2 above (i.e., harmonization of JVET-S0055 proposal 1 and JVET-S0070 proposal 1). Change IRAP or GDR flag in PH with the following options (JVET-S0193 aspect 1):

Option 1:	Change the semantics of ph_gdr_or_irap_pic_flag to be a two-way.

Option 2:	Replace the flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag with two-way semantics.

Option 3:	Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 1.

Option 4:	Move no_output_of_prior_pics_flag to slice header.

- If option 3 or option 4 from the above item is agreed, change the name of syntax element ph_gdr_or_irap_pic_flag with ph_irap_pic_flag. (JVET-S0193 aspect 1)

- Do one of the following (JVET-S0200):

Option 1:	Add a constraint such that ph_inter_slice_allowed_flag shall be equal to 1 when ph_gdr_pic_flag is equal to 1.

Option 2:	Condition the presence of ph_inter_slice_allowed_flag on ph_gdr_pic_flag being equal to 0.

- Add a constraint such that when vps_max_layers_minus1 is equal to 0, an AUD is present for the current AU, and aud_irap_or_gdr_au_flag is equal to 0, the value of ph_gdr_or_irap_pic_flag shall be equal to 0 (JVET-S0160 aspect 4).

- Add a constraint such that when an AUD is present for the current AU and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2 (JVET-S0160 aspect 2).

- Add a constraint such that when an AUD is present for the current AU and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2 (JVET-S0160 aspect 3).

- Add the following constraint for CTU size when using ILRP as collocated reference picture as follows (JVET-S0174 aspect 1):

sh_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

[…]

Let colPicList be set equal to sh_collocated_from_l0_flag ? 0 : 1. It is a requirement of bitstream conformance that the picture referred to by sh_collocated_ref_idx shall be the same for all non-I slices of a coded picture and the value of RprConstraintsActiveFlag[ colPicList ][ sh_collocated_ref_idx ] shall be equal to 0 and the value of sps_log2_ctu_size_minus5 in the SPS referred to by the picture referred to by sh_collocated_ref_idx shall be equal to or greater than the value of sps_log2_ctu_size_minus5 in the SPS referred to by the current picture.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution. the same scaling window offsets and same or larger CTU size as the current picture.

# List of proposals on PH and SH clean-ups

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 8 of this contribution belongs to this category.

- Constrain the value of ph_inter_slice_allowed_flag to be equal to 1 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0.

JVET-S0055 AHG9: On signaling IRAP and GDR pictures [B. Choi, S. Wenger, S. Liu (Tencent)]

The following semantics modifications on ph_gdr_or_irap_pic_flag are proposed:

- The constraints of the semantics of ph_gdr_or_irap_pic_flag are specified as two way, to mandate the presence of ph_no_output_of_prior_pics_flag, when the current picture is an IRAP picture.

- The value of ph_gdr_or_irap_pic_flag is set equal to aud_irap_or_gdr_au_flag, when aud_irap_or_gdr_au_flag is present.

JVET-S0070 AHG9: On flag ph_gdr_or_irap_pic_flag [L. Chen, C.-W. Hsu, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei (MediaTek)]

This contribution proposes to improve the signalling of an IRAP picture and a GDR picture in the PH.

Proposal 1:

- Rename ph_gdr_or_irap_pic_flag to ph_irap_pic_flag.

- Make ph_irap_pic_flag and ph_gdr_pic_flag the two way flags for an IRAP picture and a GDR picture, respectively.

- Signal the flag ph_no_output_of_prior_pics_flag on the condition of ph_irap_pic_flag or ph_gdr_pic_flag.

- Signal the flag ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture.

Proposal 2:

- Proposal 1 with moving down the location of syntax elements ph_inter_slice_allowed_flag and ph_intra_slice_allowed_flag.

- Signal the flag ph_inter_slice_allowed_flag only for picture with no inter-layer prediction and non-irap picture and non-gdr picture when ph_recovery_poc_cnt is equal to 0.

JVET-S0074 AHG9: On SPS, PH, SH syntax order [M. G. Sarwer, Y. Ye (Alibaba)]

Aspect 2 of this contribution belongs to this category.

In the aspect 2, picture header (PH) and slice header (SH) syntax elements related to SAO and de-blocking process are signaled right after ALF and LMCS related syntax.

JVET-S0076 AHG9: On syntax elements in the beginning of picture header [Hendry (LGE)]

This contribution asserted that the signalling of syntax elements in the beginning of picture header may need clean-ups. The proposed clean-ups is asserted help system to access syntax elements in the beginning of picture header.

The following changes are proposed:

- Move ph_non_ref_pic_flag to earlier location in picture header before any other syntax elements whose presence is conditioned (i.e., earlier than ph_gdr_pic_flag and ph_intra_slice_allowed_flag). It is asserted that ph_non_ref_pic_flag is needed for system use so it should be located in earlier location that is fixed.

- Change the coding of ph_pic_parameter_set_id from ue(v) to u(6).

- Move ph_pic_parameter to earlier location in picture header before any other syntax elements whose presence is conditioned.

- Condition the presence of ph_gdr_pic_flag such that it may be present only when sps_gdr_enabled_flag is equal to 1.

JVET-S0110 AHG9: On HLS Editorial cleanup [Y. He, V. Seregin, M. Coban, M. Karczewicz (Qualcomm)]

Aspect 3 of this contribution belongs to this category.

Code ph_pic_parameter_set_id with u(6)

JVET-S0120 AHG9: Constraints on sh_picture_header_in_slice_header_flag [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

This contribution proposes to modify constraints on the syntax element sh_picture_header_in_slice_header_flag as follows:

- When pps_no_pic_partition_flag is equal to 1 and sps_subpic_info_present_flag is equal to 1, there is no need to constrain the value of sh_picture_header_in_slice_header_flag to be equal to 0.

- When pps_alf_info_in_ph_flag is equal to 1, the value of sh_picture_header_in_slice_header_flag shall be equal to 0.

- An alternative proposal of item 2 is to remove the constraint of sh_picture_header_in_slice_header_flag dependent on the value of pps_rpl_info_in_ph_flag, pps_dbf_info_in_ph_flag, pps_sao_info_in_ph_flag, pps_wp_info_in_ph_flag and pps_qp_delta_info_in_ph_flag.

JVET-S0133 AHG9: On partition constraints override in picture header [Y.-W. Chen, X. Xiu, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai Inc.)]

When encoding one picture, the partition constrains are derived based on the partitioning used in the previously encoded pictures. In current VTM, when the derived partition constraints are identical to the values signaled in SPS, the derived values are still signaled in the PH to override the values signalled in the SPS which is obviously redundant. It is proposed to modify the VTM encoder to signal the override flag as zero when the derived partition constraints are identical to the ones in the SPS.

Moreover, it is observed that, the partition constraints signalled in the PH are usually similar to or identical to the ones signalled in the SPS. It is proposed, in the proposal, to signal the differences between the values of partitioning constraint in the PH and the corresponding values in SPS to have a more efficient override mechanism of partition constraints.

The results reportedly show that the encoder-only change to remove the redundant signalling introduces -0.12% (RA) and -0.06%(LD) BD-rate reduction for class D. On top of the encoder-only change, the proposed syntax modifications introduce -0.20%(RA), -0.19% (LD) BD-rate reduction for class D. In average, the encoder-only change introduces 0.00 (AI), -0.03% (RA) and -0.14%(LDB) BD-rate changes, and on top of the encoder-only change, the proposed syntax modification introduces 0.00(AI), -0.04% (RA) and -0.xx%(LDB) BD-rate changes under CTC. It is noted that the results show no impact on AI because current VTM always disable the override of the partition constraints for intra pictures.

JVET-S0135 AHG9: On IRAP and GDR picture signaling in picture header [X. Xiu, Y.-W. Chen, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai)]

In this contribution, three methods are proposed to prevent signaling one picture as one IRAP or GDR picture in picture header when pps_mixed_nalu_types_in_pic_flag in PPS is equal to one: 1) adding one constraint for ph_gdr_or_irap_pic_flag to be 0 when pps_mixed_nalu_types_in_pic_flag is equal to 1; 2) conditioning the presence of ph_gdr_or_irap_pic_flag on pps_mixed_nalu_types_in_pic_flag being equal to 0; 3) adding one constraint to the definition of GDR picture that pps_mixed_nalu_types_in_pic_flag is equal to 0.

# List of additional proposals on PH and SH clean-ups

JVET-S0160 AHG9/AHG12/AHG8: Miscellaneous HLS cleanups [Y.-K. Wang, Z. Deng, L. Zhang, K. Zhang (Bytedance)]

Aspects 2 to 4 of this contribution belong to this category.

- Add a constraint: When an AUD is present for the current AU, and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2.

- Add a constraint: When an AUD is present for the current AU, and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2.

- Add a constraint: When vps_max_layers_minus1 is equal to 0, an AUD is present for the current AU, and aud_irap_or_gdr_au_flag is equal to 0, the value of ph_gdr_or_irap_pic_flag shall be equal to 0.

JVET-S0174 AHG9: Miscellaneous cleanups [R. Skupin, Y. Sanchez, K. Suehring, T. Schierl (HHI)]

Aspects 1 and 3 of this contribution belong to this category.

Item 1) CTU size constraint for ILRP as collocated reference picture

sh_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

[…]

Let colPicList be set equal to sh_collocated_from_l0_flag ? 0 : 1. It is a requirement of bitstream conformance that the picture referred to by sh_collocated_ref_idx shall be the same for all non-I slices of a coded picture and the value of RprConstraintsActiveFlag[ colPicList ][ sh_collocated_ref_idx ] shall be equal to 0 and the value of sps_log2_ctu_size_minus5 in the SPS referred to by the picture referred to by sh_collocated_ref_idx shall be equal to or greater than the value of sps_log2_ctu_size_minus5 in the SPS referred to by the current picture.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution. the same scaling window offsets and same or larger CTU size as the current picture.

Item 3) Undefined value of ph_collocated_from_l0_flag when L0 empty and L1 non-empty

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present and ph_temporal_mvp_enabled_flag and pps_rpl_info_in_ph_flag are both equal to 1, the value of ph_collocated_from_l0_flag is inferred to be equal to (num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]  = =  0 ? 1 : 0).

JVET-S0193 AHG9: Bugfix and Cleanup on ph_no_output_of_prior_pics_flag and ph_gdr_or_irap_pic_flag [B. Choi, S. Wenger, S. Liu (Tencent), L. Chen, C.-W. Hsu, S.-T. Hsiang, O. Chubach, Y.-W. Huang, S.-M. Lei(MediaTek)]

This contribution proposes the harmonized solutions of JVET-S0055 proposal-1 and JVET-S0070 proposal-1 for bug fix of ph_no_output_of_prior_pics_flag and cleanups on ph_gdr_or_irap_pic_flag. The following Proposal 1 Option 3 and Proposal 2 are recommended.

Proposal-1: Four options are proposed to resolve the issue that the value of ph_no_output_of_prior_pics_flag is used without inferrence rule, when ph_no_output_of_prior_pics_flag is not present.

- Option-1: Change the semantics of ph_gdr_or_irap_pic_flag to be a two-way

- Option-2: Replace the flag ph_gdr_or_irap_pic_flag with two new flags: ph_irap_pic_flag and ph_gdr_pic_flag with two-way semantics.

- Option-3: Add the following inference: when not present, the value of ph_no_output_of_prior_pics_flag is inferred to be equal to 1.

- Option-4: Move no_output_of_prior_pics_flag to slice header

Proposal-2: When Option-3 or 4 is adopted, it is proposed to replace ph_gdr_or_irap_pic_flag with ph_irap_pic_flag for syntax cleanup.

JVET-S0200 AHG9: On ph_inter_slice_allowed_flag in GDR picture [H.-J. Jhu, X. Xiu, Y.-W. Chen, T.-C. Ma, C.-W. Kuo, X. Wang (Kwai Inc.)]

In this contribution, to allow inter-related tools in GDR picture two methods are proposed to prevent signaling one picture as one inter slice allowed picture in picture header when ph_gdr_pic_flag in PH is equal to one: 1) adding one constraint for ph_inter_slice_allowed_flag to be 1 when ph_gdr_pic_flag is equal to 1; 2) conditioning the presence of ph_inter_slice_allowed_flag on ph_gdr_pic_flag being equal to 0. It is claimed that this modification makes the specification text cleaner as it includes the spirit of the intent of the GDR design.