| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0144-v3 |
| --- | --- |

| Title: | AHG9: A summary of proposals on high level tool control |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Li Zhang, Ye-Kui Wang 8910 University Center Lane San Diego, CA 92122, USA | Email: | lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution provides a summary of the 15 proposals on High level tool control (the agenda item 3.1.1 in JVET-S0137).

It is suggested that this summary, in terms of a list of design questions, is used for the reviewing of these proposals, such that the discussions can be in a more structured and efficient manner.

# List of proposed items

## Removal of SPS BDOF/DMVR/PROF control present flag

- Remove the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag and instead condition the presence of ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag on the existing syntax elements sps_bdof_enabled_flag, sps_dmvr_enabled_flag and sps_affine_prof_enabled_flag, respectively. (S0044)

## On TMVP derivation considering inter-layer reference picture

- Disable TMVP when either RefPic or ColRefPic is inter-layer reference picture (ILRP), but not both. (S0089 #1)

- Enable TMVP with combination of STRP and ILRP on RefPic or ColRefPic, and disable TMVP with combination of LTRP and ILRP. (S0089 #2)

- Enable TMVP when either RefPic or ColRefPic is ILRP. (S0089 #3)

| RefPic (target reference picture) | ColRefPic (reference picture of collocated block) | Current VVC | Solution #1 | Solution #2 | Solution #3 |
| --- | --- | --- | --- | --- | --- |
| STRP | ILRP | X | X | O | O |
| ILRP | STRP | X | X | O | O |
| LTRP | ILRP | O | X | X | O |
| ILRP | LTRP | O | X | X | O |

## On maximum number of subblock merge candidates

- Change the range of the value of sps_five_minus_max_num_subblock_merge_cand from 0..5 − sps_sbtmvp_enabled_flag to 0..5 and change the derivation of maximum number of subblock merge candidates in PH as follows: (S0088)

if( sps_affine_enabled_flag && sps_five_minus_max_num_subblock_merge_cand < 5 )
	MaxNumSubblockMergeCand = 5 − sps_five_minus_max_num_subblock_merge_cand           (88)
else
	MaxNumSubblockMergeCand = sps_sbtmvp_enabled_flag  &&  ph_temporal_mvp_enabled_flag

## On PDPC control

- Add an SPS flag to control PDPC on/off and check it in intra prediction process. (S0072, S0136)

  - For all color formats, add an SPS PDPC disabled flag (named sps_pdpc_disabled_flag) (S0072 method 1), or an SPS PDPC enabled flag (named sps_pdpc_enabled_flag). (S0136)

  - Only for 4:4:4 and 4:2:2 cases, add an SPS PDPC disabled flag (named sps_pdpc_disabled_flag). (S0072 method 3)

- Add a PDPC constrain flag in the general constraint information (GCI) syntax structure. (S0136, S0072 v2)

## On intra reference sample filtering control

- Add an SPS flag to control intra reference sample filtering on/off and check it in intra prediction process. (S0072)

  - For all color formats, add an SPS disabled flag (named sps_intra_reference_filter_disabled_flag). (S0072 method 2)

  - Only for 4:4:4 and 4:2:2 cases, add an SPS disabled flag (named sps_intra_reference_filter_disabled_flag). (S0072 method 3)

- Add an intra reference filtering constrain flag in the GCI syntax structure to constrain the added SPS flag. (S0072 v2)

## On TSRC

- Remove no_tsrc_constraint_flag. (S0073, option 3 in section 3.3)

- Change the semantics of no_tsrc_constraint_flag (removal: italic; addition: bold):

- Add "When no_transform_skip_constraint_flag is 1, the value of no_tsrc_constraint_flag shall be one.", change the semantics of no_dep_quant_constraint_flag and no_sign_data_hiding_constraint_flag to be as follows (S0062, method 3):

no_dep_quant_constraint_flag equal to 1 specifies that sps_dep_quant_enabled_flag shall be equal to 0. no_dep_quant_constraint_flag equal to 0 does not impose such a constraint. When no_tsrc_constraint_flag is equal to 1, the value of no_dep_quant_constraint_flag shall be 1.

no_sign_data_hiding_constraint_flag equal to 1 specifies that sps_sign_data_hiding_enabled_flag shall be equal to 0. no_sign_data_hiding_constraint_flag equal to 0 does not impose such a constraint. When no_tsrc_constraint_flag is equal to 1, the value of no_sign_data_hiding_constraint_flag shall be 1.

- Change the semantics to be as follows (S0069, option 1):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 or sps_transform_skip_enabled_flag shall be equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

- Change the semantics to be as follows (S0069, option 2):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 when sps_transform_skip_enabled_flag is equal to 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

- Change the semantics to be as follows (S0105):

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

- Add a semantic constraint to no_tsrc_constraint_flag to disable DQ and SDH in SPS level (S0073, option 1 in section 3.1)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 and sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag are both equal to 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

- Add a semantic constraint to no_tsrc_constraint_flag to disable DQ and SDH in all slices (S0073, option 2 in section 3.2)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1 and sh_dep_quant_used_flag and sh_sign_data_hiding_used_flag are both equal to 0 for all slices. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

- Infer sh_ts_residual_coding_disable_flag to be equal to !sps_transform_skip_enabled_flag instead of 0. (S0062, method 1)

- Add an SPS flag for TSRC, use it to control the presence of sps_dep_quant_enabled_flag and sps_sign_data_hiding_enabled_flag, and change the semantics of no_tsrc_constraint_flag to constrain the SPS flag instead of the SH flag. (S0062, method 2)

- Change both semantics of no_tsrc_constraint_flag and sh_ts_residual_coding_disabled_flag: (S0069, option 3) (removal: italic; addition: bold)

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 0 1. no_tsrc_constraint_flag equal to 0 does not impose such a constraint. When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1.

sh_ts_residual_coding_disabled_flag equal to 1 specifies that the syntax structure residual_coding( ) is used to parse the residual samples of a transform skip block for the current slice. slice_ts_residual_coding_disabled_flag equal to 0 specifies that the syntax structure residual_ts_coding( ) is used to parse the residual samples of a transform skip block for the current slice. When slice_ts_residual_coding_disabled_flag is not present, it is infered to be equal to 0.the following applies:

- If sps_transform_skip_enabled_flag is equal to 0, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 1.

- Otherwise, the value of sh_ts_residual_coding_disabled_flag is inferred to be equal to 0.

## On residual coding

- Add the following SPS and PH flags, and revise SH gating conditions and inferences: (S0093)

  - Addition of the SPS flag sps_residual_control_present_in_sh_flag.

  - Addition of picture header flags for DQ and SDH.

  - Modification of gating conditions for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to ensure that at most one of these flags are signalled at the slice level.

  - Modifications of inference rules for sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag to match the desired behaviour.

## On presence of explicit scaling list

- Allow the aps_chroma_present_flag of the scaling list APS NAL unit referenced by the PH to be equal to 0 when ChromaArrayType is greater than 0. (S0049, aspect 9)

## On chroma QP table signalling

- Change the range of sps_num_points_in_qp_table_minus1[ i ] from 0..63 + QpBdOffset to 0..62 + QpBdOffset. (S0130 aspect 1)

- Add an SPS flag (named sps_chroma_qp_identical_to_luma_flag) to skip signalling of chroma QP information when chroma QPs are set equal to luma QPs. (S0130 aspect 2)

## On ALF APS constrains

- Add one SPS flag (named sps_one_aps_per_slice_one_filter_per_aps_flag) to support the following case and change the signalling of aps_adaptation_parameter_set_id, sh_alf_aps_id_luma, sh_alf_aps_id_chroma from u(5)/u(3) to u(v). (S0134):

  - Each ALF APS contains up to 1 luma filter, up to 1 chroma filter, and no cross-component filters, and each slice in the CLVS uses 0 or 1 ALF APSs for luma adaptive loop filtering, 0 or 1 ALF APSs for chroma adaptive loop filtering, and applies no cross-component adaptive loop filtering.

  - Allow maximum of 115 ALF APSs.

## On deblocking filter parameters

- Inference of chroma DB offsets

- If pps_chroma_tool_offsets_present_flag is equal to 1, infer the values of the chroma DB offsets in the PH and SH, when not present, to be equal to the chroma DB offsets in the PPS and PH, respectively, instead of inferring the values from the luma DB offsets in the same header (as in the current VVC text). Otherwise, infer the values from the luma DB offsets in the same haeder (as in the current VVC text) (S0053, aspect 1)

- Infer the chroma DB offsets in the PH and SH to be equal to the values of the chroma DB offsets in the PPS and PH, respectively, when not present, regardless of the value of pps_chroma_tool_offsets_present_flag. But this solution is not recommended. (S0053, aspect 1, alternative solution)

- Removal the condition check of pps_deblocking_filter_override_enabled_flag for signalling the ph_deblocking_filter_override_flag. (S0121, aspect 2)

## Semantics of pps_chroma_tool_offsets_present_flag

- Change the semantics such that the chroma deblocking tc and β offset syntax elements may be (instead of "are") present in the PHs or the SHs of pictures referring to the PPS when the flag is equal to 1. (S0053, aspect 2)

## Semantics of pps_chroma_tool_offsets_present_flag

- Change the inference rules of non-present syntax elements PH/SH SAO, ALF, CCALF flags. (S0121, aspect 1)

  - Infer ph_sao_luma_enabled_flag to be equal to sps_sao_enable_flag instead of 0.

  - Infer ph_alf_enabled_flag to be equal to sps_alf_enable_flag instead of 0.

  - Infer ph_sao_chroma_enabled_flag to be as follows, instead of 0:

- When sps_sao_enable_flag is equal to 1 and ChromaArrayType is not equal to 0, ph_sao_chroma_enabled_flag is inferred to be equal to 1.

- Otherwise, ph_sao_chroma_enabled_flag is inferred to be equal to 0.

  - Infer ph_alf_cb_flag/ph_alf_cr_flag to be 0 if one of the following two conditions are true, and to be 1 otherwise:

- ChromaArrayType is equal to 0.

- ph_alf_enabled_flag is equal to 0.

  - Infer ph_cc_alf_cb_enabled_flag/ph_cc_alf_cr_enabled_flag to be 0 if one of the following two conditions are true, and to be 1 otherwise:

- sps_ccalf_enabled_flag is equal to 0.

- ph_alf_enabled_flag is equal to 0.

  - Infer sh_alf_cb(r)_flag as follows:

- If sh_alf_enabled_flag or ph_alf_cb(r)_flag is equal to 0, sh_alf_cb(r)_flag is inferred to be equal to 0.

- Otherwise (both sh_alf_enabled_flag and ph_alf_cb(r)_flag are equal to 1), sh_alf_cb(r)_flag is inferred to be equal to 1.

  - Infer sh_cc_alf_cb(r)_enabled_flag as follows:

- If sh_alf_enabled_flag or ph_cc_alf_cb(r)_enabled_flag is equal to 0, sh_cc_alf_cb(r)_enabled_flag is inferred to be equal to 0.

- Otherwise (both sh_alf_enabled_flag and ph_cc_alf_cb(r)_enabled_flag are equal to 1), sh_cc_alf_cb(r)_enabled_flag is inferred to be equal to 1.

# List of proposals on high level tool control

JVET-S0044 AHG9: Cleanup of high-level signalling of BDOF, DMVR and PROF [M. Pettersson, R. Sjöberg, M. Damghanian, J. Enhorn, Z. Zhang, J. Ström, R. Yu, D. Liu (Ericsson)]

This contribution proposes to remove the syntax elements sps_bdof_control_present_in_ph_flag, sps_dmvr_control_present_in_ph_flag and sps_prof_control_present_in_ph_flag from the VVC specification and instead condition the presence of ph_bdof_disabled_flag, ph_dmvr_disabled_flag and ph_prof_disabled_flag on the existing syntax elements sps_bdof_enabled_flag, sps_dmvr_enabled_flag and sps_affine_prof_enabled_flag, respectively.

It is asserted that the three control present flags together at most saves three bits per picture, which is asserted to not motivate the extra complexity of the signalling mechanism. The authors estimate that the proposed cleanup removes 6 syntax table rows and 23 lines of semantics text.

JVET-S0049 AHG9/AHG8/AHG12: On parameter sets and picture header [Y.-K. Wang, L. Zhang, Z. Deng, K. Zhang (Bytedance)]

Aspect 9 of this contribution belongs to this category.

- Allow the aps_chroma_present_flag of the scaling list APS NAL unit referenced by the PH to be equal to 0 when ChromaArrayType is greater than 0.

JVET-S0053 AHG9: Inference of chroma deblocking offset values in the PH and SH [Z. Zhang, M. Pettersson, M. Damghanian, J. Enhorn, K. Andersson, J. Ström, R. Sjöberg, R. Yu, D. Liu (Ericsson)]

This contribution claims that chroma deblocking (DB) offset values (β and tC) that are signaled in the PPS or in the PH can never be used in the deblocking process due to the inference rule in the current VVC draft specification. This document contains two proposals:

Proposal 1 suggests that when the chroma DB offsets are not present in the PH and SH, the inference values are determined based on the value of pps_chroma_tool_offsets_present_flag. It is asserted that proposal 1 follows the intention of the decision in the meeting notes from the 18-th JVET meeting. The proposed changes are as follows (exact text is to be found in the contribution):

- When pps_chroma_tool_offsets_present_flag is equal to 1, the values of chroma DB offset syntax elements in the PH and SH, when not present, are inferred to be equal to the chroma DB offsets in the PPS and PH respectively.

- Otherwise, (pps_chroma_tool_offsets_present_flag is equal to 0), the values of the chroma DB offset syntax elements are inferred to be equal to the luma DB offsets in the same parameter set or header (same infererence rule as the current VVC).

As an alternative solution, the authors considered to infer the chroma DB offsets in the PH and SH to be equal to the values of the chroma DB offsets in the PPS and PH respectively when they are not present, but this solution is not recommended.

Proposal 2 changes the semantics of pps_chroma_tool_offsets_present_flag such that when the flag is equal to 1, the chroma tool offsets related syntax elements are present in the PPS RBSP syntax structure and the chroma deblocking tc and β offset syntax elements may be present in the PHs or the SHs of pictures referring to the PPS. The current draft uses "are" instead of "may be" above, and it is asserted by the proponents that there is a case when the offset syntax elements are not present in the PHs or the SHs of pictures referring to the PPS even though pps_chroma_tool_offsets_present_flag is equal to 1.

JVET-S0062 AHG9: On no_tsrc_constraint_flag [K. Naser, F. Le Leannec, T. Poirier, P. de Lagrange (InterDigital)]

In VVC draft 9, a new constraint flag "no_tsrc_constraint_flag" for defining a profile/sub-profiles where only regular residual coding (RRC) is used, i.e., transform skip residual coding (TSRC) is disabled. It is observed that when transform skip is disabled (sps_transform_skip_enabled_flag is 0), this constraint flag cannot be 1 since its corresponding SH flag (sh_ts_residual_coding_disable_flag) is inferred to be 0. Thus, although transform skip is never used, TSRC cannot be disabled by its SH flag. When "no_tsrc_constraint_flag" is one, both dependent quantization and sign data hiding cannot be used, but they can still be activated by their SPS flag. Therefore, this contribution proposes methods to well-define "no_tsrc_constraint_flag" and properly signal SPS values for such a profile. Finally, it proposes to set the value of no_tsrc_constraint_flag to one when no_transform_skip_constraint_flag is 1.

JVET-S0072 AHG9: On PDPC and reference sample filtering of non-420 sequences [M. G. Sarwer, Y. Ye (Alibaba)]

PDPC and reference sampling filtering process are very efficient coding tool for capturing the characteristic of the natural camera capture content. However, it is observed that PDPC and reference filter may produce some visual artifacts for screen content sequences. This contribution proposes to add an SPS level flag to allow encoder flexibility to disable PDPC and reference filtering process for content (such as text, graphics, computer screen) where filtering and PDPC do not help.

It is asserted that disabling PDPC and reference filtering can reduce some visual artifacts especially for 444 content. It is also reported that as compared to VTM-8.0, disabling PDPC and reference filter can achieve -0.20% (Y), -0.35% (U), -0.38% (V) coding gain of YUV-444 TGM sequences.

JVET-S0069 AHG9: General constraint information on features related to transform skip mode [S.-T. Hsiang, O. Chubach, L. Chen, Y.-W. Huang, S.-M. Lei (MediaTek)]

Aspect 2 of this contribution belongs to this category.

This contribution proposes following modifications to the general constraint information of features related to transform skip mode:

- Mandate the value of no_bdpcm_constraint_flag to be equal to 1 when no_transform_skip_constraint_flag is equal to 1.

- Fix the spec bug by having no_tsrc_constraint_flag constrain sh_ts_residual_coding_disabled_flag to be equal to 1 (#1071). Modify the constraint of no_tsrc_constraint_flag such that sps_transform_skip_enabled_flag equal to 0 satisfies the constraint imposed by no_tsrc_constraint_flag equal to 1. Mandate the value of no_tsrc_constraint_flag to be equal to 1 when no_transform_skip_constraint_flag is equal to 1.

JVET-S0073 AHG9: Bug-fix of the constraint flag of VVC [M. G. Sarwer, Y. Ye (Alibaba)]

This contribution proposes two aspects.

The first aspect of this contribution proposes a bug-fix to the  no_aps_constraint_flag. In the current design, ALF and CCALF process can be enabled when the value of no_aps_constraint_flag is equal to 1. However, it is asserted that the chroma ALF and CCALF process cannot be executed without signaling of filter sets in APS. The aspect 1 of this contribution proposes to disable chroma ALF and CCALF when the value of no_aps_constraint_flag is equal to 1. Three options are proposed. In the first option, a semantic constraint is applied to the no_aps_constraint_flag to disable CCALF and chroma ALF. In the second option, a new SPS level flag is introduced to control chroma ALF process and the new SPS flag is disabled if no_aps_constraint_flag == 1. In the third option, both ALF and CCALF processes are disabled in SPS level.

The second aspect is related to a bug-fix of the no_tsrc_constraint_flag. In VVC draft 9, TS residual coding can only be disabled if both dependent quantization (DQ) and sign data hiding (SDH) are disabled. However, in current design, DQ or SDH can be enabled independent of the value of no_tsrc_constraint_flag. When no_tsrc_constraint_flag == 1 and DQ or SDH is enabled,  there is no way to actually disable TSRC. Aspect 2 of this contribution proposes to apply semantic constraint to disable DQ and SDH when no_tsrc_constraint_flag is equal to 1. Three options are proposed. In the first option, the DQ and SDH are disabled in SPS level; whereas in the second option, DQ and SDH are disabled in slice level. In the third option, transform-skip mode is disabled, which is equivalent to removing no_tsrc_constraint_flag.

JVET-S0088 AHG9: Clean-up on derivation of the MaxNumSubblockCand [N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)]

In the previous meeting, JVET-R0371 was adopted to avoid the disallowing the SbTMVP when both Affine and SbTMVP are enabled. It was suggested to set the sps_five_minus_max_num_subblock_merge_cand in range of 0 to 5 – sps_sbtmvp_enabled_flag so that the number of the subblock merge candidate is set to be 1 at least when sps_sbtmvp_enabled_flag is equal to 1. However, SbTMVP is controlled by not only sps_sbtmvp_enabled_flag but ph_temporal_mvp_enabled_flag in PH. Hence, the intention of the JVET-R0371 for ensuring the minimum one candidate for SbTMVP was not precisely reflected. Therefore, in this proposal, the derivation method of the number of the subblock merge candidates is further cleaned-up by reflecting sps_sbtmvp_enabled_flag and ph_temporal_mvp_enabled_flag simultaneously.

JVET-S0089 AHG8: On TMVP derivation using inter-layer reference picture [N. Park, J. Nam, H. Jang, J. Lim, S. Kim(LGE)]

In the current VVC specification, inter layer reference picture (ILRP) which is marked as a long-term reference is allowed to be a reference picture of current picture or collocated picture for temporal motion vector prediction (TMVP). When ILRP and LTRP of which nuh_layer_id is equal to nuh_layer_id of the current picture are used together, undesirable behavior is observed. In addition, the combination of LTRP and ILRP is not commonly used compared to the STRP and ILRP case. In this contribution, three solutions are proposed to address TMVP behavior more clearly when ILRP is used as a reference picture for TMVP.

Solution 1: Disabling TMVP when either RefPic or ColRefPic is ILRP, but not both.

Solution 2: Enabling TMVP with combination of STRP and ILRP on RefPic or ColRefPic, and disabling TMVP with combination of LTRP and ILRP.

Solution 3: Enabling TMVP when either RefPic or ColRefPic is ILRP.

JVET-S0093 On slice header flags related to residual coding [J. Samuelsson, S. Deshpande, F. Bossen, A. Segall (Sharp)]

This contribution presents a proposal for a modification to the High-Level Syntax of the VVC draft to avoid signalling redundant information in slice header in many scenarios that are envisioned to be common use cases of VVC. The proposal is to bring back picture level control of Dependent Quantization (DQ) and Sign Data Hiding (SDH) and add an SPS level flag (sps_residual_control_present_in_sh_flag) to control the presence of the three slice header syntax elements sh_dep_quant_enabled_flag, sh_sign_data_hiding_enabled_flag, and sh_ts_residual_coding_disabled_flag. It is further proposed to change the gating conditions for the slice header elements and adjust the inference rule to disable support for undesired combinations at the slice level. No changes are proposed to the decoding process or to any syntax tables below slice header level. The complete suggested text changes are included as an attachment to this contribution. A slide deck with further details of the proposal and highlighted text changes is also provided as an attachment to this contribution as well as a software patch for the proposal relative to VTM master (commit 8d9573f1ec214868e1a4e61e38991c5a07b86449).

JVET-S0105 AHG9: Modification of general constraint information [S. McCarthy, P. Yin, T. Lu, F. Pu, W. Husak, T. Chen (Dolby)]

The aspect on changing the semantics of no_tsrc_constraint_flag belongs to this category.

The semantcis of no_tsrc_constraint_flag is proposed to be changed to:

no_tsrc_constraint_flag equal to 1 specifies that sh_ts_residual_coding_disabled_flag shall be equal to 1 instead of 0. no_tsrc_constraint_flag equal to 0 does not impose such a constraint.

And add "When no_transform_skip_constraint_flag is equal to 1, the value of no_tsrc_constraint_flag shall be equal to 1. When not present, no_tsrc_constraint_flag is inferred to be equal to 0."

JVET-S0121 AHG9: On deblocking filter, ALF, and SAO enabling flags in PH and SH [N. Hu, V. Seregin, Y. He, M. Karczewicz (Qualcomm)]

High level control flags of ALF and SAO can be present in either PH or SH. When an ALF or SAO enabling flag is not present in PH, it is inferred to be equal to 0. According to the semantics, the related tool is disabled and not used for a current picture. However, in a slice header, the corresponding enabling flag can be present and may be equal to 1. Then, the tool is enabled and may be used for a current slice, which contradicts the inference rule for non present flag in PH.

This contribution proposes to modify the inference rules of non-present syntax elements to relieve the conflicts.

In the second aspect, whether deblocking filter control information is present in a PH is simplified by being controlled by pps_dbf_info_in_ph_flag only.

JVET-S0130 On chroma QP mapping [L. Li, X. Li, B. Choi, S. Wenger, S. Liu (Tencent)]

The contribution proposes following changes:

Aspect 1: bugfix for the range of sps_num_points_in_qp_table_minus1[ i ]

Aspect 2: introduce sps_chroma_qp_identical_to_luma_flag in SPS to have a shortcut when chroma quantization parameter are set equal to luma quantization parameter. It is asserted that this shortcut is the common case for 4:2:2 and 4:4:4, and substantive bits can be saved by skipping all the chroma QP mapping related syntax elements in SPS.

JVET-S0134 AHG9/AHG12: Relaxing an ALF APS constraint [A. Aminlou, M. M. Hannuksela (Nokia)]

VVC Draft 9 includes a constraint that the content of all APS NAL units with a particular value of adaptation_parameter_set_id and a particular value of aps_params_type within a picture unit shall have the same content. Furthermore, number of ALF APSs is limited to 8. This contribution asserts that, because of this constraint, ALF needs to be turned off in many applications using adaptive subpicture-based delivery or decoding. To enable the use of ALF in such applications, the contribution proposes the following limitations in a specific case indicated by an SPS flag:

- Limit the number of ALF APSs to one for each slice.

- Limit the number of luma and chroma ALF filters to one for each ALF APS.

- Disable CCALF.

JVET-S0136 AHG9: On signaling of PDPC enabling/disabling flag in SPS [X. Xiu, Y.-W. Chen, H.-J. Jhu, T.-C. Ma, X. Wang (Kwai)]

This contribution proposes to introduce one control flag in sequence parameter set (SPS) to adaptively enable/disable position dependent intra prediction combination (PDPC). When the flag is off, all the decoding process of the PDPC are bypassed. Additionally, another constraint flag no_pdpc_constraint_flag is added to general constraint syntax table.