| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0146 |
| --- | --- |

| Title: | Status Report on 360º video Verification Test Preparation |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Mathias Wien Vittorio Baroncini Yan Ye | Tel: Email: | wien@lfb.rwth-aachen.de baroncini@gmx.com yan.ye@alibab-inc.com |
| Source: | Verification Test Coordinators |  |  |

_____________________________

# Introduction

This document summarizes the progress made to the preparation of 360º video verification testing.

# Encoding of the 360 video content

According to JVET-R2009, nine candidate 360º video sequences in Table 1 are to be encoded over the QP range of QP=24 to 50 at step of 1 for HM and QP=26 to 50 at step of 4 for VTM. In JVET-R2009, it was planned to use the following projection formats with coding resolutions as provided in Table 2:

- For HM, the padded equi-rectangular (PERP) format and the cubemap (CMP) format

- for VTM, the PERP format and the padded hybrid equi-angular cubemap (PHEC) format

However, while preparing for encoding, it was agreed that the CMP format for HM encoding should also use padding, and that instead of the PHEC format, the generalized cubemap (GCMP) is a more appropriate format to use for VTM encoding. Further investigation revealed that the 360Lib-5.0 software to be used together with HM had the following issues:

- 360Lib-5.0 did not have proper padding for the cubemap format

- The chroma sampling issue that was proposed and adopted in JVET-L0238 was implemented only in 360Lib version 8 and on, but not implemented in 360Lib-5.0

- 360Lib-5.0 was only available in combination with HM16.16, and combination with HM16.20 was not available

RWTH and MediaTek volunteers acted quickly to fix the above issues, and had provided patched version of 360Lib 5.0 for the encoding volunteers to use.

Because of the discussions about the suitable projection formats, and software issues that took time to fix, encoding was completed slightly behind schedule, by May 12th (versus the planned date of May 8th).

In the complete set of HM encodings, HM-16.20 was used in combination with the patched 360Lib-5.0 with three projection formats, PERP, CMP, and the padded cubemap (PCMP) format (i.e. one more format than documented in JVET-R2009).

In the complete set of VTM encodings, HM-8.0 was used in combination with 360Lib-10.1 with three projection formats, PERP, PHEC, and GCMP (i.e. one more format than documented in JVET-R2009).

The coded YUV resolution for each project format used in HM and VTM encoding is listed in Table 3.

It is hereby suggested to merge the patch into HM 360Lib main trunk.

Table 1 – 360 video test sequences

| No. | Sequence | Frame rate | Resolution | Provider | BD | Camera |
| --- | --- | --- | --- | --- | --- | --- |
| 01 | skateboarding | 60 | 4096x2048 | GoPro | 8 | static |
| 02 | GT_Sheriff | 30 | 4320x2160 | Nokia | 8 | static |
| 03 | basketball | 30 | 8192x4096 | GoPro | 10 | static |
| 04 | jam_session | 30 | 8192x4096 | GoPro | 8 | static |
| 05 | SkateboardTrick | 60 | 8192x4096 | GoPro | 8 | static |
| 06 | Train | 60 | 8192x4096 | GoPro | 8 | static |
| 07 | SkateBoardAtBridge | 30 | 6144x3072 | GoPro | 8 | moving |
| 08 | HarborBiking | 30 | 8192x4096 | InterDigital | 8 | moving |
| 09 | KiteFliteWalking | 30 | 8192x4096 | InterDigital | 8 | moving |

Table 2 – Coding resolution for different input resolutions and coding projection formats

|  | CMP-like (before padding) | ERP (before padding) |
| --- | --- | --- |
| 4K (4096x2048 & 4320x2160) | 1184 x 1184 per face --CodingFaceWidth=1184 --CodingFaceHeight=1184 | 4096x2048 --CodingFaceWidth=4096 --CodingFaceHeight=2048 --WrapAroundOffset=4096 (only for VTM) |
| 6K & 8K | 1280 x 1280 per face --CodingFaceWidth=1280 --CodingFaceHeight=1280 | 4432x 2216 --CodingFaceWidth=4432 --CodingFaceHeight=2216 --WrapAroundOffset=4432 (only for VTM) |

Table 3 – Coded YUV resolution for each projection format in HM and VTM encoding

| Source Resolution | HM encoding |  |  | VTM encoding |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | CMP | PCMP | PERP | PHEC | GCMP | PERP |
| 4K (4096x2048 & 4320x2160) | 3552x2368 | 3568x2400 | 4112x2048 | 3552x2368 | 3568x2400 | 3568x2400 |
| 6K & 8K | 3840x2560 | 3856x2592 | 4448x2216 | 3840x2560 | 3856x2592 | 4448x2216 |

## Encoding results

Rate distortion plots showing the E2E-WSPSNR of the Y component vs. bit rate are provided for each sequence in the following.

# Viewport selection

Viewport selection volunteers got together on a conference call on May 20, 2020 to look at the viewport video without coding. Main issues discussed are as follows:

- The stitching artefacts in the original content were discussed. For example, for HarborBiking, the content identified in the viewport video suggested by J. Boyce was considered quite nice, except that some stitching artefacts from the original content exist in the viewport video. It was noted that uncoded video containing such artefacts will be shown to the subjects during viewing, and that subjects could also be trained to learn about them. In a follow-up discussion with the MPEG test chair, it was confirmed that such stitching artefacts is not of significant concern.

- Viewports of a few candidate sequences were refined, e.g., to avoid very fast motion in the viewport video, to capture larger objects/more interesting parts of the video, to capture parts of the video more likely to contain seam artefacts, and/or to use dynamic rather than static viewport. Refined viewport video was uploaded and reviewed after the call. Annex A provides further information about the dynamic viewport path recommended for seven candidate sequences, and the corresponding viewport config files are attached to this contribution.

- Two candidate sequences, Train and Skateboarding, were identified as having lower priority for the purpose of subjective viewing, since quality of the original uncoded content seemed to be unsatisfactory.

# Future work

The following work remains to be completed:

- Selection of HEVC and VVC rate points and additional encoding if needed;

- Viewing of the encoded video at selected rate points and viewports, including further fine-tuning of the viewports if needed;

- In terms of encoding refinement, another thread of active discussion is about using the subpicture functionality to encode the VVC bitstreams in the GCMP format. The specific suggestion is to use two subpictures, with each face row in a separate subpicture, and treating sub-picture boundaries as picture boundaries. Volunteers are looking at subpicture config (e.g. layout and dimension) for each of the candidate test sequences. Once this is completed, we will run the VTM with subpicture config together with 360Lib to ensure that encoding, decoding and metric calculation is correctly performed (or fix software if problems are encountered). After subpicture encoding is completed, we will compare the results between subpicture on and off and select the one with better performance.

# Acknowledgements

The test coordinators would like to the following volunteers for their hard work:

Jill Boyce (Intel) for providing guidance on how viewport selection was conducted in the CfE and CfP time, and helping to select the viewports for the candidate sequences;

Johannes Sauer (RWTH) and Louise Lee (MediaTek) for patching 360Lib-5.0 as needed such that HM encoding can be performed correctly, preparing config files, and doing multiple rounds of encoding;

Edouard Francois (InterDigital), Ru-Ling Liao (Alibaba) and Yang Wang (ByteDance) for doing multiple rounds of encoding.

# ANNEX: Visualization of Dynamic Viewports

The plots below show the proposed viewports for 360 video test sequences under consideration. The starting point is marked with a red circle. The progression of longitude and latitude values along the frames of the test sequences are plotted in blue.

## Basketball

## GT_Sheriff

## HarbourBiking

## JamSession

## KiteFliteWalking

## SkateBoardAtBridge

- Static viewport with (0°,0°):

- Almost static viewport with 0 longitude and little elevation in latitude:

## SkateboardTrick