| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0148 |
| --- | --- |

| Title: | Crosscheck of JVET-S0093 (On slice header flags related to residual coding) |  |  |
| --- | --- | --- | --- |
| Status: | Input document |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | J. Gan Building A, 5 Talavera Rd, Macquarie Park, 2113, NSW, Australia | Tel: Email: | +61 2 9805 2844 jonathan.gan@canon.com.au |
| Source: | Canon Australia Pty Ltd |  |  |

_____________________________

# Abstract

This contribution is a cross-check report of JVET-S0093 [1].

# Text analysis

In the current working draft of VVC [2] when either dependent quantization or sign data hiding is enabled in a slice, then transform skip residual coding (TSRC) cannot be disabled. This restriction is imposed so that efficient coding of camera-captured content is supported in a mixed lossy/lossless encoding use case. The use of DQ/SDH/TSRC must be signalled by 1 or 2 bits in each slice header.

JVET-S0093 proposes adding an SPS flag, as well as restoring the picture header flags for DQ and SDH. The signalling is modified such that when the SPS flag is disabled, a common case (TSRC not disabled, DQ/SDH following the picture header flags) is supported with 0 bits signalled in the slice header. Other cases, including the mixed lossy/lossless use case described above are supported with 1 bit signalled per slice header.

In terms of performance, JVET-S0093 is identical to input document JVET-R0486 [3]. Both proposals consist of an SPS-level flag, and PH-level flags controlling DQ and SDH. The difference between the two proposals at slice level is that JVET-S0093 signals the slice-level DQ/SDH flags first, then signals slice-level TSRC disable flag conditioned on the previous flags. In contrast, JVET-R0486 signals the slice-level TSRC disable flag first. After signalling this flag, the value of slice-level DQ/SDH can always be inferred (never signalled) so such flags do not exist as syntax elements. Instead, they are expressed as internal variables.

The crosschecker believes that R0486 is a cleaner design, as only one slice-level flag (TSRC disable) is signalled in the bitstream. In S0093 only one slice-level bit is signalled in the bitstream in any particular slice, but it may be interpreted as the DQ enable flag, SDH enable flag, or the TSRC disable flag, depending on conditions imposed by higher level flags.

# Software analysis

The software patch has not been simulated. It has been inspected and found to only contain high-level changes.

# References

- J. Samuelsson, S. Deshpande, F. Bossen, A. Segall, “On slice header flags related to residual coding”, JVET-S0093, 19th Meeting: by teleconference, 15–24 April 2020.

- B. Bross, J. Chen, and S. Liu “Versatile Video Coding (Draft 9),” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-R2001, 18th Meeting: by teleconference, 15–24 April 2020.

- J. Samuelsson, S. Deshpande, F. Bossen, A. Segall, T. Hashimoto, E. Sasaki, T. Aono, T. Ikai, A. Nalci, H.E. Egilmez, M. Karczewicz, S. T. Hsiang, C. W. Hsu, Z. Y. Lin, T. D. Chuang, O. Chubach, C. Y. Chen, Y. W. Huang, S. M. Lei, M. G. Sarwer, “On TSRC, DQ and SDH signalling”, JVET-R0486, 18th Meeting: by teleconference, 15–24 April 2020.