| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0153 |
| --- | --- |

| Title: | Agenda and Report of the AHG4 Meeting on the HDR Verification Test on 2020-05-29 (Draft) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Andrew Segall Mathias Wien Vittorio Baroncini Teruhiko Suzuki | Tel: Email: | asegall@sharplabs.com wien@lfb.rwth-aachen.de baroncini@gmx.com Teruhiko.S@sony.com |
| Source: | AHG |  |  |

_____________________________

# Agenda and Notes

The meeting was held online on Friday, 2020-05-29, 5:00h UTC with the connection information provided in the JVET meeting calendar.  The agenda was:

- Review of the current progress of known side activity

  - Contribution JVET-S0151

- Discussion of sequence selection for further study

  - Goal of confirming the subset during this call.

- Discussion and planning for performing (at least) one round of expert viewing.

  - Goal to finalize the plan and conduct the first round prior to the upcoming JVET meeting and using consumer equipment

| JVET-S0151 | m54150 | 2020-05-29 06:00:53 | 2020-05-29 06:03:50 | 2020-05-29 06:03:50 | Status Report and Proposed Agenda for the AHG4 Meeting on HDR Verification Test Preparation | A. Segall (Sharp) |
| --- | --- | --- | --- | --- | --- | --- |

There was discussion on the target bit-rate of applications of interest.  One participant suggested that 25Mbps would be a reasonable, maximum target.  Another participant voiced support for this statement.

It was suggested that the finest QP for the HLG content should be between 24 and 30 of the HM.  This was supported by another participant.

The 10 test sequences suggested in JVET-S0151 were confirmed to be considered as candidates for further visual inspection. It was suggested to also add a 6th rate point for the HM that was approximately rate matched with the range of qualities in the VTM QP points.

It was further suggested to attempt to add additional QP points to rate match 3-4 points of the VTM bit-rates.

It was later suggested that a 5th rate point may be sufficient.  However, it was encouraged to create additional rate points if possible.

One participant comment that using an LG C9 OLED display may be an appropriate and available display for remote viewing. Another participant suggested than any LG HDR OLED display could be acceptable.

One participant commented that remote viewing may require calibration across the different test sites.  Any calibration should be defined in the test plan.

One participant commented that using different display types and models may be problematic, as different displays often have different visual quality.

One participant suggested that using both OLED and LCD display could desirable.  Another participant commented that we may not have enough people to perform testing with both display types.

One participant suggested that using a professional display would be most desirable.

One participant suggested generating tone mapped versions (1k nit) of previous, well studied sequences.  This was supported by several participants in the group.

As a general direction, the following was recommended by the AHG:

- Invite those participants with consumer HDR displays to participate in the expert viewing

- Treat the LG display as a ‘reference’ display among the consumer displays

- Defer the decision of using a consumer or professional display for a later stage

- Prepare tone mapped versions (1k nit) of previous, well known sequences as calibration sequences

For the QP selection process, it was commented that using the internal decoder of the consumer display could be acceptable.  However, verification testing would use the HDMI input.

Potential workflow

- Chroma format – use chroma type of original sequences for encoding.  It would be desirable to use a consistent chroma format for all test sequences during the final verification test.

- Rendering – apply linear tone-mapping as needed for each sequence.  Reference the original mastering target to determine the tone-mapping parameter.

- Display – drive display with HDMI input and HDR meta-data.  (Using internal decoder would be acceptable for initial QP selection.)

- Comment – it would be desirable to have the workflow codified with one or more scripts.

It was asked if the available content was in the desirable range.  Or, if some content type was missing.  It was suggested by one participant than including mixed content or gaming content could be desirable in a future test.

The meeting closed at approximately 6:55h UTC.