| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0155-v1 |
| --- | --- |

| Title: | AHG9: On EOS NAL units |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 (Bytedance) 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes on end of sequence (EOS) NAL units:

- In clause 3 (Definitions), the NOTE as part of the definition of CLVS picture, the phrase "the first PU in the layer of the bitstream that follows an EOS NAL unit in decoding order", change "EOS NAL unit" to "EOS NAL unit in the layer".

- In clause 7.4.2.2 (NAL unit header semantics), instead of requiring the nuh_layer_id of an EOS NAL unit to be equal to the nuh_layer_id of the associated VCL NAL unit, it is specified that the nuh_layer_id of an EOS NAL unit shall be equal to one of the nuh_layer_id values of the layers present in the CVS.

- In clause 7.4.2.4.3 (Order of PUs and their association to AUs), it is specified that, when present, the next PU of a particular layer after an EOS NAL unit that belongs to the same layer shall be an IRAP or GDR PU.

- In clause 7.4.2.4.4 (Order of NAL units and coded pictures and their association to PUs), it is specified that, when an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units within the PU other than other EOS NAL units (when present) or an EOB NAL unit (when present).

- In clause 7.4.3.10 (End of sequence RBSP semantics), it is specified that, when present, the EOS RBSP specifies that the next subsequent PU that belongs to the same layer as the EOS NAL unit in the bitstream in decoding order (if any) is an IRAP or GDR PU.

# Introduction

In clause 7.4.2.2 (NAL unit header semantics), it is specified that, when nal_unit_type is equal to EOS_NUT, nuh_layer_id shall be equal to the nuh_layer_id of associated VCL NAL unit. However, this does not fully enable temporal scalability operations, e.g., extraction of a temporal subset of a temporally scalable bitstream while keeping the EOS NAL units for each layer in the extraction output.

For example, assuming there are two layers with nuh_layer_id equal to 0 and 1, and each layer has two sublayers with TemporalId equal to 0 and 1. In AU n with n greater than 0 and TemporalId equal to 1, there is an EOS NAL unit in each PU, and the two EOS NAL units have nuh_layer_id equal to 0 and 1. And note that any EOS NAL units are required to have TemporalId equal to 0. Through an extraction process that keeps only the lowest sublayer in each layer, NAL units with TemporalId equal to 1 would be removed, and consequently, both of the EOS NAL units that were in AU n would become part of the PU with nuh_layer_id equal to 1 in AU n − 1. This would then violate the rule that the nuh_layer_id of an EOS NAL unit shall be equal to the nuh_layer_id of the associated VCL NAL unit.

Therefore, it is proposed to allow the value of the nuh_layer_id of an EOS NAL unit to be different from the nuh_layer_id of the associated VCL NAL unit, and allow one PU to contain more than one EOS NAL unit.

# Proposal

The proposed changes for allowing the value of the nuh_layer_id of an EOS NAL unit to be different from the nuh_layer_id of the associated VCL NAL unit, and for allowing one PU to contain more than one EOS NAL unit, and consequently other changes, are summarized as in the abstract above

The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### 3	Definitions

...

coded layer video sequence (CLVS): A sequence of PUs with the same value of nuh_layer_id that consists, in decoding order, of a CLVSS PU, followed by zero or more PUs that are not CLVSS PUs, including all subsequent PUs up to but not including any subsequent PU that is a CLVSS PU.

NOTE – A CLVSS PU may be an IDR PU, a CRA PU, or a GDR PU. The value of NoOutputBeforeRecoveryFlag is equal to 1 for each IDR PU, and each CRA PU that has HandleCraAsClvsStartFlag equal to 1, and each CRA or GDR PU that is the first PU in the layer of the bitstream in decoding order or the first PU in the layer of the bitstream that follows an EOS NAL unit in the layer in decoding order.

...

#### 7.4.2.2	NAL unit header semantics

...

nuh_layer_id specifies the identifier of the layer to which a VCL NAL unit belongs or the identifier of a layer to which a non-VCL NAL unit applies. The value of nuh_layer_id shall be in the range of 0 to 55, inclusive. Other values for nuh_layer_id are reserved for future use by ITU-T | ISO/IEC.

The value of nuh_layer_id shall be the same for all VCL NAL units of a coded picture. The value of nuh_layer_id of a coded picture or a PU is the value of the nuh_layer_id of the VCL NAL units of the coded picture or the PU.

When nal_unit_type is equal to PH_NUT, EOS_NUT, or FD_NUT, nuh_layer_id shall be equal to the nuh_layer_id of the associated VCL NAL unit.

When nal_unit_type is equal to EOS_NUT, nuh_layer_id shall be equal to one of the nuh_layer_id values of the layers present in the CVS.

NOTE 1 – The value of nuh_layer_id for DCI, VPS, AUD, and EOB NAL units is not constrained.

...

#### 7.4.2.4.3	Order of PUs and their association to AUs

...

It is a requirement of bitstream conformance that, when present, the next PU of a particular layer after a PU an EOS NAL unit that belongs to the same layer and contains an EOS NAL unit shall be an IRAP or GDR PU a CLVSS PU, which is either an IRAP PU with NoOutputBeforeRecoveryFlag equal to 1 or a GDR PU with NoOutputBeforeRecoveryFlag equal to 1.

...

#### 7.4.2.4.4	Order of NAL units and coded pictures and their association to PUs

A PU consists of zero or one PH NAL unit, one coded picture, which comprises of one or more VCL NAL units, and zero or more other non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.5.

When a picture consists of more than one VCL NAL unit, a PH NAL unit shall be present in the PU.

When a VCL NAL unit has sh_picture_header_in_slice_header_flag equal to 1 or is the first VCL NAL unit that follows a PH NAL unit, the VCL NAL unit is the first VCL NAL unit of a picture.

The order of the non-VCL NAL units (other than the AUD and EOB NAL units) within a PU shall obey the following constraints:

–	When a PH NAL unit is present in a PU, it shall precede the first VCL NAL unit of the PU.

–	When any DCI NAL units, VPS NAL units, SPS NAL units, PPS NAL units, prefix SEI NAL units, NAL units with nal_unit_type equal to RSV_NVCL_26, or NAL units with nal_unit_type in the range of UNSPEC_28..UNSPEC_29 are present in a PU, they shall not follow the last VCL NAL unit of the PU.

–	When any DCI NAL units, VPS NAL units, SPS NAL units, or PPS NAL units are present in a PU, they shall precede the PH NAL unit (when present) of the PU and shall precede the first VCL NAL unit of the PU.

–	NAL units having nal_unit_type equal to SUFFIX_SEI_NUT, FD_NUT, or RSV_NVCL_27, or in the range of UNSPEC_30..UNSPEC_31 in a PU shall not precede the first VCL NAL unit of the PU.

–	When any prefix APS NAL units are present in a PU, they shall precede the first VCL unit of the PU.

–	When any suffix APS NAL units are present in a PU, they shall follow the last VCL unit of the PU.

–	When an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units within the PU other than other EOS NAL units (when present) or an EOB NAL unit (when present).

#### 7.4.3.10	End of sequence RBSP semantics

When present in a bitstream, an EOS NAL unit is considered belonging to or being in the layer that has nuh_layer_id equal to the nuh_layer_id of the EOS NAL unit.

When present, the EOS RBSP specifies that the current PU is the last PU in the CLVS in decoding order and the next subsequent PU that belongs to the same layer as the EOS NAL unit in the bitstream in decoding order (if any) is an IRAP or GDR PU.

The syntax content of the SODB and RBSP for the EOS RBSP are empty.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).