| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0156-v1 |
| --- | --- |

| Title: | AHG9: On level definitions and bitstream conformance |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 (Bytedance) 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes related to the specifications of level definitions and bitstream conformance:

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices i.e., items c and d, from being based on MaxSlicesPerPicture and the maximum picture size to be based on MaxSlicesPerPicture * ( the number of pictures in the AU ) and the maximum AU size.

- In clause A.4.2 (Profile-specific level limits), change the definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles i.e., items i and j, from being based on MaxTileCols * MaxTileRows and the maximum AU size to be based on MaxTileCols * MaxTileRows * ( the number of pictures in the AU ) and the maximum AU size.

- In clause C.4 (Bitstream conformance), the 6th constraint, change "CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ]" to "DpbOutputTime[ n ] greater than DpbOutputTime[ currPic ]". This is asserted to be a bug, because all decoded pictures in the DPB are always decoded earlier than decoding of the current picture, and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ]. This is also asserted to be a bug in the latest HEVC spec.

# Introduction

The following issues exist in the specifications of level definitions and bitstream conformance:

- The definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of slices, i.e., items c and d in clause A.4.2 (Profile-specific level limits), are based on MaxSlicesPerPicture and the maximum picture size. However, MaxSlicesPerPicture is defined as a picture-level limit, while the CPB removal times are AU-level parameters.

To match between them, either MaxSlicesPerPicture should be replaced with MaxSlicesPerAu (as suggested for further study by the editor's notes) or change these two items to be based on MaxSlicesPerPicture * ( the number of pictures in the AU ) and the maximum AU size.

This contribution proposes to take the latter approach, which seems easier.

- The definitions of the two level limits on the relationship between the CPB removal times for AU 0 and for AU n ( n > 0 ) and the number of tiles, i.e., items i and j in clause A.4.2 (Profile-specific level limits), are based on MaxTileCols * MaxTileRows and the maximum AU size. However, similarly as above, MaxTileCols and MaxTileRows are defined as a picture-level limits, while the CPB removal times are AU-level parameters.

To match between them, either redefine MaxTileCols and MaxTileRows to be AU-level limits (as suggested for further study by the editor's notes) or change these two items to be based on MaxTileCols * MaxTileRows * ( the number of pictures in the AU ) and the maximum AU size.

Again, this contribution proposes to take the latter approach, which seems easier.

- The 6th constraint in cause C.4 (Bitstream conformance) is as follows:

For each current picture, after invocation of the process for removal of pictures from the DPB as specified in clause C.3.2, the number of decoded pictures in the DPB, including all pictures n that are marked as "used for reference", or that have PictureOutputFlag equal to 1 and CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to max_dec_pic_buffering_minus1[ Htid ].

The part "CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ]" describes a condition that the decoding time of a decoded picture in the DPB is less than the decoding time of the current picture. However, all decoded pictures in the DPB are always decoded earlier than decoding of the current picture and thus CpbRemovalTime[ n ] in the context is always less than CpbRemovalTime[ currPic ].

The intent should be to specify a condition that a decoded picture in the DPB has output time greater than the decoding time of current picture, as when decoding of the current picture, only decoded pictures that are needed for reference or still need to be output stay in the DPB, other earlier decoded pictures would be removed from the DPB. Therefore, the constraint is changed to reflect such an intent.

It should be noted that this issue also exists in the latest HEVC specification.

# Proposal

The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### A.4.2	Profile-specific level limits

The following is specified for expressing the constraints in this annex:

- Let the variable fR be set equal to 1  300.

The variable HbrFactor is defined as follows:

- If the bitstream is indicated to conform to the Main 10 profile or the Main 4:4:4 10 profile, HbrFactor is set equal to 1.

The variable BrVclFactor, which represents the VCL bit rate scale factor, is set equal to CpbVclFactor * HbrFactor.

The variable BrNalFactor, which represents the NAL bit rate scale factor, is set equal to CpbNalFactor * HbrFactor.

The variable MinCr is set equal to MinCrBase * MinCrScaleFactor ÷ HbrFactor.

When the specified level is not level 15.5, the value of max_dec_pic_buffering_minus1[ Htid ] + 1 shall be less than or equal to MaxDpbSize, which is derived as follows:

if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  2 ) )
	MaxDpbSize = Min( 4 * maxDpbPicBuf, 16 )
else if( PicSizeMaxInSamplesY  <=  ( MaxLumaPs  >>  1 ) )
	MaxDpbSize = Min( 2 * maxDpbPicBuf, 16 )	(A.1)
else if( PicSizeMaxInSamplesY  <=  ( ( 3 * MaxLumaPs )  >>  2 ) )
	MaxDpbSize = Min( ( 4 * maxDpbPicBuf ) / 3, 16 )
else
	MaxDpbSize = maxDpbPicBuf

where MaxLumaPs is specified in Table A.1, maxDpbPicBuf is equal to 8, and max_dec_pic_buffering_minus1[ Htid ] is found in or derived from the applicable dpb_parameters( ) syntax structure.

Let numPics[ n ]numDecPics be the number of pictures in AU n. The variable AuSizeMaxInSamplesY[ n ] is set equal to PicSizeMaxInSamplesY * numPics[ n ]numDecPics.

Bitstreams conforming to the Main 10 or Main 4:4:4 10 profile at a specified tier and level shall obey the following constraints for each bitstream conformance test as specified in Annex C:

- The nominal removal time of AU n (with n greater than 0) from the CPB, as specified in clause C.2.3, shall satisfy the constraint that AuNominalRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n − 1 ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 that applies to AU n − 1.

- The difference between consecutive output times of pictures of different AUs from the DPB, as specified in clause C.3.3, shall satisfy the constraint that DpbOutputInterval[ n ] is greater than or equal to Max( AuSizeMaxInSamplesY[ n ]  MaxLumaSr, fR ), where MaxLumaSr is the value specified in Table A.2 for AU n, provided that AU n has a picture that is output and AU n is not the last AU of the bitstream that has a picture that is output.

- The removal time of AU 0 shall satisfy the constraint that the number of slices in each picture in AU 0 is less than or equal to Min( Max( 1, MaxSlicesPerPicture * numPics[ 0 ] * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxSlicesPerPicture * numPics[ 0 ] * AuSizeMaxInSamplesY[ 0 ] PicSizeMaxInSamplesY / MaxLumaPs ), MaxSlicesPerPicture * numPics[ 0 ] ), for the value of PicSizeMaxInSamplesY of picture 0, where MaxSlicesPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2, respectively, that apply to AU 0. [Ed (YK). Further study on whether MaxSlicesPerPicture used herein should be specified as MaxSlicesPerAu, as the other variables involved in the calculation are all at the AU level.]

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of slices in each picture in AU n is less than or equal to Min( (Max( 1, MaxSlicesPerPicture * numPics[ n ] * MaxLumaSr / MaxLumaPs * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxSlicesPerPicture * numPics[ n ] ), where MaxSlicesPerPicture, MaxLumaPs and MaxLumaSr are the values specified in Table A.1 and Table A.2 that apply to AU n. [Ed (YK). Further study on whether MaxSlicesPerPicture used herein should be specified as MaxSlicesPerAu, as the other variables involved in the calculation are all at the AU level.]

- For the VCL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrVclFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrVclFactor bits/s.

- For the NAL HRD parameters, BitRate[Htid][ i ] shall be less than or equal to BrNalFactor * MaxBR for at least one value of i in the range of 0 to hrd_cpb_cnt_minus1, inclusive, where BitRate[Htid][ i ] is specified in clause 7.4.6.3 based on parameters selected as specified in clause C.1 and MaxBR is specified in Table A.2 in units of BrNalFactor bits/s.

- The sum of the NumBytesInNalUnit variables for AU 0 shall be less than or equal to FormatCapabilityFactor * ( Max( AuSizeMaxInSamplesY[ 0 ], fR * MaxLumaSr ) + MaxLumaSr * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3, respectively, that apply to AU 0.

- The sum of the NumBytesInNalUnit variables for AU n (with n greater than 0) shall be less than or equal to FormatCapabilityFactor * MaxLumaSr * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ÷ MinCr, where MaxLumaSr and FormatCapabilityFactor are the values specified in Table A.2 and Table A.3 respectively, that apply to AU n.

- The removal time of AU 0 shall satisfy the constraint that the number of tiles in each picture in AU 0 is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * numPics[ 0 ] * 120 * ( AuCpbRemovalTime[ 0 ] − AuNominalRemovalTime[ 0 ] ) + MaxTileCols * MaxTileRows * numPics[ 0 ] * AuSizeMaxInSamplesY[ 0 ] / MaxLumaPs ), MaxTileCols * MaxTileRows * numPics[ 0 ] ), where MaxTileCols and MaxTileRows are the values specified in Table A.1 that apply to AU 0. [Ed (YK). Further study on whether MaxTileCols and MaxTileRows used herein should be specified as for each AU instead of for each picture, as the other variables involved in the calculation are all at the AU level.]

- The difference between consecutive CPB removal times of AUs n and n − 1 (with n greater than 0) shall satisfy the constraint that the number of tiles in each picture in AU n is less than or equal to Min( Max( 1, MaxTileCols * MaxTileRows * numPics[ n ] * 120 * ( AuCpbRemovalTime[ n ] − AuCpbRemovalTime[ n − 1 ] ) ), MaxTileCols * MaxTileRows * numPics[ n ] ), where MaxTileCols and MaxTileRows are the values specified in Table A.1 that apply to AU n. [Ed (YK). Further study on whether MaxTileCols and MaxTileRows used herein should be specified as for each AU instead of for each picture, as the other variables involved in the calculation are all at the AU level.]

Table A.2 – Tier and level limits for the video profiles

| Level | Max luma sample rate MaxLumaSr (samples/sec) | Max bit rate MaxBR (BrVclFactor or BrNalFactor bits/s) |  | Min compression ratio MinCrBase |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Main tier | High tier | Main tier | High tier |
| 1 | 552 960 | 128 | - | 2 | 2 |
| 2 | 3 686 400 | 1 500 | - | 2 | 2 |
| 2.1 | 7 372 800 | 3 000 | - | 2 | 2 |
| 3 | 16 588 800 | 6 000 | - | 2 | 2 |
| 3.1 | 33 177 600 | 10 000 | - | 2 | 2 |
| 4 | 66 846 720 | 12 000 | 30 000 | 4 | 4 |
| 4.1 | 133 693 440 | 20 000 | 50 000 | 4 | 4 |
| 5 | 267 386 880 | 25 000 | 100 000 | 6 | 4 |
| 5.1 | 534 773 760 | 40 000 | 160 000 | 8 | 4 |
| 5.2 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6 | 1 069 547 520 | 60 000 | 240 000 | 8 | 4 |
| 6.1 | 2 139 095 040 | 120 000 | 480 000 | 8 | 4 |
| 6.2 | 4 278 190 080 | 240 000 | 800 000 | 8 | 4 |

Table A.3 – Specification of CpbVclFactor, CpbNalFactor, FormatCapabilityFactor and MinCrScaleFactor

| Profile | CpbVclFactor | CpbNalFactor | FormatCapabilityFactor | MinCrScaleFactor |
| --- | --- | --- | --- | --- |
| Main 10 | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 4:4:4 10 | 2 500 | 2 750 | 3.750 | 0.75 |
| Main 10 Still Picture | 1 000 | 1 100 | 1.875 | 1.00 |
| Main 4:4:4 10 Still Picture | 2 500 | 2 750 | 3.750 | 0.75 |

Informative clause A.4.3 shows the effect of these limits on picture rates for several example picture formats.

#### C.4	Bitstream conformance

...

- For each current picture, after invocation of the process for removal of pictures from the DPB as specified in clause C.3.2, the number of decoded pictures in the DPB, including all pictures n that are marked as "used for reference", or have PictureOutputFlag equal to 1 and DpbOutputTime[ n ] greater than CpbRemovalTime[ currPic ] that have PictureOutputFlag equal to 1 and CpbRemovalTime[ n ] less than CpbRemovalTime[ currPic ], where currPic is the current picture, shall be less than or equal to max_dec_pic_buffering_minus1[ Htid ].

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).