| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0157-v1 |
| --- | --- |

| Title: | AHG9: HRD and related cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to texts related to HRD:

- Change the following descriptions from being picture-specific to be AU-specific:

  - In clauses C.2.3, D.3.2, and D.4.2, rename prevNonDiscardablePic to prevNonDiscardableAu, firstPicInPrevBuffPeriod to firstAuInPrevBuffPeriod, notDiscardablePic to notDiscardableAu, and change their descriptions from being picture-specific to AU-specific.

  - In clause D.3.2, change the description of the semantics of bp_concatenation_flag and bp_cpb_removal_delay_delta_minus1 from being picture-specific to be AU-specific.

  - In clause D.3.2, change the description of the condition for the constraint requiring bp_alt_cpb_params_present_flag to be equal to 0 from being picture-specific to be AU-specific.

  - In clause D.4.2, change the descriptions of BpResetFlag, CpbRemovalDelayMsb[ i ], CpbRemovalDelayVal[ i ] and the constraint requiring pt_cpb_alt_timing_info_present_flag to be equal to 0 from being picture-specific to be AU-specific.

  - In clause D.4.2, change the description of the semantics of pt_dpb_output_delay, pt_dpb_output_du_delay, and pt_display_elemental_periods_minus1 from being picture-specific to be AU-specific.

  - In clause D.5.2 (DU information SEI message semantics), change the description of the semantics of dui_dpb_output_du_delay from being picture-specific to be AU-specific.

- In clause D.3.2, in the constraint requiring bp_alt_cpb_params_present_flag to be equal to 0, add the GDR AU aspect (instead of just having the IRAP aspect).

- When bp_max_sublayers_minus1 is equal to 0, skip bp_sublayer_initial_cpb_removal_delay_present_flag and infer its value to be equal to 0.

- In clause D.4.2, in the constraint requiring pt_cpb_alt_timing_info_present_flag to be equal to 0, add that the RASL pictures in the constraint are RASL pictures that do not have mixed NAL unit types.

- In clause D.5.2, the semantics of pt_display_elemental_periods_minus1, change fixed_pic_rate_within_cvs_flag[ TemporalId ] (3 instances) to be fixed_pic_rate_within_cvs_flag[ Htid ].

# Introduction

The following issues were observed on texts related to HRD:

- In the following places, the descriptions are picture-specific, while HRD operations are OLS based:

  - The names and descriptions of the variables prevNonDiscardablePic, firstPicInPrevBuffPeriod, notDiscardablePic in clauses C.2.3, D.3.2, and D.4.2.

  - The description of the semantics of bp_concatenation_flag and bp_cpb_removal_delay_delta_minus1 in clause D.3.2.

  - The description of the condition for the constraint requiring bp_alt_cpb_params_present_flag to be equal to 0 in clause D.3.2.

  - The descriptions of BpResetFlag, CpbRemovalDelayMsb[ i ], CpbRemovalDelayVal[ i ] and the constraint requiring pt_cpb_alt_timing_info_present_flag to be equal to 0 in clause D.4.2.

  - The description of the semantics of pt_dpb_output_delay, pt_dpb_output_du_delay, and pt_display_elemental_periods_minus1 in clause D.4.2.

  - The description of the semantics of dui_dpb_output_du_delay in clause D.5.2.

- In the constraint on the value of bp_alt_cpb_params_present_flag, only IRAP pictures are considered while GDR pictures also need to be considered.

- When bp_max_sublayers_minus1 is equal to 0, it is silly to signal bp_sublayer_initial_cpb_removal_delay_present_flag as it does not make any sense to have the value to be equal to 1 under this condition. In addition, other sublayer specific syntax elements in the BP SEI message syntax are conditioned, thus this is inconsistent.

- In clause D.4.2, in the constraint requiring pt_cpb_alt_timing_info_present_flag to be equal to 0, the RASL pictures in the condition need to be clarified to be RASL pictures that do not contain mixed NAL unit types because now a RASL picture may contain mixed NAL unit types, while the original intent herein was for RASL pictures without mixed NAL unit types.

- In the semantics of pt_display_elemental_periods_minus1, the syntax element fixed_pic_rate_within_cvs_flag[ TemporalId ] is used. However, since the semantics should be described in the context of a target highest TemporalId value, like in other semantics of BP, PT, and DUI SEI messages, fixed_pic_rate_within_cvs_flag[ Htid ] should be used instead.

# Proposal

The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### C.2.3	Timing of DU removal and decoding of DU

...

The nominal removal time of the AU n from the CPB is specified as follows:

–	If AU n is the AU with n equal to 0 (the AU that initializes the HRD), the nominal removal time of the AU from the CPB is specified by:

AuNominalRemovalTime[ 0 ] = InitCpbRemovalDelay[ Htid ][ ScIdx ] ÷ 90000	(C.9)

–	Otherwise, the following applies:

–	When AU n is the first AU of a BP that does not initialize the HRD, the following applies:

The nominal removal time of the AU n from the CPB is specified by:

if( !concatenationFlag ) {
	baseTime = AuNominalRemovalTime[ firstAuInPrevBuffPeriod ]
	tmpCpbRemovalDelay = AuCpbRemovalDelayVal
} else {
	baseTime1 = AuNominalRemovalTime[ prevNonDiscardableAu ]
	tmpCpbRemovalDelay1 = ( auCpbRemovalDelayDeltaMinus1 + 1 )
	baseTime2 = AuNominalRemovalTime[ n − 1 ]
	tmpCpbRemovalDelay2 =				(C.10)
			Ceil( ( InitCpbRemovalDelay[ Htid ][ ScIdx ] ÷ 90000 +
				AuFinalArrivalTime[ n − 1 ] − AuNominalRemovalTime[ n − 1 ] ) ÷ ClockTick )
	if( baseTime1 + ClockTick * tmpCpbRemovalDelay1 <
				baseTime2 + ClockTick * tmpCpbRemovalDelay2 ) {
		baseTime = baseTime2
		tmpCpbRemovalDelay = tmpCpbRemovalDelay2
	} else {
		baseTime = baseTime1
		tmpCpbRemovalDelay = tmpCpbRemovalDelay1
	}
}
AuNominalRemovalTime[ n ] = baseTime + ( ClockTick * tmpCpbRemovalDelay − CpbDelayOffset )

where AuNominalRemovalTime[ firstAuInPrevBuffPeriod ] is the nominal removal time of the first AU of the previous BP, AuNominalRemovalTime[ prevNonDiscardableAu ] is the nominal removal time of the previous AU the preceding picture in decoding order with TemporalId and ph_non_ref_pic_flag both equal to 0 with TemporalId equal to 0 that has at least one picture that has ph_non_ref_pic_flag equal to 0 that is not a RASL or RADL picture, AuCpbRemovalDelayVal is the value of CpbRemovalDelayVal[ Htid ] derived according to pt_cpb_removal_delay_minus1[ Htid ] and pt_cpb_removal_delay_delta_idx[ Htid ] in the PT SEI message, and bp_cpb_removal_delay_delta_val[ pt_cpb_removal_delay_delta_idx[ Htid ] ] in the BP SEI message, selected as specified in clause C.1, associated with AU n and concatenationFlag and CpbRemovalDelayDeltaMinus1 are the values of the syntax elements bp_concatenation_flag and bp_cpb_removal_delay_delta_minus1, respectively, in the BP SEI message, selected as specified in clause C.1, associated with AU n.

After the derivation of the nominal CPB removal time and before the derivation of the DPB output time of access unit n, the variables DpbDelayOffset and CpbDelayOffset are derived as:

–	If one or more of the following conditions are true, DpbDelayOffset is set equal to the value of the PT SEI message syntax element dpb_delay_offset[ Htid ] of AU n + 1, and CpbDelayOffset is set equal to the value of the PT SEI message syntax element cpb_delay_offset[ Htid ] of AU n + 1, where the PT SEI message containing the syntax elements is selected as specified in clause C.1:

–	UseAltCpbParamsFlag for AU n is equal to 1.

–	DefaultInitCpbParamsFlag is equal to 0.

–	Otherwise, DpbDelayOffset and CpbDelayOffset are both set equal to 0.

–	When AU n is not the first AU of a BP, the nominal removal time of the AU n from the CPB is specified by:

AuNominalRemovalTime[ n ] = AuNominalRemovalTime[ firstAuInCurrBuffPeriod ] +
		ClockTick * ( AuCpbRemovalDelayVal − CpbDelayOffset )	(C.11)

where AuNominalRemovalTime[ firstAuInCurrBuffPeriod ] is the nominal removal time of the first AU of the current BP and AuCpbRemovalDelayVal is the value of CpbRemovalDelayVal[ OpTid ] derived according to pt_cpb_removal_delay_minus1[ OpTid ] and pt_cpb_removal_delay_delta_idx[ OpTid ] in the PT SEI message, and bp_cpb_removal_delay_delta_val[ pt_cpb_removal_delay_delta_idx[ OpTid ] ] in the BP SEI message, selected as specified in clause C.1, associated with AU n.

...

#### D.3.1	Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| ... |  |
| if( bp_max_sublayers_minus1 > 0 ) |  |
| bp_sublayer_initial_cpb_removal_delay_present_flag | u(1) |
| ... |  |
| } |  |

#### D.3.2	Buffering period SEI message semantics

...

When the BP SEI message is present, an AU is said to be a notDiscardableAu when the AU has TemporalId equal to 0 and has at least one picture that has ph_non_ref_pic_flag equal to 0 that a picture is said to be a notDiscardablePic picture when the picture has TemporalId and ph_non_ref_pic_flag both equal to 0 and is not a RASL or RADL picture.

When the current AU is not the first AU the current picture is not the first picture in the bitstream in decoding order, let the AU prevNonDiscardableAu be the previous AU in decoding order with TemporalId equal to 0 that has at least one picture that has ph_non_ref_pic_flag equal to 0 let prevNonDiscardablePic be the preceding picture in decoding order with TemporalId and ph_non_ref_pic_flag both equal to 0 that is not a RASL or RADL picture.

The presence of BP SEI messages is specified as follows:

–	If NalHrdBpPresentFlag is equal to 1 or VclHrdBpPresentFlag is equal to 1, the following applies for each AU in the CVS:

–	If the AU is an IRAP or GDR AU, a BP SEI message applicable to the operation point shall be associated with the AU.

–	Otherwise, if the AU is a notDiscardableAu contains a notDiscardablePic, a BP SEI message applicable to the operation point may or may not be associated with the AU.

–	Otherwise, the AU shall not be associated with a BP SEI message applicable to the operation point.

–	Otherwise (NalHrdBpPresentFlag and VclHrdBpPresentFlag are both equal to 0), no AU in the CVS shall be associated with a BP SEI message.

NOTE 1 – For some applications, frequent presence of BP SEI messages may be desirable (e.g., for random access at an IRAP AU or a non-IRAP AU an IRAP picture or a non-IRAP picture or for bitstream splicing).

...

bp_alt_cpb_params_present_flag equal to 1 specifies the presence of the syntax element bp_use_alt_cpb_params_flag in the BP SEI message and the presence of the alternative timing information in the PT SEI messages in the current BP. When not present, the value of bp_alt_cpb_params_present_flag is inferred to be equal to 0. When the associated AU is not an IRAP or GDR AU the associated picture is neither a CRA picture nor an IDR picture, the value of bp_alt_cpb_params_present_flag shall be equal to 0.

...

bp_concatenation_flag indicates, when the current AU is not the first AU the current picture is not the first picture in the bitstream in decoding order, whether the nominal CPB removal time of the current AU picture is determined relative to the nominal CPB removal time of the previous AU associated preceding picture with a BP SEI message or relative to the nominal CPB removal time of the AU prevNonDiscardableAu the picture prevNonDiscardablePic.

...

bp_cpb_removal_delay_delta_minus1 plus 1, when the current AU is not the first AU the current picture is not the first picture in the bitstream in decoding order, specifies a CPB removal delay increment value relative to the nominal CPB removal time of the AU prevNonDiscardableAu the picture prevNonDiscardablePic. The length of this syntax element is bp_cpb_removal_delay_length_minus1 + 1 bits.

When the current AU is associated with the current picture contains a BP SEI message and bp_concatenation_flag is equal to 0 and the current AU is not the first AU the current picture is not the first picture in the bitstream in decoding order, it is a requirement of bitstream conformance that the following constraint applies:

–	If the AU prevNonDiscardableAu the picture prevNonDiscardablePic is not associated with a BP SEI message, the pt_cpb_removal_delay_minus1 of the current AU picture shall be equal to the pt_cpb_removal_delay_minus1 of the AU prevNonDiscardableAu the picture prevNonDiscardablePic plus bp_cpb_removal_delay_delta_minus1 + 1.

–	Otherwise, pt_cpb_removal_delay_minus1 shall be equal to bp_cpb_removal_delay_delta_minus1.

NOTE 2 – When the current AU is associated with the current picture contains a BP SEI message and bp_concatenation_flag is equal to 1, the pt_cpb_removal_delay_minus1 for the current AU picture is not used. The above-specified constraint can, under some circumstances, make it possible to splice bitstreams (that use suitably-designed referencing structures) by simply changing the value of bp_concatenation_flag from 0 to 1 in the BP SEI message for an IRAP or GDR AU picture at the splicing point. When bp_concatenation_flag is equal to 0, the above-specified constraint enables the decoder to check whether the constraint is satisfied as a way to detect the loss of the AU prevNonDiscardableAu the picture prevNonDiscardablePic.

...

bp_sublayer_initial_cpb_removal_delay_present_flag equal to 1 specifies that initial CPB removal delay related syntax elements are present for sublayer representation(s) in the range of 0 to bp_max_sublayers_minus1, inclusive. bp_sublayer_initial_cpb_removal_delay_present_flag equal to 0 specifies that initial CPB removal delay related syntax elements are present for the bp_max_sublayers_minus1-th sublayer representation. When not present, the value of bp_sublayer_initial_cpb_removal_delay_present_flag is inferred to be equal to 0.

...

#### D.4.2	Picture timing SEI message semantics

...

pt_cpb_alt_timing_info_present_flag equal to 1 specifies that the syntax elements pt_nal_cpb_alt_initial_removal_delay_delta[ i ][ j ], pt_nal_cpb_alt_initial_removal_offset_delta[ i ][ j ], pt_nal_cpb_delay_offset[ i ], pt_nal_dpb_delay_offset[ i ], pt_vcl_cpb_alt_initial_removal_delay_delta[ i ][ j ], pt_vcl_cpb_alt_initial_removal_offset_delta[ i ][ j ], pt_vcl_cpb_delay_offset[ i ], and pt_vcl_dpb_delay_offset[ i ] may be present in the PT SEI message. pt_cpb_alt_timing_info_present_flag equal to 0 specifies that these syntax elements are not present in the PT SEI message. When all pictures in the associated AU are RASL pictures with pps_mixed_nalu_types_in_pic_flag equal to 0 the associated picture is a RASL picture, the value of pt_cpb_alt_timing_info_present_flag shall be equal to 0.

NOTE 1 – The value of pt_cpb_alt_timing_info_present_flag might be equal to 1 for more than one AU following an IRAP AU picture in decoding order. However, the alternative timing is only applied to the first AU that has pt_cpb_alt_timing_info_present_flag equal to 1 and follows the IRAP AU picture in decoding order.

...

pt_vcl_dpb_delay_offset[ i ] specifies, for the i-th sublayer for the VCL HRD, an offset to be used in the derivation of the DPB output times of the IRAP AU associated with the BP SEI message when the AU associated with the PT SEI message directly follows in decoding order the IRAP AU associated with the BP SEI message. The length of pt_vcl_dpb_delay_offset[ i ] is bp_dpb_output_delay_length_minus1 + 1 bits. When not present, the value of pt_vcl_dpb_delay_offset[ i ] is inferred to be equal to 0.

The variable BpResetFlag of the current AU picture is derived as follows:

–	If the current AU picture is associated with a BP SEI message, BpResetFlag is set equal to 1.

–	Otherwise, BpResetFlag is set equal to 0.

...

pt_cpb_removal_delay_delta_idx[ i ] specifies the index of the CPB removal delta that applies to Htid equal to i in the list of bp_cpb_removal_delay_delta_val[ j ] for j ranging from 0 to bp_num_cpb_removal_delay_deltas_minus1, inclusive. The length of pt_cpb_removal_delay_delta_idx[ i ] is Ceil( Log2( bp_num_cpb_removal_delay_deltas_minus1 + 1 ) ) bits. When pt_cpb_removal_delay_delta_idx[ i ] is not present and pt_cpb_removal_delay_delta_enabled_flag[ i ] is equal to 1, the value of pt_cpb_removal_delay_delta_idx[ i ] is inferred to be equal to 0.

The variables CpbRemovalDelayMsb[ i ] and CpbRemovalDelayVal[ i ] of the current AU picture are derived as follows:

–	If the current AU is the AU that initializes the HRD, CpbRemovalDelayMsb[ i ] and CpbRemovalDelayVal[ i ] are both set equal to 0, and the value of cpbRemovalDelayValTmp[ i ] is set equal to pt_cpb_removal_delay_minus1[ i ] + 1.

–	Otherwise, let the AU prevNonDiscardableAu be the previous AU the picture prevNonDiscardablePic be the previous picture in decoding order with TemporalId equal to 0 that has at least one picture that has ph_non_ref_pic_flag equal to 0 that has TemporalId and ph_non_ref_pic_flag both equal to 0 that is not a RASL or RADL picture, let prevCpbRemovalDelayMinus1[ i ], prevCpbRemovalDelayMsb[ i ], and prevBpResetFlag be set equal to the values of cpbRemovalDelayValTmp[ i ] − 1, CpbRemovalDelayMsb[ i ], and BpResetFlag, respectively, for the AU prevNonDiscardablAu the picture prevNonDiscardablePic, and the following applies:

...

pt_dpb_output_delay is used to compute the DPB output time of the AU picture. It specifies how many clock ticks to wait after removal of an AU from the CPB before the decoded pictures of the AU are the decoded picture is output from the DPB.

NOTE 2 – A decoded picture is not removed from the DPB at its output time when it is still marked as "used for short-term reference" or "used for long-term reference".

The length of pt_dpb_output_delay is bp_dpb_output_delay_length_minus1 + 1 bits. When max_dec_pic_buffering_minus1[ Htid ] is equal to 0, the value of pt_dpb_output_delay shall be equal to 0.

The output time derived from the pt_dpb_output_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the pt_dpb_output_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, a CVSS AU a CLVSS picture that has ph_no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from pt_dpb_output_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

pt_dpb_output_du_delay is used to compute the DPB output time of the AU picture when DecodingUnitHrdFlag is equal to 1. It specifies how many sub clock ticks to wait after removal of the last DU in an AU from the CPB before the decoded pictures of the AU are the decoded picture is output from the DPB.

...

pt_display_elemental_periods_minus1 plus 1, when sps_field_seq_flag is equal to 0 and fixed_pic_rate_within_cvs_flag[ HtidTemporalId ] is equal to 1, indicates the number of elemental picture period intervals that the decoded pictures of the current AU occupy the current coded picture occupies for the display model.

When fixed_pic_rate_within_cvs_flag[ HtidTemporalId ] is equal to 0 or sps_field_seq_flag is equal to 1, the value of pt_display_elemental_periods_minus1 shall be equal to 0.

When sps_field_seq_flag is equal to 0 and fixed_pic_rate_within_cvs_flag[ HtidTemporalId ] is equal to 1, a value of pt_display_elemental_periods_minus1 greater than 0 may be used to indicate a frame repetition period for displays that use a fixed frame refresh interval equal to DpbOutputElementalInterval[ n ] as given by Equation 112.

...

#### D.5.2	DU information SEI message semantics

...

dui_dpb_output_du_delay is used to compute the DPB output time of the AU picture when DecodingUnitHrdFlag is equal to 1 and bp_du_dpb_params_in_pic_timing_sei_flag is equal to 0. It specifies how many sub clock ticks to wait after removal of the last DU in an AU from the CPB before the decoded pictures of the AU are the decoded picture is output from the DPB. When not present, the value of dui_dpb_output_du_delay is inferred to be equal to pt_dpb_output_du_delay. The length of the syntax element dui_dpb_output_du_delay is given in bits by bp_dpb_output_delay_du_length_minus1 + 1.

It is a requirement of bitstream conformance that all DU information SEI messages that are associated with the same AU, apply to the same operation point, and have bp_du_dpb_params_in_pic_timing_sei_flag  equal to 0 shall have the same value of dui_dpb_output_du_delay.

The output time derived from the dui_dpb_output_du_delay of any picture that is output from an output timing conforming decoder shall precede the output time derived from the dui_dpb_output_du_delay of all pictures in any subsequent CVS in decoding order.

The picture output order established by the values of this syntax element shall be the same order as established by the values of PicOrderCntVal.

For pictures that are not output by the "bumping" process because they precede, in decoding order, a CVSS AU CLVSS picture that has ph_no_output_of_prior_pics_flag equal to 1 or inferred to be equal to 1, the output times derived from dui_dpb_output_du_delay shall be increasing with increasing value of PicOrderCntVal relative to all pictures within the same CVS.

For any two pictures in the CVS, the difference between the output times of the two pictures when DecodingUnitHrdFlag is equal to 1 shall be identical to the same difference when DecodingUnitHrdFlag is equal to 0.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).