| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0158-v2 |
| --- | --- |

| Title: | AHG9/AHG8: On the general sub-bitstream extraction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the general sub-bitstream extraction process:

- In the general sub-bitstream extraction process, the conditions under which an output sub-bitstream is required to be a conforming bitstream is specified such that the value of tIdTarget is specified be in the range of 0 to sps_video_parameter_set_id > 0 ? vps_ptl_max_tid[ vps_ols_ptl_idx[ targetOlsIdx ] ] : sps_max_sublayers_minus1, inclusive.

- The general sub-bitstream extraction process is specified such that it would remove, from the output bitstream outBitstream, SEI NAL units that contain a scalable nesting SEI message with sn_ols_flag equal to 0 while the applicable layers as indicted in the scalable nesting SEI message does not include any layer in the target OLS.

- Make the following changes to the 4th step in the general sub-bitstream extraction process:

- Remove the redundant condition "nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive". (This is asserted to be editorial.)

- When removing from the output bitstream the VCL NAL units, instead of removing the associated SEI NAL units containing SEI messages other than the BP, PT, or DUI SEI messages, remove all SEI NAL units that contain SEI messages that become not applicable to any picture or subpicture remaining in the final output bitstream. (Since this is not easy to do, another option is not to remove SEI NAL units in this step.)

- Make the following changes to the last step in the general sub-bitstream extraction process:

- The condition "When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all NAL units in the bitstream" is changed to be "When the list LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all VCL NAL units in the bitstream inBitstream".

- When at least one layer is removed by the extraction process, also remove all SEI NAL units that contain a non-scalable-nested SLI SEI message (same as for BP, PT, and DUI SEI messsages).

- Insert SEI NAL units to directly contain those SEI messages that were scalable-nested SEI messages (with sn_subpic_flag equal to 0 and regardless of the value of sn_ols_flag) that apply only to the output bitstream, and remove their original container SEI NAL units from the output bitstream.

# Introduction

The existing design of the general sub-bitstream extraction process in the latest VVC draft text has the following issues:

- In the conditions under which an output sub-bitstream is required to be a conforming bitstream, the value of tIdTarget is currently specified to be in the range of 0 to 6, inclusive. However, in many bitstreams, the highest TemporalId value is less than 6, and that value is specified by vps_ptl_max_tid[ vps_ols_ptl_idx[ targetOlsIdx ] ] if sps_video_parameter_set_id is greater than 0 and  sps_max_sublayers_minus1 otherwise.

- SEI NAL units containing HRD-related scalable-nested SEI messages that become useless in the output bitstream are removed. However, SEI NAL units containing non-HRD-related scalable-nested SEI messages that become useless in the output bitstream should also be removed.

- The 4th step has the following issues:

- When removing the VCL NAL units, the step also removes the associated SEI NAL units containing SEI messages other than the BP, PT, or DUI SEI messages. However, some of those SEI messages being removed may apply to OLSs or layers that contain pictures thhat remain in the output bitstream.

- The 2nd condition for the VCL NAL units, "nuh_layer_id is equal to LayerIdInOls[ targetOlsIdx ][ j ] for a value of j in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1 inclusive" is redundant, due to the 3rd step.

- The last step (i.e., step 6) has the following issues:

- The condition "When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all NAL units in the bitstream" has the following two issues:

  - The condition does not work for cases when DCI, VPS, AUD, or EOB NAL units are present and have nuh_layer_id not equal to any of the nuh_layer_id values of the VCL NAL units.

  - The phrase "the bitstream" is not clear, as there are two bitstreams involved in the context, inBitstream and outBitstream.

- The subpicture level information (SLI) SEI messages, when present, apply to OLSs, like the other HRD-related SEI messages, i.e., BP, PT, DUI SEI messages. However, in step 6.a, when at least one layer is removed, SEI NAL units that contain a non-scalable-nested SLI SEI message is not removed, while SEI NAL unit containing non-scalable-nested BP, PT and DUI SEI messages are removed.

- Step 6.c has the following issues:

  - It would extract scalable-nested SEI messages, to generate non-scalable-nested SEI messages, from scalable nesting SEI messages with both sn_ols_flag equal to 1 and sn_subpic_flag equal to 1. However, such scalable-nested SEI messages only apply to specific subpictures and should not be extracted.

  - HRD-related scalable-nested SEI messages are handled. Non-HRD-related scalable-nested SEI messages should be handled similarly.

  - When multiple scalable-nested SEI messages are extracted from one SEI NAL unit seiNalUnitA to be non-scalable-nested SEI messages, they should still be included in one SEI NAL unit seiNalUnitB, and the SEI NAL unit seiNalUnitB should be included in the same PU, and the original container SEI NAL unit (seiNalUnitA) should be removed.

# Proposal

The proposal for addressing the issues above is summarized as above in the abstract. The proposed text changes are as follows, with changes marked relative to the VVC draft text in JVET-S0152-v5:

#### C.7	General sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, and a target highest TemporalId value tIdTarget.

Output of this process is a sub-bitstream outBitstream.

The OLS with OLS index targetOlsIdx is referred to as the target OLS. Let the variable maxTid be set equal to sps_video_parameter_set_id > 0: vps_ptl_max_tid[ vps_ols_ptl_idx[ targetOlsIdx ] ] : sps_max_sublayers_minus1.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and tIdTarget equal to any value in the range of 0 to maxTid, inclusive, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

The output sub-bitstream OutBitstream is derived by applying the following ordered steps:

- The bitstream outBitstream is set to be identical to the bitstream inBitstream.

- Remove from outBitstream all NAL units with TemporalId greater than tIdTarget.

- Remove from outBitstream all NAL units that have nuh_layer_id not included in the list LayerIdInOls[ targetOlsIdx ] and are not DCI NAL units, VPS NAL units, EOB NAL units, or SEI NAL units containing non-scalable-nested SEI messages.

- Remove from outBitstream all VCL NAL units for which all of the following conditions are true and their associated non-VCL NAL units that have nal_unit_type equal to PH_NUT or FD_NUT:

–	nal_unit_type is equal to TRAIL_NUT, STSA_NUT, RADL_NUT, or RASL_NUT, or nal_unit_type is equal to GDR_NUT and the associated ph_recovery_poc_cnt is greater than 0.

–	TemporalId is greater than or equal to NumSubLayersInLayerInOLS[ targetOlsIdx ][ GeneralLayerIdx[ nuh_layer_id ] ].

- Remove from outBitstream all SEI NAL units that only contain SEI messages that do not apply to the OLSs, layers, pictures, subpictures, or slices in outBitstream, as follows:

–		Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 1 and there is no value of i in the range of 0 to sn_num_olss_minus1, inclusive, such that NestingOlsIdx[ i ] is equal to targetOlsIdx.

–	Remove from outBitstream all SEI NAL units that contain a scalable nesting SEI message that has sn_ols_flag equal to 0 and there is no value in the list NestingLayerId equal to a value in the list LayerIdInOls[ targetOlsIdx ].

–	Remove from outBitstream all SEI NAL units that contain SEI messages that only apply to one or more of the pictures or subpictures for which all VCL NAL units have been removed by step 4.

- When LayerIdInOls[ targetOlsIdx ] does not include all values of nuh_layer_id in all VCL NAL units in the bitstream inBitstream, the following applies in the order listed:

- Remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 0 (BP), 130 (DUI), or 203 (SLI).

- When general_same_pic_timing_in_all_ols_flag is equal to 0, remove from outBitstream all SEI NAL units that contain a non-scalable-nested SEI message with payloadType equal to 1 (PT).

- When outBitstream contains an SEI NAL unit seiNalUnitA that contains a scalable nesting SEI message with sn_subpic_flag equal to 0 that applies to the OLSs (when sn_ols_flag is equal to 1) or the layers (when sn_ols_flag is equal to 0) that have the same set of layers as in outBitstream, generate a new SEI NAL unit seiNalUnitB, include it in the PU containing seiNalUnitA immediately after seiNalUnitA, extract the scalable-nested SEI messages from the scalable nesting SEI message and include them directly in seiNalUnitB (as non-scalable-nested SEI messages), and remove seiNalUnitA from outBitstream.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).