| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0160-v1 |
| --- | --- |

| Title: | AHG9/AHG12/AHG8: Miscellaneous HLS cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 Li Zhang1, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following miscellaneous HLS changes:

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

- Add a constraint: When the AUD is present for the current AU, and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2.

- Add a constraint: When the AUD is present for the current AU, and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2.

- Change "less than" in the following condition (that's part of the specification of the first picture of an AU) to "less than or equal to": The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

- Remove the following constraint from the definition of the two still picture profiles: The referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0.

- Remove general_one_picture_only_constraint_flag.

- Make sure that each VUI syntax element that describes a property of the bitstream has an inferred/default value when sps_vui_parameters_present_flag is equal to 0.

# Introduction

This section provides justifications for some of the proposed items listed in the abstract:

- Justification for proposal item 1: In the latest VVC text, the value of pps_loop_filter_across_tiles_enabled_flag is inferred to be equal to 1 when not present. This is odd, compared to the inference of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

pps_loop_filter_across_tiles_enabled_flag is not present when pps_no_pic_partition_flag is equal to 1 or NumTilesInPic is equal to 1. Under either of these two conditions, there is only one tile per picture, thus tile boundaries are picture boundaries, and there is no loop filtering across picture boundaries. Therefore, it makes more sense to infer the value of pps_loop_filter_across_tiles_enabled_flag is inferred to be equal to 0 when not present.

- Justification for proposal item 2: When the AUD is present for the current AU, and aud_pic_type equal to 0, it specifies that the sh_slice_type values that may be present in the AU are I slices.

- Justification for proposal item 3: When the AUD is present for the current AU, and aud_pic_type equal to 1, it specifies that the sh_slice_type values that may be present in the AU are P or I slices.

- Justification for proposal item 4: When the current picture has the same layer ID as the previous picture, then the current picture would also start a new AU.

- Justification for proposal item 5: When the constraint that the referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0 is removed, then it is possible to extract an intra coded picture from a video bitstream with inter prediction applied without the need of rewriting the DPB parameters in the referenced SPS.

- Justification for proposal item 6: The general_one_picture_only_constraint_flag was introduced for the signalling of the still picture profiles. Now that it was decided not to use this flag for that purpose, there is no longer a need of it.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).