| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0160-v3 |
| --- | --- |

| Title: | AHG9/AHG12/AHG8: Miscellaneous HLS cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 Li Zhang1, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following miscellaneous HLS changes:

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

- Add a constraint: When an AUD is present for the current AU, and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2.

- Add a constraint: When an AUD is present for the current AU, and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2.

- Add a constraint: When vps_max_layers_minus1 is equal to 0, an AUD is present for the current AU, and aud_irap_or_gdr_au_flag is equal to 0, the value of ph_gdr_or_irap_pic_flag shall be equal to 0.

- Change "less than" in the following condition (that's part of the specification of the first picture of an AU) to "less than or equal to": The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

- Remove the following constraint from the definition of the two still picture profiles: The referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0.

- Remove general_one_picture_only_constraint_flag.

- Make sure that each VUI syntax element that describes a property of the bitstream has an inferred/default value when sps_vui_parameters_present_flag is equal to 0.

- Change an existing constraint such that the value of ph_poc_msb_cycle_present_flag is required to be equal to 0 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is an ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture (instead of "and there is a picture in the current AU in a reference layer of the current layer").

V2 of this contribution added a proposal item, and added justifiations for two items.

V2 of this contribution added item 9.

# Introduction

This section provides justifications for some of the proposed items listed in the abstract:

- Justification for proposal item 1: In the latest VVC text, the value of pps_loop_filter_across_tiles_enabled_flag is inferred to be equal to 1 when not present. This is odd, compared to the inference of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

pps_loop_filter_across_tiles_enabled_flag is not present when pps_no_pic_partition_flag is equal to 1 or NumTilesInPic is equal to 1. Under either of these two conditions, there is only one tile per picture, thus tile boundaries are picture boundaries, and there is no loop filtering across picture boundaries. Therefore, it makes more sense to infer the value of pps_loop_filter_across_tiles_enabled_flag is inferred to be equal to 0 when not present.

- Justification for proposal item 2: When an AUD is present for the current AU, and aud_pic_type equal to 0, it specifies that the sh_slice_type values that may be present in the AU are I slices.

- Justification for proposal item 3: When an AUD is present for the current AU, and aud_pic_type equal to 1, it specifies that the sh_slice_type values that may be present in the AU are P or I slices.

- Justification for proposal item 4: For a single-layer bitstream, when an AUD is present for the current AU and aud_irap_or_gdr_au_flag is equal to 0, the AU is not an IRAP or GDR AU, and consequently the current picture is not an IRAP or GDR picture, and therefore the value of ph_gdr_or_irap_pic_flag should no be equal to 1, otherwise the semantics are conflicting with each other.

- Justification for proposal item 5: When the current picture has the same layer ID as the previous picture, then the current picture would also start a new AU.

- Justification for proposal item 6: When the constraint that the referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0 is removed, then it is possible to extract an intra coded picture from a video bitstream with inter prediction applied without the need of rewriting the DPB parameters in the referenced SPS.

- Justification for proposal item 7: The general_one_picture_only_constraint_flag was introduced for the signalling of the still picture profiles. Now that it was decided not to use this flag for that purpose, there is no longer a need of it.

- Justification for item 8 (which is currently just a suggestion): Some new fields have been moved from GCI to VUI. Those fields probably don't have default/inferred values specified. The inclusion of VUI in the SEI spec instead of in the main VVC spec is another reason for checking this. For example, the meaning of "when not present" for a VUI syntax element in the SEI spec may need to be clarified on what it exactly means.

- It is required that ph_poc_msb_cycle_present_flag shall be to equal to 0 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is a picture in the current AU in a reference layer of the current layer. However, such a picture in a reference layer could be removed by the general sub-bitstream extraction process specified in clause C.6, and in this case, the POC derivation for the current picture can be messed up.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).