| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0160-v4 |
| --- | --- |

| Title: | AHG9/AHG12/AHG8: Miscellaneous HLS cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 Li Zhang1, Kai Zhang1 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com lizhang.idm@bytedance.com zhangkai.video@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following miscellaneous HLS changes:

- Infer the value of pps_loop_filter_across_tiles_enabled_flag to be equal to 0 (instead of 1) when not present, for consistency with the inferences of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

- Add a constraint: When an AUD is present for the current AU, and aud_pic_type is equal to 0, the value of sh_slice_type shall be equal to 2.

- Add a constraint: When an AUD is present for the current AU, and aud_pic_type is equal to 1, the value of sh_slice_type shall be equal to 1 or 2.

- Add a constraint: When vps_max_layers_minus1 is equal to 0, an AUD is present for the current AU, and aud_irap_or_gdr_au_flag is equal to 0, the value of ph_gdr_or_irap_pic_flag shall be equal to 0.

- Change "less than" in the following condition (that's part of the specification of the first picture of an AU) to "less than or equal to": The value of nuh_layer_id of the VCL NAL unit is less than the nuh_layer_id of the previous picture in decoding order.

- Remove the following constraint from the definition of the two still picture profiles: The referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0.

- Remove general_one_picture_only_constraint_flag.

- Make sure that each VUI syntax element that describes a property of the bitstream has an inferred/default value when sps_vui_parameters_present_flag is equal to 0.

- Change an existing constraint such that the value of ph_poc_msb_cycle_present_flag is required to be equal to 0 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is an ILRP entry in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture (instead of "and there is a picture in the current AU in a reference layer of the current layer").

- Instead of signalling a set of width, height, chroma format, and bit depth of a DPB picture storage buffer for each multi-layer OLS in the VPS, signal these DPB parameters in the dpb_parameters( ) syntax strucures, each of which (per the existing VVC text) can be shared by two or more OLSs.

V2 of this contribution added a proposal item, and added justifiations for two items.

V3 of this contribution added item 9.

V4 of this contribtion added item 10.

# Introduction

This section provides justifications for some of the proposed items listed in the abstract:

- Justification for proposal item 1: In the latest VVC text, the value of pps_loop_filter_across_tiles_enabled_flag is inferred to be equal to 1 when not present. This is odd, compared to the inference of sps_loop_filter_across_subpic_enabled_flag[ i ] and pps_loop_filter_across_slices_enabled_flag.

pps_loop_filter_across_tiles_enabled_flag is not present when pps_no_pic_partition_flag is equal to 1 or NumTilesInPic is equal to 1. Under either of these two conditions, there is only one tile per picture, thus tile boundaries are picture boundaries, and there is no loop filtering across picture boundaries. Therefore, it makes more sense to infer the value of pps_loop_filter_across_tiles_enabled_flag is inferred to be equal to 0 when not present.

- Justification for proposal item 2: When an AUD is present for the current AU, and aud_pic_type equal to 0, it specifies that the sh_slice_type values that may be present in the AU are I slices.

- Justification for proposal item 3: When an AUD is present for the current AU, and aud_pic_type equal to 1, it specifies that the sh_slice_type values that may be present in the AU are P or I slices.

- Justification for proposal item 4: For a single-layer bitstream, when an AUD is present for the current AU and aud_irap_or_gdr_au_flag is equal to 0, the AU is not an IRAP or GDR AU, and consequently the current picture is not an IRAP or GDR picture, and therefore the value of ph_gdr_or_irap_pic_flag should no be equal to 1, otherwise the semantics are conflicting with each other.

- Justification for proposal item 5: When the current picture has the same layer ID as the previous picture, then the current picture would also start a new AU.

- Justification for proposal item 6: When the constraint that the referenced SPS shall have max_dec_pic_buffering_minus1[ sps_max_sublayers_minus1 ] equal to 0 is removed, then it is possible to extract an intra coded picture from a video bitstream with inter prediction applied without the need of rewriting the DPB parameters in the referenced SPS.

- Justification for proposal item 7: The general_one_picture_only_constraint_flag was introduced for the signalling of the still picture profiles. Now that it was decided not to use this flag for that purpose, there is no longer a need of it.

- Justification for item 8 (which is currently just a suggestion): Some new fields have been moved from GCI to VUI. Those fields probably don't have default/inferred values specified. The inclusion of VUI in the SEI spec instead of in the main VVC spec is another reason for checking this. For example, the meaning of "when not present" for a VUI syntax element in the SEI spec may need to be clarified on what it exactly means.

- It is required that ph_poc_msb_cycle_present_flag shall be to equal to 0 when vps_independent_layer_flag[ GeneralLayerIdx[ nuh_layer_id ] ] is equal to 0 and there is a picture in the current AU in a reference layer of the current layer. However, such a picture in a reference layer could be removed by the general sub-bitstream extraction process specified in clause C.6, and in this case, the POC derivation for the current picture can be messed up.

- The DPB parameters syntax elements vps_ols_dpb_pic_width[ i ], vps_ols_dpb_pic_height[ i ], vps_ols_dpb_chroma_format[ i ], and vps_ols_dpb_bitdepth_minus8[ i ] are signalled in the VPS for each multi-layer OLS. However, in many cases, multiple OLSs have the same values for these parameters, while the DPB parameters syntax elements max_dec_pic_buffering_minus1[ i ], max_num_reorder_pics[ i ], and max_latency_increase_plus1[ i ] are signalled in the dpb_parameters( ) syntax strucures, each of which can be shared by two or more OLSs.

# Proposed text changes for item 10

The proposed text canges for item 10 are as follows, where additions are highlighted in turquoise, and deletions are highlighted in yellow strikethrough in red fonts):

#### 7.3.2.2	Video parameter set RBSP syntax

| video_parameter_set_rbsp( ) { | Descriptor |  |
| --- | --- | --- |
| ... |  |  |
| if( !vps_each_layer_is_an_ols_flag ) { |  |  |
| vps_num_dpb_params_minus1 | ue(v) |
| if( vps_max_sublayers_minus1 > 0 ) |  |  |
| vps_sublayer_dpb_params_present_flag | u(1) |  |
| for( i = 0; i < VpsNumDpbParams; i++ ) { |  |  |
| if( !vps_default_ptl_dpb_hrd_max_tid_flag ) |  |  |
| vps_dpb_max_tid[ i ] | u(3) |  |
| dpb_parameters( 1, vps_dpb_max_tid[ i ], vps_sublayer_dpb_params_present_flag ) |  |
| } |  |
| for( i = 0; i < NumMultiLayerOlss; i++ ) { |  |  |
| vps_ols_dpb_pic_width[ i ] | ue(v) |  |
| vps_ols_dpb_pic_height[ i ] | ue(v) |  |
| vps_ols_dpb_chroma_format[ i ] | u(2) |  |
| vps_ols_dpb_bitdepth_minus8[ i ] | ue(v) |  |
| if( VpsNumDpbParams > 1 && VpsNumDpbParams != NumMultiLayerOlss ) |  |  |
| vps_ols_dpb_params_idx[ i ] | ue(v) |  |
| } |  |  |
| ... |  |  |
| } |  |  |

#### 7.3.2.3	Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| ... |  |
| if( sps_ptl_dpb_hrd_params_present_flag ) { |  |
| if( sps_max_sublayers_minus1 > 0 ) |  |
| sps_sublayer_dpb_params_flag | u(1) |
| dpb_parameters( 0, sps_max_sublayers_minus1, sps_sublayer_dpb_params_flag ) |  |
| } |  |
| ... |  |
| } |  |

#### 7.3.4	DPB parameters syntax

| dpb_parameters( formatInfoFlag, maxSubLayersMinus1, subLayerInfoFlag ) { | Descriptor |
| --- | --- |
| if( formatInfoFlag ) { |  |
| dpb_ols_pic_width | ue(v) |
| dpb_ols_pic_height | ue(v) |
| dpb_ols_chroma_format | u(2) |
| dpb_ols_bitdepth_minus8 | ue(v) |
| } |  |
| for( i = ( subLayerInfoFlag ? 0 : maxSubLayersMinus1 ); i <= maxSubLayersMinus1; i++ ) { |  |
| max_dec_pic_buffering_minus1[ i ] | ue(v) |
| max_num_reorder_pics[ i ] | ue(v) |
| max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| } |  |

#### 7.4.3.2	Video parameter set RBSP semantics

...

vps_ols_dpb_pic_width[ i ] specifies the width, in units of luma samples, of each picture storage buffer for the i-th multi-layer OLS.

vps_ols_dpb_pic_height[ i ] specifies the height, in units of luma samples, of each picture storage buffer for the i-th multi-layer OLS.

vps_ols_dpb_chroma_format[ i ] specifies the greatest allowed value of sps_chroma_format_idc for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS.

vps_ols_dpb_bitdepth_minus8[ i ] specifies the greatest allowed value of sps_bitdepth_minus8 for all SPSs that are referred to by CLVSs in the CVS for the i-th multi-layer OLS.

NOTE 2 – For decoding the i-th multi-layer OLS, the deoder can safely allocate memory for the DPB according to the values of the syntax elements vps_ols_dpb_pic_width[ i ], vps_ols_dpb_pic_height[ i ], vps_ols_dpb_chroma_format[ i ], and vps_ols_dpb_bitdepth_minus8[ i ].

...

#### 7.4.3.3	Sequence parameter set RBSP semantics

...

sps_chroma_format_idc specifies the chroma sampling relative to the luma sampling as specified in clause 6.2.

When sps_video_parameter_set_id is greater than 0 and the SPS is referenced by a layer that is included in the i-th multi-layer OLS specified by the VPS for any i in the range of 0 to NumMultiLayerOlss − 1, inclusive, it is a requirement of bitstream conformance that the value of sps_chroma_format_idc shall be less than or equal to the value of dpb_ols_chroma_format in the vps_ols_dpb_params_idx[ i ]-th dpb_parameters( ) syntax structure in the VPSvps_ols_dpb_chroma_format[ i ].

...

sps_pic_width_max_in_luma_samples specifies the maximum width, in units of luma samples, of each decoded picture referring to the SPS. sps_pic_width_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ).

When sps_video_parameter_set_id is greater than 0 and the SPS is referenced by a layer that is included in the i-th multi-layer OLS specified by the VPS for any i in the range of 0 to NumMultiLayerOlss − 1, inclusive, it is a requirement of bitstream conformance that the value of sps_pic_width_max_in_luma_samples shall be less than or equal to the value of dpb_ols_pic_width in the vps_ols_dpb_params_idx[ i ]-th dpb_parameters( ) syntax structure in the VPSvps_ols_dpb_pic_width[ i ].

sps_pic_height_max_in_luma_samples specifies the maximum height, in units of luma samples, of each decoded picture referring to the SPS. sps_pic_height_max_in_luma_samples shall not be equal to 0 and shall be an integer multiple of Max( 8, MinCbSizeY ).

When sps_video_parameter_set_id is greater than 0 and the SPS is referenced by a layer that is included in the i-th multi-layer OLS specified by the VPS for any i in the range of 0 to NumMultiLayerOlss − 1, inclusive, it is a requirement of bitstream conformance that the value of sps_pic_height_max_in_luma_samples shall be less than or equal to the value of dpb_ols_pic_height in the vps_ols_dpb_params_idx[ i ]-th dpb_parameters( ) syntax structure in the VPSvps_ols_dpb_pic_height[ i ].

...

sps_bitdepth_minus8 specifies the bit depth of the samples of the luma and chroma arrays, BitDepth, and the value of the luma and chroma quantization parameter range offset, QpBdOffset, as follows:

BitDepth      = 8 + sps_bitdepth_minus8		(47)

QpBdOffset = 6 * sps_bitdepth_minus8		(48)

sps_bitdepth_minus8 shall be in the range of 0 to 8, inclusive.

When sps_video_parameter_set_id is greater than 0 and the SPS is referenced by a layer that is included in the i-th multi-layer OLS specified by the VPS for any i in the range of 0 to NumMultiLayerOlss − 1, inclusive, it is a requirement of bitstream conformance that the value of sps_bitdepth_minus8 shall be less than or equal to the value of dpb_ols_bitdepth_minus8 in the vps_ols_dpb_params_idx[ i ]-th dpb_parameters( ) syntax structure in the VPS vps_ols_dpb_bitdepth_minus8[ i ].

...

#### 7.4.5	DPB parameters semantics

...

When a dpb_parameters( ) syntax structure is included in a VPS, the OLSs to which the dpb_parameters( ) syntax structure applies are specified by the VPS. When a dpb_parameters( ) syntax structure is included in an SPS, it applies to the OLS that includes only the layer that is the lowest layer among the layers that refer to the SPS, and this lowest layer is an independent layer.

dpb_ols_pic_width specifies the width, in units of luma samples, of each picture storage buffer for each of the OLSs to which the dpb_parameters( ) syntax structure applies.

dpb_ols_pic_height specifies the height, in units of luma samples, of each picture storage buffer for each of the OLSs to which the dpb_parameters( ) syntax structure applies.

dpb_ols_chroma_format specifies the greatest allowed value of sps_chroma_format_idc for all SPSs that are referred to by CLVSs in the CVS for each of the OLSs to which the dpb_parameters( ) syntax structure applies.

dpb_ols_bitdepth_minus8 specifies the greatest allowed value of sps_bitdepth_minus8 for all SPSs that are referred to by CLVSs in the CVS for each of the OLSs to which the dpb_parameters( ) syntax structure applies.

NOTE – For decoding each of the OLSs to which the dpb_parameters( ) syntax structure applies, the deoder can safely allocate memory for the DPB according to the values of the syntax elements dpb_ols_pic_width, dpb_ols_pic_height, dpb_ols_chroma_format, and dpb_ols_bitdepth_minus8.

...

#### A.4.1	General tier and level limits

...

- For an OLS with OLS index TargetOlsIdx, the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, and PicSizeMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure are derived as follows:

  - If NumLayersInOls[ TargetOlsIdx ] is equal to 1, PicWidthMaxInSamplesY is set equal to sps_pic_width_max_in_luma_samples, PicHeightMaxInSamplesY is set equal to sps_pic_height_max_in_luma_samples, and PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, where  sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples are found in the SPS referred to by the layer in the OLS, and the applicable dpb_parameters( ) syntax structure is also found in that SPS.

  - Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), PicWidthMaxInSamplesY is set equal to dpb_ols_pic_width vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ], PicHeightMaxInSamplesY is set equal to dpb_ols_pic_height vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ], PicSizeMaxInSamplesY is set equal to PicWidthMaxInSamplesY * PicHeightMaxInSamplesY, and the applicable dpb_parameters( ) syntax structure is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS.

...

#### C.1	(HRD) General

...

For each bitstream conformance test, the CPB size (number of bits) is CpbSize[ Htid ][ ScIdx ] as specified in clause 7.4.6.3, where ScIdx and the HRD parameters are specified above in this clause, and the DPB parameters max_dec_pic_buffering_minus1[ Htid ], max_num_reorder_pics[ Htid ], and MaxLatencyPictures[ Htid ] are found in or derived from the dpb_parameters( ) syntax structure that applies to the target OLS as follows:

–	If NumLayersInOls[ TargetOlsIdx ] is equal to 1, the dpb_parameters( ) syntax structure is found in the SPS referred to be the layer in the target OLS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to sps_pic_width_max_in_luma_samples, sps_pic_height_max_in_luma_samples, sps_chroma_format_idc, and sps_bitdepth_minus8, respectively, found in the SPS referred to by the layer in the target OLS.

–	Otherwise (NumLayersInOls[ TargetOlsIdx ] is greater than 1), the applicable dpb_parameters( ) syntax structure is identified by vps_ols_dpb_params_idx[ MultiLayerOlsIdx[ TargetOlsIdx ] ] found in the VPS, and the variables PicWidthMaxInSamplesY, PicHeightMaxInSamplesY, MaxChromaFormat, and MaxBitDepthMinus8 are set equal to dpb_ols_pic_width, dpb_ols_pic_height, dpb_ols_chroma_format, and dpb_ols_bitdepth_minus8, vps_ols_dpb_pic_width[ MultiLayerOlsIdx[ TargetOlsIdx ] ], vps_ols_dpb_pic_height[ MultiLayerOlsIdx[ TargetOlsIdx ] ], vps_ols_dpb_chroma_format[ MultiLayerOlsIdx[ TargetOlsIdx ] ], and vps_ols_dpb_bitdepth_minus8[ MultiLayerOlsIdx[ TargetOlsIdx ] ], respectively, found in the applicable dpb_parameters( ) syntax structure VPS.

...

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).