| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0165-v1 |
| --- | --- |

| Title: | AHG9: Cleanup on signalling virtual boundaries |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Shih-Ta Hsiang, Lulin Chen, Olena Chubach, Yu-Wen Huang, Shaw-Min Lei No. 1, Dusing 1st Rd., Hsinchu City 30078, Taiwan | Tel: Email: | +886-3-5670766 {shih-ta.hsiang, lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek Inc. |  |  |

_____________________________

# Abstract

In this contribution following high-level syntax cleanups are proposed, related to signalling information on virtual boundaries:

- Proposal 1: The presence of sps_virtual_boundaries_present_flag is conditioned on sps_res_change_in_clvs_allowed_flag.

- Proposal 2: The presence of virtual boundaries is further conditioned on sps_subpic_info_present_flag.

  - Proposal 2a: The presence of ph_virtual_boundaries_present_flag is further conditioned on sps_subpic_info_present_flag

  - Proposal 2b: The presence of sps_virtual_boundaries_present_flag is further conditioned on sps_subpic_info_present_flag

# Proposals

## Proposal 1: The presence of sps_virtual_boundaries_present_flag conditioned on sps_res_change_in_clvs_allowed_flag

In VVC Draft 9, when the value of sps_res_change_in_clvs_allowed_flag is equal to 1, it is a requirement of bitstream conformance that the value of sps_virtual_boundaries_present_flag shall be equal to 0. In this contribution, it is proposed that the presence of sps_virtual_boundaries_present_flag is conditioned on sps_res_change_in_clvs_allowed_flag. When sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_virtual_boundaries_present_flag is inferred to be equal to 0. In this way, the bitsream conformance requirement will always be met under the syntax constraint, and related non-compliant bitstreams cannot be generated.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| if( !sps_res_change_in_clvs_allowed_flag ) |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_x_minus1[ i ] | ue(v) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_y_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of sps_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that when the value of sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_virtual_boundaries_present_flag shall be equal to 0.

## Proposal 2: The presence of virtual boundaries further conditioned on sps_subpic_info_present_flag

In VVC Draft 9, when the value of sps_subpic_info_present_flag is equal to 1, it is a requirement of bitstream conformance that the value of ph_virtual_boundaries_present_flag shall be equal to 0. In proposal 2, it is proposed that the presence of virtual boundaries is further conditioned on sps_subpic_info_present_flag.

### Proposal 2a: The presence of ph_virtual_boundaries_present_flag further conditioned on sps_subpic_info_present_flag

In Proposal 2a, it is proposed that the presence of ph_virtual_boundaries_present_flag is further conditioned on sps_subpic_info_present_flag. When sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0. In this way, the bitsream conformance requirement will always be met under the syntax constraint, and related non-compliant bitstreams cannot be generated.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

#### 7.3.2.7 Picture header structure syntax

| picture_header_structure ( ) { | Descriptor |
| --- | --- |
| gdr_or_irap_pic_flag | u(1) |
| … |  |
| if( sps_virtual_boundaries_enabled_flag && !sps_virtual_boundaries_present_flag && !sps_subpic_info_present_flag ) { |  |
| ph_virtual_boundaries_present_flag | u(1) |
| if( ph_virtual_boundaries_present_flag ) { |  |
| ph_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_ver_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_x_minus1[ i ] | ue(v) |
| ph_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < ph_num_hor_virtual_boundaries; i++ ) |  |
| ph_virtual_boundary_pos_y_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| …. |  |
| } |  |

#### 7.4.3.3 Picture header RBSP semantics

…

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

The variable VirtualBoundariesPresentFlag is derived as follows:

VirtualBoundariesPresentFlag = 0
if( sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPresentFlag = sps_virtual_boundaries_present_flag  | |
			ph_virtual_boundaries_present_flag		(84)

### Proposal 2b: The presence of sps_virtual_boundaries_present_flag further conditioned on sps_subpic_info_present_flag

In Proposal 2b, it is proposed that the presence of sps_virtual_boundaries_present_flag is further conditioned on sps_subpic_info_present_flag. When sps_virtual_boundaries_enabled_flag is equal to 1 and sps_subpic_info_present_flag is equal to 1, the value of sps_virtual_boundaries_present_flag is inferred to be equal to 1 and the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0. In this way, the bitsream conformance requirement will always be met under the syntax constraint, and related non-compliant bitstreams cannot be generated.

The proposed modifications to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| if( !sps_subpic_info_present_flag ) |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_x_minus1[ i ] | ue(v) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_y_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of sps_virtual_boundaries_present_flag is inferred to be equal to ( sps_virtual_boundaries_enabled_flag && sps_subpic_info_present_flag ) ? 1 : 0.

It is a requirement of bitstream conformance that when the value of sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_virtual_boundaries_present_flag shall be equal to 0.

#### 7.4.3.3 Picture header RBSP semantics

…

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

The variable VirtualBoundariesPresentFlag is derived as follows:

VirtualBoundariesPresentFlag = 0
if( sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPresentFlag = sps_virtual_boundaries_present_flag  | |
			ph_virtual_boundaries_present_flag		(84)

### Proposal 1+ Proposal 2b

The proposed modifications combining Proposal 1 and Proposal 2b to the VVC Draft 9 with integration of the May 2020 HLS AHG meeting outcome [1] are marked in red as follows:

#### 7.3.2.3 Sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| … |  |
| sps_virtual_boundaries_enabled_flag | u(1) |
| if( sps_virtual_boundaries_enabled_flag ) { |  |
| if( !sps_res_change_in_clvs_allowed_flag && !sps_subpic_info_present_flag ) |  |
| sps_virtual_boundaries_present_flag | u(1) |
| if( sps_virtual_boundaries_present_flag ) { |  |
| sps_num_ver_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_ver_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_x_minus1[ i ] | ue(v) |
| sps_num_hor_virtual_boundaries | u(2) |
| for( i = 0; i < sps_num_hor_virtual_boundaries; i++ ) |  |
| sps_virtual_boundary_pos_y_minus1[ i ] | ue(v) |
| } |  |
| } |  |
| … |  |
| } |  |

#### 7.4.3.3 Sequence parameter set RBSP semantics

sps_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the SPS. sps_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the SPS. When there is one or more than one virtual boundaries signalled in the SPS, the in-loop filtering operations are disabled across the virtual boundaries in pictures referring to the SPS. In-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of sps_virtual_boundaries_present_flag is inferred to be equal to ( sps_virtual_boundaries_enabled_flag && sps_subpic_info_present_flag ) ? 1 : 0.

It is a requirement of bitstream conformance that when the value of sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_virtual_boundaries_present_flag shall be equal to 0.

#### 7.4.3.3 Picture header RBSP semantics

…

ph_virtual_boundaries_present_flag equal to 1 specifies that information of virtual boundaries is signalled in the PH. ph_virtual_boundaries_present_flag equal to 0 specifies that information of virtual boundaries is not signalled in the PH. When there is one or more than one virtual boundaries signalled in the PH, the in-loop filtering operations are disabled across the virtual boundaries in the picture. The in-loop filtering operations include the deblocking filter, sample adaptive offset filter, and adaptive loop filter operations. When not present, the value of ph_virtual_boundaries_present_flag is inferred to be equal to 0.

It is a requirement of bitstream conformance that, when sps_subpic_info_present_flag is equal to 1, the value of ph_virtual_boundaries_present_flag shall be equal to 0.

The variable VirtualBoundariesPresentFlag is derived as follows:

VirtualBoundariesPresentFlag = 0
if( sps_virtual_boundaries_enabled_flag )
	VirtualBoundariesPresentFlag = sps_virtual_boundaries_present_flag  | |
			ph_virtual_boundaries_present_flag		(84)

# References

- Y.-K. Wang “AHG2: Editorial input of a text integration for the May 2020 HLS AHG meeting outcome,” Document of Joint Video Experts Team of ITU-T SG16 WP3 and ISO/IEC JTC1/SC29/WG11, JVET-S0152, 19th Meeting: by teleconference, 22 June – 1 July 2020.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).