| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June –1 July 2020 | Document: JVET-S0174-v2 |
| --- | --- |

| Title: | AHG9: Miscellaneous cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Robert Skupin Yago Sanchez Karsten Suehring Thomas Schierl | Tel: Email: | {firstname.lastname}@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI |  |  |

_____________________________

# Abstract

This document reports three asserted miscellaneous issues and proposes cleanups to resolve the issues.

Item 1) CTU size constraint for ILRP as collocated reference picture

Item 2) EOS NAL presence in multi-layer bitstreams

Item 3) Undefined value of ph_collocated_from_l0_flag when L0 empty and L1 non-empty

Revision 1 of this document adds a tighter constraint variant to item 1 enforcing equal CTU sizes for ILRP as collocated reference picture after offline discussion.

# Introduction

Item 1) CTU size constraint for ILRP as collocated reference picture

This contribution asserts that it is an oversight that an ILRP with a different value of sps_log2_ctu_size_minus5 than the respective value of the current picture can be used as collocated reference picture.

Considering first the single layer case: the collocated reference picture of any trailing or RADL picture in the same layer is part of the same CLVS with a single value of sps_log2_ctu_size_minus5 in the referred to SPS. When the collocated reference picture of any RASL picture in the same layer has different value of sps_log2_ctu_size_minus5, it belongs to another CLVS and an EOS NAL unit or external means will trigger the proper CLVSS behavior (DPB flush, non-available RefPic generation and no output of RASL). Hence, stored MVs of a collocated reference pictures with different value of sps_log2_ctu_size_minus5 than the current picture are not accessed in single layer coding. The authors assert that the rational for this design is that implementations may observe difficulties when MV data is accessed in a fashion unaligned with the reference picture CTU boundaries.

However, in the multi-layer case, the latest VVC draft specification allows the MV storage of a collocated reference picture be to accessed by current pictures of different layers with different values of sps_log2_ctu_size_minus5 in the referred to SPS. Even multiple layer pictures with varying values of sps_log2_ctu_size_minus5 may use the same ILRP as collocated reference picture in the AU.

This means that the MV candidates accessed during decoding of a single CTU of the current picture, may be spread over several CTU lines of the collocated reference picture and hence, the respective data may be interspersed in memory with the intermediate CTUs in the two (or more) lines. Note that for the clipping involved in sub-block TMVP, CTU boundaries of the current picture are used.

It is asserted that this behavior is an oversight and undesirable complexity increase and should be prohibited by enforcing a constraint for the value of sps_log2_ctu_size_minus5 of collocated reference picture and the current picture. The authors assert that a greater CTU size of the collocated reference picture compared to the current picture does not have the described problem and thus does not increase the implementation complexity significantly. Therefore, it is proposed to add the following condition to the semantics of sh_collocated_ref_idx:

sh_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

[…]

Let colPicList be set equal to sh_collocated_from_l0_flag ? 0 : 1. It is a requirement of bitstream conformance that the picture referred to by sh_collocated_ref_idx shall be the same for all non-I slices of a coded picture and the value of RprConstraintsActiveFlag[ colPicList ][ sh_collocated_ref_idx ] shall be equal to 0 and the value of sps_log2_ctu_size_minus5 in the SPS referred to by the picture referred to by sh_collocated_ref_idx shall be equal to or greater than the value of sps_log2_ctu_size_minus5 in the SPS referred to by the current picture.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution. the same scaling window offsets and same or larger CTU size as the current picture.

Offline discussion pointed out memory access issues for the case that the collocated reference picture has a greater CTU size compared to the current picture. In order to address said concerns, a tighter constraint is proposed as follows.

sh_collocated_ref_idx specifies the reference index of the collocated picture used for temporal motion vector prediction.

[…]

Let colPicList be set equal to sh_collocated_from_l0_flag ? 0 : 1. It is a requirement of bitstream conformance that the picture referred to by sh_collocated_ref_idx shall be the same for all non-I slices of a coded picture and the value of RprConstraintsActiveFlag[ colPicList ][ sh_collocated_ref_idx ] shall be equal to 0 and the value of sps_log2_ctu_size_minus5 in the SPS referred to by the picture referred to by sh_collocated_ref_idx shall be equal to the value of sps_log2_ctu_size_minus5 in the SPS referred to by the current picture.

NOTE – The above constraint requires the collocated picture to have the same spatial resolution, the same scaling window offsets, the same number of subpictures and same CTU size as the current picture.

The turquoise highlight adds mentioning of the requirement that the number of subpictures in the collocated reference picture must be equal to the number of subpictures in the current picture to the Note below the semantics of sh_collocated_ref_idx for the sake of completeness.

Item 2) EOS in multi-layer bitstreams

When a bitstream contains more than one layer and an EOS NAL unit is present in a reference layer, it may occur that dependent layer pictures may be impacted through faulty reference pictures. For instance, consider the example shown in Figure 1, where layer L0 contains a CRA NAL unit at AU4 that is a CLVSS picture as an EOS NAL unit is inserted in the bitstream directly preceding the CRA NAL unit. The CLVSS picture has NoOutputBeforeRecoveryFlag equal to 1 and the RASL picture, which cannot be correctly reconstructed because of missing references from before the CLVSS picture, is omitted from output.

Figure 1: Exemplary two layer CVS with unequal IRAP period and EOS NAL unit in the lower layer.

An issue of the bitstream is that during continuous decoding from AU 0 on, when the other pictures in the AU 5 of the RASL picture use the RASL picture as reference picture, e.g. the other picture (AU 5 trailing picture) would also be reconstructed incorrectly and subsequently output.

While one option to solve this issue (prevent output of incorrectly reconstructed pictures) would be to simply forbid the red highlighted ILRP reference to RASL pictures. In this case, all L1 pictures could be decoded and output correctly.  However, the authors assert that this general constraint is more restrictive than necessary, as this coding structure works fine without an EOS NAL unit or when random accessing layer L0 at the CRA AU4.

Therefore, it is proposed to condition the constraint (disallowing RASL as ILRP reference picture) on the condition that either an EOS NAL unit before the associated CRA picture of an RASL picture is present or the associated CRA has NoOutputBeforeRecoveryFlag and HandleCraAsClvsStartFlag both is set to 1 by external means. The two cases both have NoOutputBeforeRecoveryFlag equal to 1 which is used in the following proposed change to Section 8.3.2 of the VVC draft specification:

- The following constraints apply for the picture referred to by each ILRP entry, when present, in RefPicList[ 0 ] or RefPicList[ 1 ] of a slice of the current picture:

  - The picture shall be in the same AU as the current picture.

  - The picture shall be present in the DPB.

  - The picture shall have nuh_layer_id refPicLayerId less than the nuh_layer_id of the current picture.

  - The picture shall not be a RASL picture when the associated CRA has NoOutputBeforeRecoveryFlag set to 1 and the current picture is not a RASL.

  - The picture shall not precede the recovery point picture in decoding order of the associated GDR in the layer with refPicLayerId that has NoOutputBeforeRecoveryFlag set to 1.

  - Either of the following constraints applies:

  - The picture shall be an IRAP picture.

  - The picture shall have TemporalId less than or equal to Max( 0, vps_max_tid_il_ref_pics_plus1[ currLayerIdx ][ refLayerIdx ] − 1 ), where currLayerIdx and refLayerIdx are equal to GeneralLayerIdx[ nuh_layer_id ] and GeneralLayerIdx[ refpicLayerId ], respectively.

Alternatively, another option to solve the issue is to require EOS NAL units to be aligned across layers in such a situation as follows

It is a requirement of bitstream conformance that when a layer k that depends on a layer l and layer l contains an EOS NAL unit in AU n, layer k shall also contain an EOS NAL in AU n or the next AU n + 1 must contain a CLVSS picture for layer k.

Item 3) Undefined value of ph_collocated_from_l0_flag when L0 empty and L1 non-empty

At the intermediate AhG meeting, JVET-S0096 was reviewed and aspect 1 was recommended for adoption (adding “num_ref_entries[ 0 ][ RplsIdx[ 0 ] ] > 0 &&” condition to ph_collocated_from_l0_flag).

The reasoning as noted in the AhG notes JVET-S0137 is as follows.

If list 0 is empty, we should not be able to indicate that we are getting the collocated picture from that list.

List 0 could only be empty for an I slice. Usually, for an I slice, if we have a non-empty list, we would make that list 0, but we do not have a constraint that requires that. It was remarked that we might not want to establish such a constraint.

The existing syntax condition seems to have a built-in assumption that list 0 will always be non-empty. This is not a valid assumption.

While aspect 1 of JVET-S0096 was recommended for adoption and added the condition (signal the SE only when both lists are non-empty), the respective inference was not changed by the proposal and still triggers only when L1 is empty as seen from the following:

When ph_temporal_mvp_enabled_flag and pps_rpl_info_in_ph_flag are both equal to 1 and num_ref_entries[ 1 ][ RplsIdx[ 1 ] ] is equal to 0, the value of ph_collocated_from_l0_flag is inferred to be equal to 1.

In the case that L0 is empty and L1 is non-empty, the discussed SE is not signalled and not inferred, i.e. its value remains undefined, while it is required to determine the presence of ph_collocated_ref_idx in the PH. This document proposes to fix the inference to point to the non-empty list, when only one list is empty.

ph_collocated_from_l0_flag equal to 1 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 0. ph_collocated_from_l0_flag equal to 0 specifies that the collocated picture used for temporal motion vector prediction is derived from reference picture list 1. When not present and ph_temporal_mvp_enabled_flag and pps_rpl_info_in_ph_flag are both equal to 1, the value of ph_collocated_from_l0_flag is inferred to be equal to (num_ref_entries[ 1 ][ RplsIdx[ 1 ] ]  = =  0 ? 1 : 0).

# Patent rights declaration(s)

Fraunhofer HHI may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form)