| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0177-v2 |
| --- | --- |

| Title: | AHG9: On the scalable nesting SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ye-Kui Wang1, Zhipin Deng2 18910 University Center Lane San Diego, CA 92122, USA 2Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China | Tel: Email: | yekui.wang@bytedance.com zhipin.deng@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution proposes the following changes to the scalable nesting SEI message syntax and semantics:

- Change sn_subpic_id[ i ] to sn_subpic_idx[ i ] and remove_subpic_id_len_minus1.

- Disallow containing of filler payload SEI messages in a scalable nesting SEI message.

- Constraint that, when a scalable nesting SEI message contains one or more subpicture level information (SLI) SEI messages, the value of sn_ols_flag shall be equal to 1, and the value of sn_subpic_flag shall be equal to 0.

- Modify an existing constraint, to included the SLI message, such that when a scalable nesting SEI message contains a BP, PT, DUI, or SLI SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), 130 (DUI), or 203 (SLI).

- Modify an existing constraint, to include recently added SEI messages, such that when a scalable nesting SEI message contains an SEI message that has payloadType not equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT.

- Add a constraint such that when a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT.

- Specify the semantics of sn_num_subpics_minus1 and sn_subpic_idx[ i ] in a way such that the syntax elements are about the subpictures of the layers with multiple subpictures per picture.

# Introduction

The existing design of the scalable nesting message in the latest VVC draft text has the following issues:

- For associating a scalable-nested SEI message to one or more subpictures, the scalable nesting SEI message uses subpicture IDs. However, the persistency scope of a scalable-nested SEI message can be a number of consecutive AUs, while the subpicture ID of the subpictures with a particular subpicture index in a layer can change within a CLVS. Therefore, instead of using subpicture IDs, subpicture indices should be used in the scalable nesting SEI message.

- Filler payload SEI messages, when present, need to be removed from the output bitstream in the subpicture sub-bitstream extraction process when the associated subpictures are removed. However, when it is possible to include filler payload SEI messages in a scalable nesting SEI message, the removal of filler payload SEI messages in the subpicture sub-bitstream extraction process would sometimes need to extract some scalable-nested SEI messages from a scalable nesting SEI message.

- Since the SLI SEI message applies to OLSs, like the other three HRD related SEI messages (i.e., the BP/PT/DUI SEI messages), when SLI SEI messages are scalable-nested, the value of sn_ols_flag need to be equal to 1. Furthermore, since the SLI SEI message specifies information for all subpictures in the pictures in the OLSs to which the SLI SEI message applies, it does not make sense for the value of sn_subpic_flag to be equal to 1 for a scalable nesting SEI message containing SLI SEI messages.

- In the constraint that a scalable nesting SEI message containing BP, PT, DUI SEI messages shall not contain other SEI messages, the SLI SEI message is not included. However, the SLI SEI message is also HRD related, and should be treated similarly.

- It is specified that when a scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (BP), 1 (PT), 130 (DUI), 145 (DRAP indication), or 168 (frame-field information), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT. However, when nesting many other SEI messages that have been recently added to VVC, the value of the scalable nesting SEI message shall also have nal_unit_type equal to PREFIX_SEI_NUT.

- There lacks a constraint that, when a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT.

- The semantics of sn_num_subpics_minus1 and sn_subpic_idx[ i ] need to be specified in a way such that the syntax elements are about the subpictures of the layers with multiple subpictures per picture, to be able to support cases wherein an OLS has some layers with multiple subpictures per picture and some other layers with a single subpicture per picture.

# Proposal

The proposal for addressing the issues above is summarized as above in the abstract. The proposed text changes are as follows, where added parts are highlighted in turquoise, and deleted parts are highlighted in yellow strikethrough in red fonts:

#### D.6.1	Scalable nesting SEI message syntax

| scalable_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| sn_ols_flag | u(1) |
| sn_subpic_flag | u(1) |
| if( sn_ols_flag ) { |  |
| sn_num_olss_minus1 | ue(v) |
| for( i = 0; i <= sn_num_olss_minus1; i++ ) |  |
| sn_ols_idx_delta_minus1[ i ] | ue(v) |
| } else { |  |
| sn_all_layers_flag | u(1) |
| if( !sn_all_layers_flag ) { |  |
| sn_num_layers_minus1 | ue(v) |
| for( i = 1; i <= sn_num_layers_minus1; i++ ) |  |
| sn_layer_id[ i ] | u(6) |
| } |  |
| } |  |
| if( sn_subpic_flag ) { |  |
| sn_num_subpics_minus1 | ue(v) |
| sn_subpic_id_len_minus1 | ue(v) |
| for( i = 0; i <= sn_num_subpics_minus1; i++ ) |  |
| sn_subpic_idx[ i ] | ue(v) |
| } |  |
| sn_num_seis_minus1 | ue(v) |
| while( !byte_aligned( ) ) |  |
| sn_zero_bit /* equal to 0 */ | u(1) |
| for( i = 0; i <= sn_num_seis_minus1; i++ ) |  |
| sei_message( ) |  |
| } |  |

#### D.6.2	Scalable nesting SEI message semantics

The scalable nesting SEI message provides a mechanism to associate SEI messages with specific OLSs or with specific layers as well as to associate SEI messages with specific sets of subpictures.

A scalable nesting SEI message contains one or more SEI messages. The SEI messages contained in the scalable nesting SEI message are also referred to as the scalable-nested SEI messages.

It is a requirement of bitstream conformance that the following restrictions apply on containing of SEI messages in a scalable nesting SEI message:

–	An SEI message that has payloadType equal to 132 (decoded picture hash) shall only be contained in a scalable nesting SEI message with sn_subpic_flag equal to 1.

–	An SEI message that has payloadType equal to 3 (filler payload) or 133 (scalable nesting) shall not be contained in a scalable nesting SEI message.

–	When a scalable nesting SEI message contains a BP, PT, or DUI, or SLI SEI message, the scalable nesting SEI message shall not contain any other SEI message with payloadType not equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI).

It is a requirement of bitstream conformance that the following restriction applies on the value of the nal_unit_type of the SEI NAL unit containing a scalable nesting SEI message:

–	When a scalable nesting SEI message contains an SEI message that has payloadType not equal to 132 (decoded picture hash) equal to 0 (BP), 1 (PT), 130 (DUI), 145 (DRAP indication), or 168 (frame-field information), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to PREFIX_SEI_NUT.

–	When a scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the SEI NAL unit containing the scalable nesting SEI message shall have nal_unit_type equal to SUFFIX_SEI_NUT.

sn_ols_flag equal to 1 specifies that the scalable-nested SEI messages apply to specific OLSs. sn_ols_flag equal to 0 specifies that the scalable-nested SEI messages apply to specific layers.

It is a requirement of bitstream conformance that the following restrictions apply on the value of sn_ols_flag:

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to 0 (BP), 1 (PT), or 130 (DUI), or 203 (SLI), the value of sn_ols_flag shall be equal to 1.

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to a value in NestingForLayersSeiList but not equal to 203 (SLI), the value of sn_ols_flag shall be equal to 0.

sn_subpic_flag equal to 1 specifies that the scalable-nested SEI messages that apply to specified OLSs or layers apply only to specific subpictures of the specified OLSs or layers. sn_subpic_flag equal to 0 specifies that the scalable-nested SEI messages that apply to specific OLSs or layers apply to all subpictures of the specified OLSs or layers.

It is a requirement of bitstream conformance that the following restrictions apply on the value of sn_subpic_flag:

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to 132 (decoded picture hash), the value of sn_subpic_flag shall be equal to 1.

–	When the scalable nesting SEI message contains an SEI message that has payloadType equal to 203 (SLI), the value of sn_subpic_flag shall be equal to 0.

sn_num_olss_minus1 plus 1 specifies the number of OLSs to which the scalable-nested SEI messages apply. The value of sn_num_olss_minus1 shall be in the range of 0 to TotalNumOlss − 1, inclusive.

sn_ols_idx_delta_minus1[ i ] is used to derive the variable NestingOlsIdx[ i ] that specifies the OLS index of the i-th OLS to which the scalable-nested SEI messages apply when sn_ols_flag is equal to 1. The value of sn_ols_idx_delta_minus1[ i ] shall be in the range of 0 to TotalNumOlss − 2, inclusive, inclusive.

The variable NestingOlsIdx[ i ] is derived as follows:

if( i  = =  0 )
	NestingOlsIdx[ i ] = sn_ols_idx_delta_minus1[ i ]	(D.4)
else
	NestingOlsIdx[ i ] = NestingOlsIdx[ i − 1 ] + sn_ols_idx_delta_minus1[ i ] + 1

sn_all_layers_flag equal to 1 specifies that the scalable-nested SEI messages apply to all layers that have nuh_layer_id greater than or equal to the nuh_layer_id of the current SEI NAL unit. sn_all_layers_flag equal to 0 specifies that the scalable-nested SEI messages may or may not apply to all layers  that have nuh_layer_id greater than or equal to the nuh_layer_id of the current SEI NAL unit.

sn_num_layers_minus1 plus 1 specifies the number of layers to which the scalable-nested SEI messages apply. The value of sn_num_layers_minus1 shall be in the range of 0 to vps_max_layers_minus1 − GeneralLayerIdx[ nuh_layer_id ], inclusive, where nuh_layer_id is the nuh_layer_id of the current SEI NAL unit.

sn_layer_id[ i ] specifies the nuh_layer_id value of the i-th layer to which the scalable-nested SEI messages apply when sn_all_layers_flag is equal to 0. The value of sn_layer_id[ i ] shall be greater than nuh_layer_id, where nuh_layer_id is the nuh_layer_id of the current SEI NAL unit.

When sn_ols_flag is equal to 0, the variable NestingNumLayers, specifying the nubmer of layer to which the scalable-nested SEI messages apply, and the list NestingLayerId[ i ] for i in the range of 0 to NestingNumLayers − 1, inclusive, specifying the list of nuh_layer_id value of the layers to which the scalable-nested SEI messages apply, are derived as follows, where nuh_layer_id is the nuh_layer_id of the current SEI NAL unit:

if( sn_all_layers_flag ) {
	NestingNumLayers = vps_max_layers_minus1 + 1 − GeneralLayerIdx[ nuh_layer_id ]
	for( i = 0; i < NestingNumLayers; i ++)
		NestingLayerId[ i ] = vps_layer_id[ GeneralLayerIdx[ nuh_layer_id ] + i ]	(D.5)
} else {
	NestingNumLayers = sn_num_layers_minus1 + 1
	for( i = 0; i < NestingNumLayers; i ++)
		NestingLayerId[ i ] = ( i  = = 0 ) ? nuh_layer_id : sn_layer_id[ i ]
}

Among the layers in the OLSs (when sn_ols_flag is equal to 1) to which the scalable-nested SEI messages apply, or among the layers (when sn_ols_flag is equal to 0) to which the scalable-nested SEI messages apply, those for which the referenced SPSs have sps_num_subpics_minus1 greater than 0 are referred to as the multiSubpicLayers.

sn_num_subpics_minus1 plus 1 specifies the number of subpictures in each picture in the multiSubpicLayers to which the scalable-nested SEI messages apply. The value of sn_num_subpics_minus1 shall be less than or equal to the value of sps_num_subpics_minus1 in the SPSs referred to by the pictures in the multiSubpicLayers CLVS.

sn_subpic_id_len_minus1 plus 1 specifies the number of bits used to represent the syntax element sn_subpic_id[ i ]. The value of sn_subpic_id_len_minus1 shall be in the range of 0 to 15, inclusive.

It is a requirement of bitstream conformance that the value of sn_subpic_id_len_minus1 shall be the same for all scalable nesting SEI messages that are present in a CLVS.

sn_subpic_idx[ i ] indicates specifies the subpicture index of the i-th subpicture ID associated with the scalable-nested SEI messages in each picture in the multiSubpicLayers. The value of sn_subpic_idx[ i ] shall be less than or equal to the value of sps_num_subpics_minus1 in the SPSs referred to by the pictures in the multiSubpicLayers. The length of the sn_subpic_id[ i ] syntax element is sn_subpic_id_len_minus1 + 1 bits. The scalable-nested SEI messages also apply to the single subpicure in each picture in the layers that are not in the multiSubpicLayers but are among the layers in the OLSs (when sn_ols_flag is equal to 1) to which the scalable-nested SEI messages apply or among the layers (when sn_ols_flag is equal to 0) to which the scalable-nested SEI messages apply.

sn_num_seis_minus1 plus 1 specifies the number of scalable-nested SEI messages. The value of sn_num_seis_minus1 shall be in the range of 0 to 63, inclusive.

sn_zero_bit shall be equal to 0.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).