| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0185-v1 |
| --- | --- |

| Title: | AHG9: On HRD Cleanups |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following are proposed as cleanups:

- Proposal 1: It is proposed to move the syntax element pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ]  to locate it near the other CPB removal delay syntax elements, in particular other pt_cpb_removal_delay_minus1[ i ] syntax elements (and pt_cpb_removal_delay_delta_idx[ i ] syntax elements) for i in the range of TemporalId to bp_max_sub_layers_minus1− 1, inclusive.

- Proposal 2: It is proposed to move the signalling of syntax element bp_alt_cpb_params_present_flag to locate it near the syntax element bp_use_alt_cpb_params_flag where it is used.

# Introduction

VVC Draft text [1] includes a design of HRD including a picture timing SEI message and buffering period SEI message signalling.

It is asserted that some syntax element position rearrangement is desirable for PT and BP SEI message syntax.

Following are proposed as cleanups:

- Proposal 1: It is proposed to move the syntax element pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ]  to locate it near the other CPB removal delay syntax elements, in particular other pt_cpb_removal_delay_minus1[ i ] syntax elements (and pt_cpb_removal_delay_delta_idx[ i ] syntax elements) for i in the range of TemporalId to bp_max_sub_layers_minus1− 1, inclusive.

- Proposal 2: It is proposed to move the signalling of syntax element bp_alt_cpb_params_present_flag to locate it near the syntax element bp_use_alt_cpb_params_flag where it is used.

# Proposals

## Proposal 1

It is asserted that some syntax element position rearrangement is desirable for PT SEI message syntax.

It is proposed to move the syntax element pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ] to locate it near the other CPB removal delay syntax elements, in particular other pt_cpb_removal_delay_minus1[ i ] syntax elements  (and pt_cpb_removal_delay_delta_idx[ i ] syntax elements) for i in the range of TemporalId to bp_max_sub_layers_minus1− 1, inclusive.

It is asserted that signalling pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ] away from other pt_cpb_removal_delay_minus1[ TemporalId ], … , pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 − 1 ] is confusing and a collocated syntax will be beneficial for parsing and filling up the entire array of syntax elements pt_cpb_removal_delay_minus1[ i ] for i in the range of TemporalId to bp_max_sub_layers_minus1, inclusive.

The proposed changes are shown highlighted compared with VVC Draft text (JVET-S0152).

D.4.1 Picture timing SEI message syntax

| pic_timing( payloadSize ) { | Descriptor |
| --- | --- |
| pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ] | u(v) |
| if( bp_alt_cpb_params_present_flag ) { |  |
| pt_cpb_alt_timing_info_present_flag | u(1) |
| if( pt_cpb_alt_timing_info_present_flag ) { |  |
| if( bp_nal_hrd_params_present_flag ) { |  |
| for( i = ( bp_sublayer_initial_cpb_removal_delay_present_flag ? 0 : bp_max_sublayers_minus1 ); i <= bp_max_sublayers_minus1; i++ ) { |  |
| for( j = 0; j < bp_cpb_cnt_minus1 + 1; j++ ) { |  |
| pt_nal_cpb_alt_initial_removal_delay_delta[ i ][ j ] | u(v) |
| pt_nal_cpb_alt_initial_removal_offset_delta[ i ][ j ] | u(v) |
| } |  |
| pt_nal_cpb_delay_offset[ i ] | u(v) |
| pt_nal_dpb_delay_offset[ i ] | u(v) |
| } |  |
| } |  |
| if( bp_vcl_hrd_params_present_flag ) { |  |
| for( i = ( bp_sublayer_initial_cpb_removal_delay_present_flag ? 0 : bp_max_sublayers_minus1 ); i <= bp_max_sublayers_minus1; i++ ) { |  |
| for( j = 0; j < bp_cpb_cnt_minus1 + 1; j++ ) { |  |
| pt_vcl_cpb_alt_initial_removal_delay_delta[ i ][ j ] | u(v) |
| pt_vcl_cpb_alt_initial_removal_offset_delta[ i ][ j ] | u(v) |
| } |  |
| pt_vcl_cpb_delay_offset[ i ] | u(v) |
| pt_vcl_dpb_delay_offset[ i ] | u(v) |
| } |  |
| } |  |
| } |  |
| } |  |
| pt_cpb_removal_delay_minus1[ bp_max_sublayers_minus1 ] | u(v) |
| for( i = TemporalId; i < bp_max_sublayers_minus1; i++ ) { |  |
| pt_sublayer_delays_present_flag[ i ] | u(1) |
| if( pt_sublayer_delays_present_flag[ i ] ) { |  |
| if( bp_cpb_removal_delay_deltas_present_flag ) |  |
| pt_cpb_removal_delay_delta_enabled_flag[ i ] | u(1) |
| if( pt_cpb_removal_delay_delta_enabled_flag[ i ] ) |  |
| if( bp_num_cpb_removal_delay_deltas_minus1 > 0 ) |  |
| pt_cpb_removal_delay_delta_idx[ i ] | u(v) |
| else |  |
| pt_cpb_removal_delay_minus1[ i ] | u(v) |
| } |  |
| } |  |
| pt_dpb_output_delay | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag && bp_du_dpb_params_in_pic_timing_sei_flag ) |  |
| pt_dpb_output_du_delay | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag && bp_du_cpb_params_in_pic_timing_sei_flag ) { |  |
| pt_num_decoding_units_minus1 | ue(v) |
| if( pt_num_decoding_units_minus1 > 0 ) { |  |
| pt_du_common_cpb_removal_delay_flag | u(1) |
| if( pt_du_common_cpb_removal_delay_flag ) |  |
| for( i = TemporalId; i <= bp_max_sublayers_minus1; i++ ) |  |
| if( pt_sublayer_delays_present_flag[ i ] ) |  |
| pt_du_common_cpb_removal_delay_increment_minus1[ i ] | u(v) |
| for( i = 0; i <= pt_num_decoding_units_minus1; i++ ) { |  |
| pt_num_nalus_in_du_minus1[ i ] | ue(v) |
| if( !pt_du_common_cpb_removal_delay_flag && i < pt_num_decoding_units_minus1 ) |  |
| for( j = TemporalId; j <= bp_max_sublayers_minus1; j++ ) |  |
| if( pt_sublayer_delays_present_flag[ j ] ) |  |
| pt_du_cpb_removal_delay_increment_minus1[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( bp_additional_concatenation_info_present_flag ) |  |
| pt_delay_for_concatenation_ensured_flag | u(1) |
| pt_display_elemental_periods_minus1 | u(4) |
| } |  |

pt_cpb_removal_delay_minus1[ i ] plus 1 is used to calculate the number of clock ticks between the nominal CPB removal times of the AU associated with the PT SEI message and the preceding AU in decoding order that contains a BP SEI message when Htid is equal to i. This value is also used to calculate an earliest possible time of arrival of AU data into the CPB for the HSS. The length of pt_cpb_removal_delay_minus1[ i ] is bp_cpb_removal_delay_length_minus1 + 1 bits.

## Proposal 2

It is asserted that some syntax element position rearrangement is desirable for BP SEI message syntax.

It is proposed to move the signalling of syntax element bp_alt_cpb_params_present_flag to locate it near the syntax element bp_use_alt_cpb_params_flag where it is used.

The proposed modifications are shown highlighted compared to VVC Draft text (JVET-S0152).

D.3.1 Buffering period SEI message syntax

| buffering_period( payloadSize ) { | Descriptor |
| --- | --- |
| bp_nal_hrd_params_present_flag | u(1) |
| bp_vcl_hrd_params_present_flag | u(1) |
| bp_cpb_initial_removal_delay_length_minus1 | u(5) |
| bp_cpb_removal_delay_length_minus1 | u(5) |
| bp_dpb_output_delay_length_minus1 | u(5) |
| bp_alt_cpb_params_present_flag | u(1) |
| bp_decoding_unit_hrd_params_present_flag | u(1) |
| if( bp_decoding_unit_hrd_params_present_flag ) { |  |
| bp_du_cpb_removal_delay_increment_length_minus1 | u(5) |
| bp_dpb_output_delay_du_length_minus1 | u(5) |
| bp_du_cpb_params_in_pic_timing_sei_flag | u(1) |
| bp_du_dpb_params_in_pic_timing_sei_flag | u(1) |
| } |  |
| bp_concatenation_flag | u(1) |
| bp_additional_concatenation_info_present_flag | u(1) |
| if( bp_additional_concatenation_info_present_flag ) |  |
| bp_max_initial_removal_delay_for_concatenation | u(v) |
| bp_cpb_removal_delay_delta_minus1 | u(v) |
| bp_cpb_removal_delay_deltas_present_flag | u(1) |
| if( bp_cpb_removal_delay_deltas_present_flag ) { |  |
| bp_num_cpb_removal_delay_deltas_minus1 | ue(v) |
| for( i = 0; i <= bp_num_cpb_removal_delay_deltas_minus1; i++ ) |  |
| bp_cpb_removal_delay_delta_val[ i ] | u(v) |
| } |  |
| bp_max_sublayers_minus1 | u(3) |
| bp_cpb_cnt_minus1 | ue(v) |
| bp_sublayer_initial_cpb_removal_delay_present_flag | u(1) |
| for( i = ( bp_sublayer_initial_cpb_removal_delay_present_flag ? 0 : bp_max_sublayers_minus1 ); i <= bp_max_sublayers_minus1; i++ ) { |  |
| if( bp_nal_hrd_params_present_flag ) |  |
| for( j = 0; j < bp_cpb_cnt_minus1 + 1; j++ ) { |  |
| bp_nal_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| bp_nal_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag) { |  |
| bp_nal_initial_alt_cpb_removal_delay[ i ][ j ] | u(v) |
| bp_nal_initial_alt_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| if( bp_vcl_hrd_params_present_flag ) |  |
| for( j = 0; j < bp_cpb_cnt_minus1 + 1; j++ ) { |  |
| bp_vcl_initial_cpb_removal_delay[ i ][ j ] | u(v) |
| bp_vcl_initial_cpb_removal_offset[ i ][ j ] | u(v) |
| if( bp_decoding_unit_hrd_params_present_flag ) { |  |
| bp_vcl_initial_alt_cpb_removal_delay[ i ][ j ] | u(v) |
| bp_vcl_initial_alt_cpb_removal_offset[ i ][ j ] | u(v) |
| } |  |
| } |  |
| } |  |
| if( bp_max_sublayers_minus1 > 0 ) |  |
| bp_sublayer_dpb_output_offsets_present_flag | u(1) |
| if( bp_sublayer_dpb_output_offsets_present_flag ) |  |
| for( i = 0; i < bp_max_sublayers_minus1; i++ ) |  |
| bp_dpb_output_tid_offset[ i ] | ue(v) |
| bp_alt_cpb_params_present_flag | u(1) |
| if( bp_alt_cpb_params_present_flag ) |  |
| bp_use_alt_cpb_params_flag | u(1) |
| } |  |

bp_alt_cpb_params_present_flag equal to 1 specifies the presence of the syntax element bp_use_alt_cpb_params_flag in the BP SEI message and the presence of the alternative timing information in the PT SEI messages in the current BP. When not present, the value of bp_alt_cpb_params_present_flag is inferred to be equal to 0. When the associated picture is neither a CRA picture nor an IDR picture, the value of bp_alt_cpb_params_present_flag shall be equal to 0.

…

bp_use_alt_cpb_params_flag may be used to derive the value of UseAltCpbParamsFlag. When bp_use_alt_cpb_params_flag is not present, it is inferred to be equal to 0.

When one or more of the following condtions apply, UseAltCpbParamsFlag is set equal to 1:

–	bp_use_alt_cpb_params_flag is equal to 1.

–	When some external means not specified in this Specification is available to set UseAltCpbParamsFlag and the value of UseAltCpbParamsFlag is set equal to 1 by the external means.

# Conclusion

This document proposes modifications related to HRD related syntax order. It is proposed to adopt the proposals in the next version of VVC draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-S0152, AHG2: Editorial Input of a text integration for the May 2020 HLS AHG meeting outcome, May 2020