| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0186-v1 |
| --- | --- |

| Title: | AHG9: On SPS Cleanup |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Sachin Deshpande Jonatan Samuelsson Andrew Segall Philip Cowan | Tel: Email: | +1 360 817 8486 sdeshpande@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

Following is proposed as cleanups:

- Proposal 1: It is proposed to move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits.

- Proposal 2: It is proposed to keep sps_seq_parameter_set_id value equal to 15 as a reserved value for future extensibility.

# Introduction

VVC Draft text [1] includes a sequence parameter set (SPS).

Following is proposed as cleanups:

- Proposal 1: It is proposed to move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits.

- Proposal 2: It is proposed to keep sps_seq_parameter_set_id value equal to 15 as a reserved value for future extensibility.

# Proposal

## Proposal 1

It is proposed to move the two syntax elements (sps_chroma_format_idc and sps_log2_ctu_size_minus5) to take the place of sps_reserved_zero_4bits so as not to waste 4 bits in SPS.

It is asserted that the purpose of current 4 reserved bits is to provide a byte-alignment for the start of profile_tier_level( 1, sps_max_sublayers_minus1 ) syntax structure. However, it is asserted that there is no need to waste these 4 bits and they can instead be used, since SPS already has an extension mechanism which can be used in future if needed.

It is also asserted that no such reserved bits are included in PPS or VPS thus making a case for using parameter set extension mechanism in future if needed for any extensions instead of keeping such reserved bits.

The proposed changes are shown highlighted compared with VVC Draft text (JVET-S0152).

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_seq_parameter_set_id | u(4) |
| sps_video_parameter_set_id | u(4) |
| sps_max_sublayers_minus1 | u(3) |
| sps_chroma_format_idc | u(2) |
| sps_log2_ctu_size_minus5 | u(2) |
| sps_reserved_zero_4bits | u(4) |
| sps_ptl_dpb_hrd_params_present_flag | u(1) |
| if( sps_ptl_dpb_hrd_params_present_flag ) |  |
| profile_tier_level( 1, sps_max_sublayers_minus1 ) |  |
| sps_gdr_enabled_flag | u(1) |
| sps_chroma_format_idc | u(2) |
| sps_ref_pic_resampling_enabled_flag | u(1) |
| if( sps_ref_pic_resampling_enabled_flag ) |  |
| sps_res_change_in_clvs_allowed_flag | u(1) |
| sps_pic_width_max_in_luma_samples | ue(v) |
| sps_pic_height_max_in_luma_samples | ue(v) |
| sps_conformance_window_flag | u(1) |
| if( sps_conformance_window_flag ) { |  |
| sps_conf_win_left_offset | ue(v) |
| sps_conf_win_right_offset | ue(v) |
| sps_conf_win_top_offset | ue(v) |
| sps_conf_win_bottom_offset | ue(v) |
| } |  |
| sps_log2_ctu_size_minus5 | u(2) |
| sps_subpic_info_present_flag | u(1) |
| … |  |
| } |  |

## Proposal 2

It is asserted that VVC design allows some sharing of SPSs across layers, however there are constraints on this.

In VVC sps_seq_parameter_set_id is coded as u(4). As a result  a maximum of 16 SPSs with unique sps_seq_parameter_set_id are possible. In contrast with this HEVC coded sequence parameter set ID - sps_seq_parameter_set_id with a ue(v) coding, thus allowing much higher number of SPSs for future extensibility.

It is proposed to keep sps_seq_parameter_set_id value equal to 15 as a reserved value for future extensibility in VVC.

sps_seq_parameter_set_id provides an identifier for the SPS for reference by other syntax elements. The value of sps_seq_parameter_set_id shall be in the range of 0 to 14, inclusive. The value 15 for sps_seq_parameter_set_id is reserved for future use by ITU-T | ISO/IEC.

SPS NAL units, regardless of the nuh_layer_id values, share the same value space of sps_seq_parameter_set_id.

Let spsLayerId be the value of the nuh_layer_id of a particular SPS NAL unit, and vclLayerId be the value of the nuh_layer_id of a particular VCL NAL unit. The particular VCL NAL unit shall not refer to the particular SPS NAL unit unless spsLayerId is less than or equal to vclLayerId and all OLSs specified by the VPS that contain the layer with nuh_layer_id equal to vclLayerId also contain the layer with nuh_layer_id equal to spslayerId.

…

pps_seq_parameter_set_id specifies the value of sps_seq_parameter_set_id for the SPS. The value of pps_seq_parameter_set_id shall be in the range of 0 to 1514, inclusive. The value of pps_seq_parameter_set_id shall be the same in all PPSs that are referred to by coded pictures in a CLVS.

Additionally, the value 63 can be similarly kept reserved for pps_pic_parameter_set_id.

# Conclusion

This document proposes modification related to SPS. It is proposed to adopt the proposal in the next version of VVC draft text.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

# References

[1] JVET-S0152, AHG2: Editorial Input of a text integration for the May 2020 HLS AHG meeting outcome, May 2020