| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0189-v2 |
| --- | --- |

| Title: | AHG9/AHG8/AHG12: On subpicture bitstream extraction process |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the modification to the subpicture sub-bitstream extraction process, specified in C.7, so that the scaling window can be properly updated after the sub-bitstream extraction, especially when the reference picture resampling (RPR) is used for inter-prediction or interlayer prediction. The main aspect of the proposed modification is that the scaling window sizes of the current picture and the reference pictures are never changed, while only the left-top position of the scaling window is properly adjusted by updating the scaling window offset parameters (pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, pps_scaling_win_bottom_offset) in PPS.

# Problem statement

A picture in the layer k can be partitioned into multiple subpictures, and refers to reference pictures in the same layer and/or interlayer reference pictures in the reference layer of the layer k. In the example shown in Fig. 1, when the value of sps_ref_pic_resampling_enabled_flag is equal to 1 and sps_res_change_in_clvs_allowed_flag is equal to 0, the current picture and each reference picture may have different scaling windows, even though the picture size is the same.

The asserted problem is that the existing scaling windows are not available to the extracted subpictures, because the subpictures have different picture sizes than the original picture size. The values of the scaling window offset parameters (pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, pps_scaling_win_bottom_offset) in PPS shall be appropriately updated through the subpicture extraction process.

One of possible approaches is to update the scaling window offsets to be fit to the subpicture size, as shown in Fig. 2. Then, the scaling windows of the reference pictures should be adjusted depending on the size and the position of the extracted subpicture. This approach has two problems. First, when the original scaling window of the reference picture is different from that of the current picture, the updated scaling window of the reference picture may have a fractional pel value of position and size, depending on the relative size of the scaling windows. The fractional pel values cannot be represented by the existing scaling window offset parameters. Second, depending on the size and the location of the extracted subpicture, the scaling window parameters of reference pictures should be updated. Especially, as shown in Fig. 2, when the interlayer reference picture is not partitioned into multiple subpictures, all the scaling windows of the interlayer reference pictures should be updated. It results in a complicated process of the sub-bitstream extraction.

Fig. 1. Example of scaling windows with subpicture partitioning and interlayer prediction

Fig. 2. Example of scaling window updates by rescaling the scaling windows according to the subpicture size

Fig. 3. Example of scaling window updates by keeping the scaling window sizes, but only adjusting left-top position of the scaling windows.

# Proposed method

To address the asserted issues of the first approach, a much simpler method is proposed. The proposed method keeps the original scaling window sizes, but only adjust the left-top position of the scaling window, according to the location of the subpicture. For example, in Fig. 3, in order to keep the existing scaling windows when the right-bottom subpicture is extracted, the left-top position of the existing scaling window in relation to the left-top position of the extracted subpicture is calculated, and the scaling window offset parameters are updated to compensate the shift, keeping the scaling window sizes. The updates process of the scaling window parameters are as follows:

pps_scaling_win_left_offset = orgScalingWinLeft - (SubpicLeftBoundaryPos / SubWidthC);

pps_scaling_win_right_offset = orgScalingWinRight - (SubpicLeftBoundaryPos / SubWidthC);

pps_scaling_win_top_offset = orgScalingWinTop - (SubpicTopBoundaryPos / SubWidthC);

pps_scaling_win_bottom_offset = orgScalingWinBottom - (SubpicTopBoundaryPos / SubWidthC);

where orgScalingWinLeft, orgScalingWinRight, orgScalingWinTop, orgScalingWinBottom are equal to the values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset of the original picture respectively, and the position and size of the extracted subpicture are represented by SubpicLeftBoundaryPos, SubpicRightBoundaryPos, SubpicTopBoundaryPos, SubpicBotBoundaryPos.

In the proposed method, the scaling window parameters of the reference pictures including interlayer references do not need to be updated, regardless of which subpicture is extracted. This method is much simpler than the first approach and has no issue on fractional pel representation. However, the following constraint should be relaxed.

“The value of SubWidthC * ( Abs( pps_scaling_win_left_offset ) + Abs( pps_scaling_win_right_offset ) ) shall be less than pps_pic_width_in_luma_samples, and the value of SubHeightC * ( Abs( pps_scaling_win_top_offset ) + Abs( pps_scaling_win_bottom_offset ) ) shall be less than pps_pic_height_in_luma_samples.”

It is because the scaling windows of the example in Fig. 3 cannot be represented with the above constraint, when the subpicture size is quite smaller than the original picture size. So, it is proposed to have the relaxed constraint same as in JVET-S0048.

“The value of SubWidthC * ( pps_scaling_win_left_offset +  pps_scaling_win_right_offset ) shall be greater than or equal to −pps_pic_width_in_luma_samples * 15 and less than pps_pic_width_in_luma_samples, and the value of SubHeightC * ( pps_scaling_win_top_offset + pps_scaling_win_bottom_offset ) shall be greater than or equal to −pps_pic_height_in_luma_samples * 15 and less than pps_pic_height_in_luma_samples.”

Proposed spec. text

C.7 	Subpicture sub-bitstream extraction process

Inputs to this process are a bitstream inBitstream, a target OLS index targetOlsIdx, a target highest TemporalId value tIdTarget, and an array of target subpicture index values for each layer subpicIdxTarget[ ].

Output of this process is a sub-bitstream outBitstream.

It is a requirement of bitstream conformance for the input bitstream that any output sub-bitstream that satisfies all of the following conditions shall be a conforming bitstream:

–	The output sub-bitstream is the output of the process specified in this clause with the bitstream, targetOlsIdx equal to an index to the list of OLSs specified by the VPS, and subpicIdxTarget[ ] equal to a subpicture index present in the OLS, as inputs.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to each of the nuh_layer_id values in LayerIdInOls[ targetOlsIdx ].

–	The output sub-bitstream contains at least one VCL NAL unit with TemporalId equal to tIdTarget.

NOTE – A conforming bitstream contains one or more coded slice NAL units with TemporalId equal to 0, but does not have to contain coded slice NAL units with nuh_layer_id equal to 0.

–	The output sub-bitstream contains at least one VCL NAL unit with nuh_layer_id equal to LayerIdInOls[ targetOlsIdx ][ i ] and with sh_subpic_id equal to the value in SubpicIdVal[ subpicIdxTarget[ i ] ] for each i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive.

The output sub-bitstream outBitstream is derived as follows:

–	The sub-bitstream extraction process, specified in Annex C.6, is invoked with inBitstream, targetOlsIdx, and tIdTarget as inputs and the output of the process is assigned to outBitstream.

–	If some external means not specified in this Specification is available to provide replacement parameter sets for the sub-bitstream outBitstream, replace all parameter sets with the replacement parameter sets.

–	Otherwise, when subpicture level information SEI messages are present in inBitstream, the following applies:

–	The variable subpicIdx is set equal to the value of subpicIdxTarget[ [ NumLayersInOls[ targetOlsIdx ] − 1 ] ].

–	Rewrite the value of general_level_idc in the vps_ols_ptl_idx[ targetOlsIdx ]-th entry in the list of profile_tier_level( ) syntax structures in all the referenced VPS NAL units to be equal to SubpicLevelIdc derived in Equation D.10 for the set of subpictures consisting of the subpictures with subpicture index equal to subpicIdx.

–	When VCL HRD parameters or NAL HRD parameters are present, rewrite the respective values of cpb_size_value_minus1[ tIdTarget ][ j ] and bit_rate_value_minus1[ tIdTarget ][ j ] of the j-th CPB in the vps_ols_hrd_idx[ MultiLayerOlsIdx[ targetOlsIdx ] ]-th ols_hrd_parameters( ) syntax structure in all the referenced VPS NAL units and in the ols_hrd_parameters( ) syntax structures in all SPS NAL units referred to by the i-th layer, such that they correspond to SubpicCpbSizeVcl[ SubpicSetLevelIdx ][ subpicIdx ], and SubpicCpbSizeNal[ SubpicSetLevelIdx ][ subpicIdx ] as derived by Equations D.6 and D.7, respectively, SubpicBitrateVcl[ SubpicSetLevelIdx ][ subpicIdx ] and SubpicBitrateNal[ SubpicSetLevelIdx ][ subpicIdx ] as derived by Equations D.8 and D.9, respectively, where SubpicSetLevelIdx is derived by Equation D.10 for the subpicture with subpicture index equal to subpicIdx, j is in the range of 0 to hrd_cpb_cnt_minus1, inclusive, and i is in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, inclusive.

For the i-th layer with i in the range of 0 to NumLayersInOls[ targetOlsIdx ] − 1, the following applies.

–	Rewrite the value of general_level_idc in the profile_tier_level( ) syntax structure in all the referenced SPS NAL units with sps_ptl_dpb_hrd_params_present_flag equal to 1 to be equal to SubpicLevelIdc derived by Equation D.10 for the set of subpictures consisting of the subpicture with subpicture index equal to subpicIdx.

–	The variables subpicWidthInLumaSamples and subpicHeightInLumaSamples are derived as follows:

subpicWidthInLumaSamples = 	min( ( sps_subpic_ctu_top_left_x[ subpicIdx ] +	(C.24)
	sps_subpic_width_minus1[ subpicIdx ] + 1 ) * CtbSizeY, pps_pic_width_in_luma_samples ) −
	sps_subpic_ctu_top_left_x[ subpicIdx ] * CtbSizeY

subpicHeightInLumaSamples = 	min( ( sps_subpic_ctu_top_left_y[ subpicIdx ] +	(C.25)
	sps_subpic_height_minus1[ subpicIdx ] + 1 ) * CtbSizeY, pps_pic_height_in_luma_samples ) −
	sps_subpic_ctu_top_left_y[ subpicIdx ] * CtbSizeY

–	Rewrite the values of the sps_pic_width_max_in_luma_samples and sps_pic_height_max_in_luma_samples in all the referenced SPS NAL units and the values of pps_pic_width_in_luma_samples and pps_pic_height_in_luma_samples in all the referenced PPS NAL units to be equal to subpicWidthInLumaSamples and subpicHeightInLumaSamples, respectively.

–	Rewrite the value of sps_num_subpics_minus1 in all the referenced SPS NAL units and pps_num_subpics_minus1 in all the referenced PPS NAL units to 0.

–	Rewrite the syntax elements sps_subpic_ctu_top_left_x[ subpicIdx ] and sps_subpic_ctu_top_left_y[ subpicIdx ], when present, in all the referenced SPS NAL units to 0.

–	Remove the syntax elements sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], sps_subpic_treated_as_pic_flag[ j ], sps_loop_filter_across_subpic_enabled_flag[ j ], and sps_subpic_id[ j ] in all the referenced SPS NAL units and for each j that is not equal to subpicIdx.

–	Rewrite the syntax elements in all the referenced PPS for signalling of tiles and slices to remove all tile rows, tile columns, and slices that are not associated with the subpicture with subpicture index equal to subpicIdx.

–	The variables subpicConfWinLeftOffset, subpicConfWinRightOffset, subpicConfWinTopOffset and subpicConfWinBottomOffset are derived as follows:

subpicConfWinLeftOffset = sps_subpic_ctu_top_left_x[ subpicIdx ]  = =  0 ?	(C.26)
		sps_conf_win_left_offset : 0

subpicConfWinRightOffset = ( sps_subpic_ctu_top_left_x[ subpicIdx ] +	(C.27)
		sps_subpic_width_minus1[ subpicIdx ] + 1 ) *CtbSizeY  >=
		sps_pic_width_max_in_luma_samples ? sps_conf_win_right_offset : 0

subpicConfWinTopOffset = sps_subpic_ctu_top_left_y[ subpicIdx ]  = =  0 ?	(C.28)
		sps_conf_win_top_offset : 0

subpicConfWinBottomOffset = ( sps_subpic_ctu_top_left_y[ subpicIdx ] +	(C.29)
		sps_subpic_height_minus1[ subpicIdx ] + 1 ) * CtbSizeY  >=
		sps_pic_height_max_in_luma_samples ? sps_conf_win_bottom_offset : 0

–	Rewrite the values of sps_conf_win_left_offset, sps_conf_win_right_offset, sps_conf_win_top_offset, and sps_conf_win_bottom_offset in all the referenced SPS NAL units and the values of pps_conf_win_left_offset, pps_conf_win_right_offset, pps_conf_win_top_offset, and pps_conf_win_bottom_offset in all the referenced PPS NAL units to be equal to subpicConfWinLeftOffset, subpicConfWinRightOffset, subpicConfWinTopOffset, and subpicConfWinBottomOffset, respectively.

–	Rewrite the values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset as follows:

pps_scaling_win_left_offset = orgScalingWinLeft - (SubpicLeftBoundaryPos / SubWidthC)

pps_scaling_win_right_offset = orgScalingWinRight - ((SubpicLeftBoundaryPos + sps_pic_width_in_luma_samples -subpicWidthInLumaSamples)/ SubWidthC)

pps_scaling_win_top_offset = orgScalingWinTop - (SubpicTopBoundaryPos / SubWidthC)

pps_scaling_win_bottom_offset = orgScalingWinBottom - ((SubpicTopBoundaryPos + pps_pic_height_in_luma_samples -subpicHeightInLumaSamples)/ SubWidthC),

where orgScalingWinLeft, orgScalingWinRight, orgScalingWinTop and orgScalingWinBottom are equal to the values of pps_scaling_win_left_offset, pps_scaling_win_right_offset, pps_scaling_win_top_offset, and pps_scaling_win_bottom_offset of the original coded picture.

–	Remove from outBitstream all VCL NAL units with nuh_layer_id equal to the nuh_layer_id of the i-th layer and with sh_subpic_id not equal to SubpicIdVal[ subpicIdx ].

–	When sli_cbr_constraint_flag is equal to 1, remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages that are not associated with the VCL NAL units of a subpicture in subpicIdTarget[ ] and set cbr_flag[ tIdTarget ][ j ] equal to 1 of the j-th CPB in the vps_ols_hrd_idx[ MultiLayerOlsIdx[ targetOlsIdx ] ]-th ols_hrd_parameters( ) syntax structure in all the referenced VPS NAL units and SPS NAL units and j in the range of 0 to hrd_cpb_cnt_minus1. Otherwise, (sli_cbr_constraint_flag is equal to 0), remove all NAL units with nal_unit_type equal to FD_NUT and filler payload SEI messages and set cbr_flag[ tIdTarget ][ j ] equal to 0.

–	When outBitstream contains SEI NAL units that contain a scalable nesting SEI message with sn_ols_flag equal to 1 and sn_subpic_flag equal to 1 that are applicable to outBitstream, extract appropriate non-scalable-nested SEI message with payloadType equal to 1 (PT), 130 (DUI), or 132 (decoded picture hash) from the scalable nesting SEI message and place the extracted SEI messages into outBitstream.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).