| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29/WG 11 19th Meeting: by teleconference, 22 June – 1 July 2020 | Document: JVET-S0190 |
| --- | --- |

| Title: | AHG8/AHG9/AHG12: On reference picture resampling with scalability |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Byeongdoo Choi Stephan Wenger Shan Liu | Email: | bdchoi@tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

This contribution proposes the following changes, related to the combination of reference picture resampling and subpicture with multiple layers:

- Add the constraints to the semantics of sps_subpic_treated_as_pic_flag, so that the pictures in the dependent layers have the same picture size and the same subpicture layout, when the reference layer is not partitioned into subpictures while the dependent layer referring the reference layer are partitioned into multiple subpictures.

- Add the following constraint that, when the value of sps_subpic_info_present_flag of the current layer shall be equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layer of the current layer shall be equal to 0.

# Proposal-1

In the current VVC spec, it is constrained that all dependent layers referring to the current layer have the same picture size and the same subpicture layout, when sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1.

At the last meeting, in order to enable the use case of 360 viewport dependent process with subpictures, it is agreed to allows that an independent layer is not partitioned into multiple subpictures while one or more dependent layer referring the independent layer may be partitioned into multiple subpictures. In that case, it is asserted that the pictures in the reference layers shall have the same picture sizes and the same subpicture layouts.

To express the intention correctly, it is proposed to modify the semantics of sps_subpic_treated_as_pic_flag as follows:

Proposed Spec Text

sps_subpic_treated_as_pic_flag[ i ] equal to 1 specifies that the i-th subpicture of each coded picture in the CLVS is treated as a picture in the decoding process excluding in-loop filtering operations. sps_subpic_treated_as_pic_flag[ i ] equal to 0 specifies that the i-th subpicture of each coded picture in the CLVS is not treated as a picture in the decoding process excluding in-loop filtering operations. When not present, the value of sps_subpic_treated_as_pic_flag[ i ] is inferred to be equal to 1.

When sps_num_subpics_minus1 is greater than 0 and sps_subpic_treated_as_pic_flag[ i ] is equal to 1, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of pps_pic_width_in_luma_samples and the same value of pps_pic_height_in_luma_samples.

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

When sps_num_subpics_minus1 is equal to 0, for each CLVS of a current layer referring to the SPS, let targetAuSet be all the AUs starting from the AU containing the first picture of the CLVS in decoding order, to the AU containing the last picture of the CLVS in decoding order, inclusive, it is a requirement of bitstream conformance that all of the following conditions are true for the targetLayerSet that consists of the current layer and all the layers that have the current layer as a reference layer:

–	All the SPSs referred to by the layers in targetLayerSet shall have the same value of sps_num_subpics_minus1 and shall have the same values of sps_subpic_ctu_top_left_x[ j ], sps_subpic_ctu_top_left_y[ j ], sps_subpic_width_minus1[ j ], sps_subpic_height_minus1[ j ], and sps_subpic_treated_as_pic_flag[ j ], respectively, for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

–	For each AU in targetAuSet, all pictures of the layers in targetLayerSet shall have the same value of SubpicIdVal[ j ] for each value of j in the range of 0 to sps_num_subpics_minus1, inclusive.

# Proposal-2

When a picture is partitioned into multiple subpictures (i.e. sps_num_subpics_minus1 is greater than 0), all reference pictures (including interlayer references) of the picture shall have the same picture width and height as the current picture. All constraints related to reference picture resampling and subpicture implicitly impose that constraint. One example is that, when sps_res_change_in_clvs_allowed_flag is equal to 1, the value of sps_subpic_info_present_flag shall be equal to 0. Another example is that, when the current layer has sps_num_subpics_minus1 greater than 0. all layers referring to the current layer shall have the same subpicture layout as the current layer.

Only the exceptional case is that a reference layer is not partitioned into multiple subpictures, while the depedent layer referring to the reference layer is partitioned into mulitiple subpictures. In that case, we cannot impose a constraint that the picture size of the interlayer reference picture in the referece layer shall be the same as the current picture size in the dependent layer, because the subpictures of the dependent layer can be extracted. If the constriant is imposed, multiple subpictures cannot be extracted in that use case. Note that the current subpicture sub-bitstream specifies only a signgle subpicture extraction, but multiple subpictures can be extracted in real implementations.

Instead, to simplify the refernce picture resampling process with multiple layers, it is proposed to constrain that, when the value of sps_subpic_info_present_flag of the current layer shall be equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layer of the current layer shall be equal to 0. This constraint imposes that the picture sizes of the interlayer reference pictures cannot be randomly changed, when the current picture in the dependent layer has multiple subpictures.

Proposed Spec Text

When the value of sps_subpic_info_present_flag of the current layer shall be equal to 1, the value of sps_res_change_in_clvs_allowed_flag of the reference layer of the current layer shall be equal to 0.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).